3GPP TSG-WG4 Meeting #79	R4-164411

# Nanjing, China, 23rd – 27th, May 2016

Agenda item: 	6.13.4.7

Source: 	Qualcomm Incorporated

Title: 	On Power headroom reporting for NB-IoT

Document for:	Approval

# 1		Introduction

RAN1 discussed power head room (PHR) reporting as agreed the following:

- NB-PHR is computed based on a 15kHz single-tone transmit power for NB-PUSCH data transmission regardless of the actual subcarrier spacing:        
PH(i) = PCMAX,c(i)-{PO_NPUSCH,c(1)+αc(1)∙PLc}

- There will be 4 reportable values of PHR

- Exact values to be determined by RAN4 and specified in 36.133.

RAN4 should discuss and determine the PHR reporting values/ranges. This paper makes a proposal on PHR reporting in 36.133.

# 2	LTE Power Headroom Report Mapping

The power headroom reporting range is from -23 ...+40 dB. Table 9.1.8.4-1 in section 9.1.8.4 in [1] defines the report mapping as follows:

Table 1: Power headroom report mapping

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -23 PH -22 |
| POWER_HEADROOM_1 | -22 PH -21 |
| POWER_HEADROOM_2 | -21 PH -20 |
| POWER_HEADROOM_3 | -20 PH -19 |
| POWER_HEADROOM_4 | -19 PH -18 |
| POWER_HEADROOM_5 | -18 PH -17 |
| POWER_HEADROOM_57 | 34 PH 35 |
| POWER_HEADROOM_58 | 35 PH 36 |
| POWER_HEADROOM_59 | 36 PH 37 |
| POWER_HEADROOM_60 | 37 PH 38 |
| POWER_HEADROOM_61 | 38 PH 39 |
| POWER_HEADROOM_62 | 39 PH 40 |
| POWER_HEADROOM_63 | PH ≥ 40 |

# 3	NB-IoT Power Headroom Report Mapping

The power headroom reporting range is from -23 ...+40 dB. The following table is the proposed report mapping for NB-IoT. We don’t need lot of granularity after 10 dB since only QPSK modulation is supported for Category NB1.

Table 2: NB-IoT Power headroom report mapping

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -23 PH < 3 |
| POWER_HEADROOM_1 | 3 PH < 6 |
| POWER_HEADROOM_2 | 6 PH < 10 |
| POWER_HEADROOM_3 | PH ≥ 10 |

# 4	Conclusion

This paper has made the following proposal on PHR report mapping for NB-IOT:

Proposal: The NB-IoT PHR report mapping can be specified as follows:

Table 2: NB-IoT Power headroom report mapping

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -23 PH < 3 |
| POWER_HEADROOM_1 | 3 PH < 6 |
| POWER_HEADROOM_2 | 6 PH < 10 |
| POWER_HEADROOM_3 | PH ≥ 10 |

# 5	References

- TS36.133 v13.2.0, “Requirements for support of radio resource management”