3GPP TSG-RAN WG4 #79	R4-164528

May 23rd – 27th, 2016, Nanjing, China

Agenda item:	9.8

Source: 	NTT DOCOMO, INC.

Title: 	WF on WP 5D co-existence simulation assumptions

Document for:	Approval

Introduction

This contribution summarises agreed simulation assumptions in two evening AHs held in RAN4#79 meeting [1, 2] to proceed with the simulation calibration for the future meetings. Specifically, it is significantly essential to compare each company’s simulation results with common simulation parameters as much as possible to access and identify factors to produce the large difference.

Way forward

Adopt the main simulation assumptions summarised in below Table 1.

Table 1. Main simulation assumption.

|  | Way forward | Remarks |
| --- | --- | --- |
| Methodology | Static MonteCarlo simulation |  |
| 3.4. Usage scenario (eMBB, mMTC, URLLC) | eMBB | Start with eMBB as 1st step. We shall consider the other scenarios in SI. |
| Deployment scenario | At least, Indoor hotspot, Urban macro and Dense urban are the scenarios for WP5D co-existence. | The other scenarios are FFS. |
| Victim system | NR |  |
| PathLoss Model | Follow RAN1 channel modelling SI outcome |  |
| Carrier frequency | 30 and 70 GHz | Using 28GHz is not precluded in addition to 30 and 70GHz. |
| Channel Bandwidth | 200 MHz | Even wider channel bandwidth for example 1GHz can be considered later after understanding the baseline in SI |
| Channel arrangement | Symmetrical (200MHz vs 200MHz) | Start with symmetrical as 1st step |
| UL power control | The same as TR 36.942, adopting PC set parameters to the scenario |  |
| DL power control | No power control |  |
| The number of antenna element | For 30GHz: UE: 16, eNB: 256 For 70GHz: UE: 32, eNB: 256 | Companies are encouraged to provide technical justification on these values. These number can be discussed after checking the latest RAN1 status. |
| UE antenna | FFS | Further discuss the model based on RAN1 SI Channel modelling SI |
| BS antenna | FFS | Further discuss the model based on RAN1 SI Channel modelling SI |
| Beamforming | FFS |  |
| SINR to throughput mapping | Adopt scaled Shannon's formula with update truncation and attenuation parameters until we get RAN1, we will use 36.942Scaled Shannon's formula. |  |
| Co-existence scenario in terms of interference | UL to UL and DL to DL | Not consider UE to DL and DL to UL for the moment. Specifically for UE to UE, this can be discussed after understanding specific mitigation techniques. |
| Traffic | Full buffer |  |
| ACI leakage model | In addition, model in ACLR is two step models in frequency Ues the different correlation levels to study the spatial patterns for ACLR. ACLR metric defines integrated over the whole space. If study should ACLR can be modelled as flat in space, then, we just use integrated ACLR model. Assume flat ACS in frequency. In space, study if ACS model is flat. |  |
| Performance metric | Throughput degradation compared to single operator case, i.e. no ACI. Both mean and 5%-tile are considered. |  |
| NF | FFS | Companies are encourage to provide more analysis data on this aspect in the future meeting. |

Conclusions

In this paper, we summarized main simulation assumptions for the future meetings for co-existence study for WP 5D.

- Reference

- R4-164541, “Evening AH minutes for WP 5D co-existence simulation assumptions”, NTT DOCOMO, INC, 3GPP TSG-RAN WG4 #79, May 23rd – 27th, 2016, Nanjing, China.

- R4-164543, “Evening AH minutes for NR AH for WP5D co-existence and PA models”, NTT DOCOMO, INC, 3GPP TSG-RAN WG4 #79, May 23rd – 27th, 2016, Nanjing, China.