3GPP TSG-RAN4 Meeting #79	R4-164633

# Nanjing, China, 23rd May 2016 - 27th May 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3611 | rev | 1 | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on eD2D demodulation performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eD2D_Prox-Perf |  |  |  |  |  | Date: |  |  | 2016-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Introduction of eD2D demodulation performance requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Update applicability of WAN SDR test for single CC, different channel BWs, and different CA configurations. Change 2: Extended WAN SDR test with active Sidelink for D2D communication due to multicarrier support. Change 3: Update applicability of ProSe Direct Discovery tests (due to support of discSLSS and discPeriodicSLSS). Change 4: Introduce OOC discovery test requirement Change 5: Introduce pre-configuration for OOC discovery |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eD2D demodulation performance requirements will not be verified |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  |  |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

## 12.1	General

### 12.1.1	Applicability of requirements

### 12.1.1.1	Applicability of requirements for different channel bandwidths

The requirements in this clause are applicable to UEs that support ProSe Direct Communication. Test cases defined for 5MHz channel bandwidth are applicable to UEs that support ProSe Direct Communication on only Band 31.

#### 12.1.1.2	Test coverage for different number of component carriers

For FDD tests specified in 12.8, if corresponding CA tests are tested, the test coverage can be considered fulfilled without executing single carrier tests.

#### 12.1.1.3	Applicability and test rules for different CA configurations and bandwidth combination sets

The performance requirement for CA UE demodulation tests with active Sidelink in Clause 12 are defined independent of CA configurations and bandwidth combination sets specified in Clause 5.6A.1. For UEs supporting different CA configurations and bandwidth combination sets, the applicability and test rules are defined in Table 12.1.1.3-1. For simplicity, CA configuration below refers to combination of CA configuration and bandwidth combination set.

Table 12.1.1.3-1: Applicability and test rules for CA UE demodulation tests with active Sidelink

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 2CCs in Clause 12.8 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported FDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |

< END OF CHANGE #1 >

< START OF CHANGE #2 >

## 12.8	Sustained downlink data rate with active Sidelink

The purpose of this test is to verify the downlink data rate is not impacted when Sidelink resource are also configured. The test parameters are in Table 12.8.1-1. Cell 1 is the serving cell and UE 1 and UE 2 are transmitters of Prose Direct Communication. The test UE is expected to receive all PDSCH transmissions, and prioritize the transmission of ACK/NACK over the reception of UE 2’s PSSCH.

The test cases apply to UE categories and bandwidth combinations with maximum aggregated bandwidth as specified in Table 12.8.1-2. The minimum requirements are specified in Table 12.8.1-3. The TB success rate in the cellular link shall be sustained during at least 300 frames.

Table 12.8.1-1: Test parameters for sustained downlink data rate (FDD 64QAM) with active Sidelink

| Parameter |  | Unit | Test 1, 2, 3A, 3B, 4A, 6C |
| --- | --- | --- | --- |
| Communication resource pool configurationNote 5 |  |  | As specified in Table A.7.2.1-5 (Configuration #5-FDD) |
| Active cell(s) |  |  | Cell 1 (Pcell) Cell 2 (SCell) for Test 3B, 4A, 6C |
| Cell 1, Cell 2 | Test parameters |  | As specified in clause 8.7.1: Table 8.7.1-1 and Test 1, 2, 3A, 3B, 4A, 6C in Table 8.7.1-2 |
| Active Sidelink UE(s) |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSCCH RMC |  | 10 MHz: CC.2 FDD with ITRP=0 (NOTE 1) |
|  | PSCCH subframe allocation |  | As defined by TS 36.213 with = 0 |
|  | PSCCH RB allocation |  |  |
|  | PSSCH RMC |  | 10 MHz: CD.7 FDD |
|  | PSSCH subframe allocation |  | As per time repetition pattern specified in PSCCH |
|  | PSSCH RB allocation |  | Fully allocated |
|  | Time offset (NOTE 3) | s | 0 |
|  | Frequency offset (NOTE 4) | Hz | 0 |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 Low |
|  | at antenna port | dBm/15kHz | -85 |
| Sidelink UE 2 | Sidelink Transmissions |  | PSCCH (NOTE 2) |
|  | PSCCH RMC |  | 10 MHz: CC.2 FDD with ITRP=1 (NOTE 1) |
|  | PSCCH subframe allocation |  | As defined by TS 36.213 with = 1 |
|  | PSCCH RB allocation |  |  |
|  | Time offset (NOTE 3) | s | 0 |
|  | Frequency offset (NOTE 4) | Hz | 0 |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 Low |
|  | at antenna port | dBm/15kHz | -85 |
| NOTE 1: For NTRP = 8 (FDD) and trpt-Subset = 001, ITRP = 0 corresponds to a time repetition pattern of (1,0,0,0,0,0,0,0), ITRP = 1 corresponds to a time repetition pattern of (0,1,0,0,0,0,0,0). NOTE 2: Sidelink UE 2 transmits PSCCH but not PSSCH. NOTE 3: Time offset of Sidelink UE receive signal with respect to Cell 1 downlink timing at the tested UE. NOTE 4: Frequency offset of Sidelink UE with respect to Cell 1 uplink frequency. NOTE 5: Sidelink Communication resources are configured on the primary serving cell. |  |  |  |

Table 12.8.1-2: Test cases for sustained data rate

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) | Cat. 1 | Cat. 2 | Cat. 3 | Cat. 4 | Cat. 6,7 | Cat. 9,10 | Cat 11, 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Single carrier | 10 | 1 | 2 | 3A | 3A | 3A | 3A | 3A |
| CA with 2CCs (Note1) | 10+10 | 3B | 4A | 4A | 4A | 4A | 4A | 4A |
|  | 10+20 | 3B | 4A | 4A | 4A | 6C | 6C | 6C |
| NOTE 1: Sidelink operation is configured on PCC |  |  |  |  |  |  |  |  |

Table 12.8.1-3: Minimum requirements (FDD 64QAM) with active Sidelink

| Test | Bandwidth (MHz) | Number of bits of a DL-SCH transport block received within a TTI | Measurement channel | Reference value |
| --- | --- | --- | --- | --- |
|  |  |  |  | PDSCH TB success rate (%) |
| 1 | 10 | 10296 | R.31-1 FDD (NOTE 2) | 95 |
| 2 | 10 | 25456 | R.31-2 FDD (NOTE 2) | 95 |
| 3A | 10 | 36696 (NOTE 1) | R.31-3A FDD (NOTE 2) | 85 |
| 3B | 10+10 | 25456 | R.31-2 FDD (NOTE 2) | 95 |
| 4A | 10+10 | 75376 (NOTE 3) | R.31-4 FDD (NOTE 2) | 85 |
| 6C | 10+20 | 36696 (NOTE 1) for 10MHz CC 75376 (NOTE 3) for 20MHz CC | R.31-3A FDD for 10MHz CC R.31-4 FDD for 20MHz CC | 85 |
| NOTE 1: 35160 bits for sub-frame 5. NOTE 2: PDSCH scheduling pattern is changed as per the following bitmap that repeats every 40ms. PDSCH scheduling subframe bitmap = {01110111 11110111 11110111 11110111 11111110}. NOTE 3: 71112 bits for sub-frame 5. |  |  |  |  |

< END OF CHANGE #2 >

< START OF CHANGE #3 >

# 11	Performance requirement (ProSe Direct Discovery)

This clause contains the performance requirements for the Sidelink physical channels specified for ProSe Direct Discovery.

## 11.1	General

### 11.1.1	Applicability of requirements

The requirements in this clause are applicable to UEs that support ProSe Direct Discovery. The test case applicability is in according to table 11.1.1-1 depending on UE capability.

Table 11.1.1-1: ProSe Direct Discovery test applicability

| FDD/TDD | Tests / clause | Applicable if UE indicates at least the following capability |
| --- | --- | --- |
| FDD | 11.3.1, 11.5.1 | ProSe Direct Discovery support |
|  | 11.2.1 | ProSe Direct Discovery support without support of disc-SLSS |
|  | 11.4.1 | ProSe Direct Discovery support with support of disc-SLSS |
|  | 11.6.1 | ProSe Direct Discovery with support of discPeriodicSLSS, and ProSe Direct Communication support |
| TDD | 11.2.2, 11.3.2, 11.5.2 | ProSe Direct Discovery support |

For maximum Sidelink Processes test specified in clause 11.5, the UE is required to only meet the test for the maximum channel bandwidth over the ProSe operating bands supported by the UE.

< END OF CHANGE #3 >

< START OF CHANGE #4 >

## 11.6	Demodulation of PSDCH (out of network coverage)

The purpose of the requirements in this subclause is to verify the PSDCH demodulation performance with a single active PSDCH link for the out of network coverage scenario.

### 11.6.1	FDD

The minimum requirements are specified in Table 11.6.1-2 with the test parameters specified in Table 11.6.1-1. This test specifies an out-of-coverge scenario where Sidelink UE 1 is the synchronization reference only and Sidelink UE 2 transmits PSDCH.

Table 11.6.1-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Discovery resource pool configuration |  |  | As specified in Table A.7.1.1-4 (Configuration #4-FDD) |
| DRX configuration |  |  | As specified in Table 11.1.2-1 |
| at antenna port (NOTE 1) |  | dBm/15kHz | -98 |
| Active cell(s) |  |  | None |
| Active Sidelink UE(s) |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  | SLSS + PSBCH |
|  | networkControlledSyncTx |  | ON |
|  | slssid |  | 30 |
|  | inCoverage (in MIB-SL) |  | TRUE |
|  | syncOffsetIndicator |  | Set same as syncOffsetIndicator1 in Configuration #4-FDD |
|  | Propagation channel |  | EPA5 |
|  | Antenna configuration |  | 1x2 Low |
|  | at antenna port | dBm/15kHz | -85 |
| Sidelink UE 2 | Sidelink Transmissions |  | PSDCH |
|  | PSDCH RB allocation |  | PRB pairs {2i..2i+1}, where i is chosen randomly uniformly from [0,11] (for 5MHz) or [0,24] (for 10MHz) in each discovery period. |
|  | Time offset (NOTE 2) | s | +1 |
|  | Frequency offset (NOTE 3) | Hz | +200 |
|  | Propagation Channel |  | EPA5 |
|  | Antenna configuration |  | 1x2 Low |
| NOTE 1: Applicable to both DL channel and ProSe Direct Communication Subframes on UL. NOTE 2: Time offset of Sidelink UE 2 receive signal timing with respect to Sidelink UE 1 receive signal timing at the tested UE. NOTE 3: Frequency offset of Sidelink UE 2 with respect to Sidelink UE 1 transmit frequency. |  |  |  |

Table 11.6.1-2: Minimum performance

| Test num. | Sidelink UE | Band-width | Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | BLER of PSDCH (%) | SNR (dB) |
| 1 | 2 | 5 MHz | D.1 FDD | 30 | 4.6 |
|  |  | 10 MHz |  |  |  |

< END OF CHANGE #4 >

< START OF CHANGE #5 >

# A.7	Sidelink reference resource pool configurations

## A.7.1	Reference resource pool configurations for ProSe Direct Discovery demodulation tests

### A.7.1.1	FDD

Table A.7.1.1-1: ProSe Direct Discovery configuration for E-UTRA FDD (Configuration #1-FDD)

| Information Element |  |  | Value |
| --- | --- | --- | --- |
| discRxPool | cp-Len |  | Normal |
|  | discPeriod |  | rf32 |
|  | numRetx |  | 0 |
|  | numRepetition |  | 1 |
|  | tf-ResourceConfig | prb-Num | 12 |
|  |  | prb-Start | 0 |
|  |  | prb-End | 23 |
|  |  | offsetIndicator | 160 |
|  |  | subframeBitmap | 10000000 00000000 00000000 00000000 00000000 |
|  | txParameters |  | not present |
|  | rxParameters |  | not present |
| discTxPoolCommon |  |  | not present |
| discTxPowerInfo |  |  | not present |
| SL-SyncConfig |  |  | not present |
| discInterFreqList |  |  | not present |

Table A.7.1.1-2: ProSe Direct Discovery configuration for E-UTRA FDD (Configuration #2-FDD)

| Information Element |  |  | Value |
| --- | --- | --- | --- |
| discRxPool(0) | cp-Len |  | Normal |
|  | discPeriod |  | rf32 |
|  | numRetx |  | 0 |
|  | numRepetition |  | 1 |
|  | tf-ResourceConfig | prb-Num | 12 |
|  |  | prb-Start | 0 |
|  |  | prb-End | 23 |
|  |  | offsetIndicator | 150 |
|  |  | subframeBitmap | 10000000 00000000 00000000 00000000 00000000 |
|  | txParameters |  | not present |
|  | rxParameters |  | not present |
| discRxPool(1) | cp-Len |  | Normal |
|  | discPeriod |  | rf32 |
|  | numRetx |  | 0 |
|  | numRepetition |  | 1 |
|  | tf-ResourceConfig | prb-Num | 12 |
|  |  | prb-Start | 0 |
|  |  | prb-End | 23 |
|  |  | offsetIndicator | 170 |
|  |  | subframeBitmap | 10000000 00000000 00000000 00000000 00000000 |
|  | txParameters |  | not present |
|  | rxParameters | tdd-Config | not present |
|  |  | syncConfigIndex | 0 |
| discTxPoolCommon |  |  | not present |
| discTxPowerInfo |  |  | not present |
| SL-SyncConfig(0) | syncCP-Len |  | Normal |
|  | syncOffsetIndicator |  | 0 (160 mod 40) |
|  | slssid |  | 30 |
|  | txParameters |  | not present |
|  | rxParamsNCell | physCellId | 1 |
|  |  | discSyncWindow | w1 |
| discInterFreqList |  |  | not present |

Table A.7.1.1-3: ProSe Direct Discovery configuration for E-UTRA FDD (Configuration #3-FDD)

| Information Element |  |  | Value |
| --- | --- | --- | --- |
| discRxPool(iPool), iPool = 0…NPool-1 | cp-Len |  | Normal |
|  | discPeriod |  | rf32 |
|  | numRetx |  | 3 |
|  | numRepetition |  | =2 if NPool > 10, =1 otherwise |
|  | tf-ResourceConfig | prb-Num | 5MHz: min{24, 2N-24*iPool} / 2 10MHz: 25 15MHz: min{74, 2N-74*iPool} / 2 20MHz: 50 |
|  |  | prb-Start | 0 |
|  |  | prb-End | 5 MHz: min{24, 2N-24*iPool} - 1 10 MHz: 49 15 MHz: min{74, 2N-74*iPool} - 1 20 MHz: 99 |
|  |  | offsetIndicator | 160 |
|  |  | subframeBitmap | a(0), a(1), …, a(39), s.t. a(i * NPool + iPool) = 1, i = 0,..,K; a(k) = 0 otherwise where K = 1 is NPool > 10, K = 3 otherwise |
|  | txParameters |  | not present |
|  | rxParameters |  | not present |
| discTxPoolCommon |  |  | not present |
| discTxPowerInfo |  |  | not present |
| SL-SyncConfig |  |  | not present |
| discInterFreqList |  |  | not present |
| NOTE 1: The resource pool configuration description is parameterized using channel BW, number of configured resource pools (NPool), and maximum number of configured Sidelink UEs to be supported (N). |  |  |  |

Table A.7.1.1-4: ProSe Direct Discovery pre-configuration for E-UTRA FDD for out-of-network coverage operation (Configuration #4-FDD)

| Information Element / (BW configuration) |  |  |  |  | Value (5MHz) |  | Value (10MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| preconfigSync |  | syncCP-Len-r12 |  |  |  | Normal |  |
|  |  | syncOffsetIndicator1 |  |  |  | 1 |  |
|  |  | syncOffsetIndicator2 |  |  |  | 2 |  |
|  |  | syncTxParameters |  |  |  | 23 |  |
|  |  | syncTxThreshOoC |  |  |  | 0 (-110dBm / 15kHz) |  |
|  |  | filterCoefficient |  |  |  | fc0 |  |
|  |  | syncRefMinHyst |  |  |  | dB0 |  |
|  |  | syncRefDiffHyst |  |  |  | dB0 |  |
| preconfigDisc |  | cp-Len |  |  |  | Normal |  |
|  |  | discPeriod |  |  |  | rf4 |  |
|  |  | numRetx |  |  |  | 0 |  |
|  |  | numRepetition |  |  |  | 1 |  |
|  | tf-ResourceConfig |  | prb-Num |  | 12 |  | 25 |
|  |  |  | prb-Start |  | 0 |  | 0 |
|  |  |  | prb-End |  | 23 |  | 49 |
|  |  |  |  | offsetIndicator |  | 0 |  |
|  |  |  |  | subframeBitmap |  | 00000000 10000000 00000000 00000000 00000000 |  |
|  |  | txParameters |  |  |  | not present |  |

< END OF CHANGE #5 >