3GPP TSG-RAN WG4 #79	R4-164746

Nanjing, China, May.23~May.27, 2016

Source: 	Ericsson, Huawei

Title:	Simulation assumption for LAA demodulation

Agenda Item:	6.10.4

Document for:	Approval

# Introduction

In this paper, simulation assumption for LAA demodulation is provided for some test cases. It is subjected to change when new test cases are agreed.

# Common parameters for Scell

Table 1 provides all common parameters for Scell. Without special mention, the parameters in Table 1 will be applied for all the following sections.

Table 1: Common parameters for Scell

| Parameters | unit | Value |
| --- | --- | --- |
| MBSFN configuration |  | According to the corresponding agreements in R4-164749 |
| DRS periodicity | ms | 80 |
| Frequency offset related to primary carrier | Hz | According to the corresponding agreements in R4-164749 |
| Time offset related to primary carrier | us | According to the corresponding agreements in R4-164749 |
| Bandwidth | MHz | 20 |
| Number of allocated resource blocks (Note 2) | PRB | 100 |
| Number of control OFDM symbols |  | 2 |
| Signal transmission model |  | According to the corresponding agreements in R4-164748 |

# Simulation assumption for DMRS-based transmission scheme

For Pcell, the parameters defined in Table 8.2.1.3.1-4 and Table 8.2.1.3.1-3 of 36.101 are reused and recapped in Table 2. For Scell, the parameters are shown in

Table 3.

Table 2: Parameters for PCell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| bandwidth |  | MHz | 20 |
| Propagation condition |  |  | EVA5 |
| Correlation matrix |  |  | 2x2 Low |
| Reference channel |  |  | R.30 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . NOTE 2: PUCCH format 1b with channel selection is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-4, PUCCH format 3 is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-6. NOTE 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 3: Parameters for Scell

| parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| DMRS ports (dual layer transmission) |  |  | port 7 and port 8 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Correlation matrix |  |  | 2x2 Low |
| Propagation condition |  |  | EVA5 |
| TBS for each TTI |  |  | A desire coding rate R is given and TBS for each TTI is derived according to A.2.1.2 of 36.101 and the TBS for initial partial subframe shall be derived according to 36.213. |
| MCS |  |  | Option 1: 16QAM 1/2 Option 2: 64QAM 1/2 Other options are not precluded |
| PDSCH transmission mode |  |  | 9 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

# Simulation assumption for CRS-based transmission scheme

## Closed-loop spatial multiplexing performance

The parameters for Pcell are the same as Table 8.2.1.4.3-3 and recapped in Table 4, and the parameters for Scell are provided in Table 5.

Table 4: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA (Table 8.2.1.4.3-3)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for Tests in Table 8.2.1.4.3-4, and with PUCCH format 3 for Tests in Table 8.2.1.4.3-6. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 5: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for Scell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 8 |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| TBS for each TTI |  |  | A desire coding rate R is given and TBS for each TTI is derived according to A.2.1.2 of 36.101 and TBS for initial subframe shall be decided according to 36.213 with the given coding rate |
| MCS |  |  | Option 1: 16QAM 1/2 Option 2: 64QAM 1/2 Other options are not precluded |
| Propagation condition |  |  | EVA5 |
| Correlation matrix |  |  | 4x2 Low |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for Tests in Table 8.2.1.4.3-4, and with PUCCH format 3 for Tests in Table 8.2.1.4.3-6. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

# Reference

- R4-164749, Way forward on LAA demodulation performance, May, 2016

- R4-164748, Way forward on the burst transmission model for LAA demodulation, May, 2016