3GPP TSG-RAN WG4 Meeting #80	R4-164982

# Gothenburg, Sweden, 22 - 26 Aug, 2016

Agenda Item:	6.1 Base Station (BS) RF requirements for Active Antenna System (AAS)

Source:	Kathrein

Title:	Text proposal: Near Field Measurement Method using Standard 3GPP Interfaces

Document for:	Approval

# Introduction

In the last meeting RAN4#79 in [2] was proposed a near field measurement method using standard 3GPP interfaces. It was proposed to include this method as special aspect in the near field test range chapter.

We propose to include this method into the technical report 37.842 chapter section 10.3.2.1.4 [1].

# References

[1]	TS 37.842,  Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station

[2]	R4-163895, Near Field Measurement Method using Standard 3GPP Interfaces

# Text proposal TR 37.842

##### 10.3.2.1.4	Near Field Test range

###### 10.3.2.1.4.1	Description

The Near Field measurement techniques consist in measuring amplitude and phase of the modulated signal at the AAS under test, on some specific surfaces such as planar, cylindrical, and spherical. Each of the near field test methods can be implemented by one or more mechanical rotations of the probe and/or AAS under test as shown in Figure 10.3.2.1.4.1-1. One or more mechanical movement can be substitute by a probe array. All the scanning methods will need an RF transmit and receive system equipped with an automated scanning, a data collection and control system, and computerized analysis ability.

Figure 10.3.2.1.4.1-1. Probe/Scanner Near Field System

In case the radiated field is sampled on a sphere surrounding completely the AAS under test, the 3D full sphere EIS values can be measured in Near Field when the AAS is receiving a defined modulated signal. The near field measurement techniques would imply the use of mathematical artefact, NF to FF transform in order to have the EIS value the in Far Field. The Near Field to Far Field transform is based on the well-known Hyugens-Fresnel principle. The spherical modal wave expansion is the implicit application of the Hyugens principle. A direct solution of the Helmholtz equations is found by applying boundary conditions on the surface S at infinity. From the tangential fields over the surface, the modal coefficients can be determined using the orthogonality of the modal expansion.  Based on this formulation, the Near Field sampling criteria does play an important role.

###### 10.3.2.1.4.2	Test Method limitations and scope

The Near Field Test Method is suitable for OTA sensitivity measurement of AAS BS implementations where the beam forming is done on in a fixed or preset manner for the measured OSDD declaration, i.e. there is only one interface to digital/baseband processing. It is not suitable if for the measured OSDD declaration any combining is not fixed or preset. Some examples of architectures for which near field test method is possible and for which it is not possible are depicted below. The AAS BS under test must not have any other non-linear behaviour that would cause the Near field to Far Field transformation to fail or cause increased measurement uncertainty.

To solve the problem of unknown phase information, it is possible to use a sniffer antenna. With the additional reference antenna the phase can be calculated via superimposing of two signals, see chapter 10.3.2.1.4.zz.

Figure 10.3.2.1.4.2-1	Examples of limit of OSDD functionality testable with Near field test method for a single OSDD

Figure 10.3.2.1.4.2-2	Examples of limit of OSDD functionality not testable with Near field test method for a single OSDD

###### 10.3.2.1.4.3	Procedure Calibration

Calibration shall account for the various factors affecting the measurements of the EIS. These factors include components such as range length path loss, cable losses, gain of the receiving antenna, etc… Each measured data point for radiated sensitivity is transformed from a relative value in dB to an absolute value in dBm. For doing that the total path loss from the DUT to the measurement receiver, named L path loss shall be calibrated out. The calibration measurement is usually done by using a reference antenna with known gain. This approach is based on the so called gain-comparison method [14]. Figure 10.3.1.1.4.3-1 shows the typical configuration for measuring path loss:

###### Figure 10.3.1.1.4.3-1 Typical Lpath loss measurement configuration

The Lpath loss can be determined from the power into the reference antenna by adding the gain of the reference antenna:

Piso=Pref+GrefPIsotropic Radiator=Preference antenna+Greference antenna

so that:

Lpath loss=Pref+Gref-PtestLpath loss=Preference antenna+ Greference antenna- Ptest equipment

In order to determine Pref , a cable reference measurement shall be performed in order to calibrate out the A, and B pathsLtest equipment-receiver. Assuming that the power at the source is fixed, it can be showed that:

Pref-Ptes=Prec’-Prec

Where Prec Preceiver and Prec’ are the power measured at the receiver during the calibration measurement with the reference antenna and the power measured at the receiver during the cable reference measurement respectively. Lpath loss Lpath loss is then given by:

Lpath loss=Gref+Prec’-Prec

Measurement

The testing procedure consists in the following steps:

- AAS EIS Near Field radiation pattern measurement

  - This is performed when the AAS will be receiving a defined modulated signal

- AAS EIS Near Field Sensitivity Measurement

  - This is performed by measuring the sensitivity in Near Field when a modulated signal is received by the AAS

- AAS EIS Near Field to Far Field Transformation

Sensitivity measurement is appliedAAS EIS Near Field to Far Field Transformation

The Near Field to Far Field transformation is a mathematical computation which is applied to the Near Field measured radiation pattern in order to compute the Far Field radiation pattern. It is typically performed expanding the measured near filed over a set of orthogonal basis functions. The Near to Far field transform is then performed in two steps:

- Expansion (or projection) of the measured Near Field over a set of orthogonal basis functions in order to evaluate the transformed spectrum Emeas(r) = Spectrum * Fbasis(r)

- FF (EFF) computation using a the previuosly calculated spectrum and with the basis functions evaluated at r∞ : EFF = Spectrum * Fbasis(r  ∞)

When performing the near field to far field transformation, the EIS sensitivity measurement is applied so that the EIS near field pattern will be transformed from relative power, dB to absolute power, dBm. In this specific case, the EIS Far Field pattern is expressed in terms of the absolute power received by the AAS.

AAS OTA Sensitivity

Once the full 3D Far Field EIS pattern has been computed, the OTA sensitivity can be derived.

###### 10.3.2.1.4.zz				Near Field Measurement Method using Standard 3GPP Interfaces

This method requires only standardized power measurements using standard 3GPP interfaces. It requires an additional reference antenna and a phase shifter.

Figure xx shows the uplink AAS setup for near field measurements in an anechoic chamber. The method is based on the superposition of two signals transmitted by the probe antenna and the reference antenna. The only difference between the signals at the two antennas is that they can be adjusted in phase and amplitude with respect to each other.

The reference antenna is mounted close to the AAS on the turn table to make the over the air path between reference antenna and AAS independent from any angle settings. Therefore the phase difference between the two superimposed signals can directly be used for the near field far field transformation.

Figure xx: Near field system setup for testing AAS in UL direction using 3GPP standardized interfaces only.

Combination of the different power measurements gives the final result:

γϑ,φ=γ0+arctan PAAS,2ϑ,φ- PAAS,4ϑ,φPAAS,1ϑ,φ- PAAS,3ϑ,φ

The power of the superposition of these signals can be calculated as

PAAS,iϑ,φ= 12Ap2ϑ,φ+Aref2+Apϑ,φAref cosγϑ,φ- γi

where i denotes the index of the phase setting.

Ap(,), Aref,i 	amplitudes of probe and reference antenna

(,)	phase which has to be determined

The fundamental result is, that the required phase value can be deduced from simple power measurement data and that internal phase drifts of the AAS receiver are completely cancelled out.

###### 10.3.2.1.4.4 	Uncertainty budget format

Table 10.3.2.1.4.4-1: Near Field Test Range Uncertainty contributors in AAS EIS measurement

| UID | Description of uncertainty contribution | Details in paragraph | Uncertainty Value f≦3GHz | Uncertainty Value 3GHz≦f<4.2GHz |
| --- | --- | --- | --- | --- |
|  | Stage 1, EIS Near Field Radiation Pattern Measurement and EIS Near Field DUT power measurement |  |  |  |
| 1 | Axes Intersection | C.4.1 | 0 | 0 |
| 2 | Axes Orthogonality | C.4.2 | 0 | 0 |
| 3 | Horizontal Pointing | C.4.3 | 0 | 0 |
| 4 | Probe Vertical Position | C.4.4 | 0 | 0 |
| 5 | Probe H/V pointing | C.4.5 | 0 | 0 |
| 6 | Measurement Distance | C.4.6 | 0 | 0 |
| 7 | Amplitude and Phase Drift | C.4.7 | 0 | 0 |
| 8 | Amplitude and Phase Noise | C.4.8 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk | C.4.9 | 0 | 0 |
| 10 | Amplitude Non-Linearity | C.4.10 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints | C.4.11 | 0 | 0 |
| 12 | Channel Balance Amplitude and Phase | C.4.12 | 0 | 0 |
| 13 | Probe Polarization Amplitude and Phase | C.4.13 | 0.0001 | 0.0001 |
| 14 | Probe Pattern Knowledge | C.4.14 | 0 | 0 |
| 15 | Multiple Reflections | C.4.15 | 0 | 0 |
| 16 | Room Scattering | C.4.16 | 0.09 | 0.09 |
| 17 | DUT support Scattering | C.4.17 | 0 | 0 |
| 18 | Scan Area Truncation | C.4.18 | 0.003 | 0.003 |
| 19 | Sampling Point Offset | C.4.19 | 0.0058 | 0.0058 |
| 20 | Spherical Mode Truncation | C.4.20 | 0.015 | 0.015 |
| 21 | Positioning | C.4.21 | 0.03 | 0.03 |
| 22 | Probe Array Uniformity | C.4.22 | 0.055 | 0.055 |
| 23 | Mismatch of transmitter chain (i.e. between receiving antenna and vector signal generator) | C.4.23 | 0.2 | 0.2 |
| 24 | Insertion loss of transmitter chain | C.4.24 | 0 | 0 |
| 25 | Uncertainty of the absolute gain of the probe antenna | C.4.25 | 0 | 0 |
| 26 | Vector Signal Generator a)uncertainty of the absolute power level b)stability c)stability over temperature d)linearity | C.4.26 | FFS | FFS |
| 27 | Measurement repeatability – Positioning Repeatability | C.4.27 | 0.15 | 0.15 |
|  | Stage 2, Calibration measurement |  |  |  |
| 28 | Uncertainty of network analyser a)drift (temp, oscillators, filters, etc.) start to end time of measurements | C.4.28 | FFS | FFS |
| 29 | Mismatch of transmitter chain | C.4.29 | 0.05 | 0.05 |
| 30 | Insertion loss of transmitter chain | C.4.30 | 0 | 0 |
| 31 | Mismatch in the connection of the calibration antenna | C.4.31 | 0.014 | 0.014 |
| 32 | Influence of the calibration antenna feed cable | C.4.32 | 0 | 0 |
| 33 | Influence of the probe antenna cable | C.4.33 | 0 | 0 |
| 34 | Uncertainty of the absolute gain of the calibration antenna | C.4.34 | FFS | FFS |
| 35 | Short term repeatability | C.4.35 | 0.088 | 0.088 |
|  | Expanded uncertainty (2σ - confidence interval of 95%) |  | FFS | FFS |

###### 10.3.2.1.4.5	Uncertainty assessment

Text to be added