3GPP TSG-RAN WG4 Meeting #80	R4-165035

Gothenburg, Sweden, 22 - 26 Aug, 2016

Agenda Item:	8.10.3.3

Source: 	Intel Corporation

Title: 	Network-assisted signaling for performance enhancement in high speed scenarios

Document for:	Discussion

- Introduction

The work item (WI) “Performance enhancements for high speed scenario” was approved at RAN plenary meeting #70 [1]. One objective of this WI is to specify higher-layer assistance signalling for UEs complying with the enhanced downlink demodulation performance requirements under the high speed scenarios.

In RAN4#78bis Meeting, a network-assisted signalling was suggested [2], which can be used to provide assistance and indicate a high speed train (HST) that it is entering/leaving a HST SFN area. Because of the different properties between the HST SFN channel model and conventional fading channels model [3]–[8], such as EVA channels, it would be beneficial for the UE to know if it is under the HST SFN environment or not. In this contribution, we continue the discussion on the network-assisted signalling to indicate that a train is within the HST SFN area, and to help UE achieve satisfactory demodulation performance.

Moreover, our companion contribution [9] provides more details on network-assisted signalling for HST from RRM requirements perspective. The existing HST RRM signalling can be re-used for demodulation improvement purpose.

- Network-assisted signalling for HST SFN

As shown in our companion contribution [9], a preferable network-assisted signalling option is to re-use the existing HST signalling as possible. For example, the new field “HSTIndictor” in SIB4 below can indicate the neighbor cells to be measured which are deployed along with the high speed train. Therefore, UE could reliably justify its mobility status depending on this indicator.

# SystemInformationBlockType4 information element

-- ASN1START

SystemInformationBlockType4 ::=		SEQUENCE {

…

IntraFreqNeighCellList ::=		SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo

IntraFreqNeighCellInfo ::=		SEQUENCE {

physCellId								PhysCellId,

q-OffsetCell							Q-OffsetRange,

HSTIndicator				{true, false}

...

}

e

-- ASN1STOP

Depending on whether a “HST” signalling is received, the UE may decide to start or stop the blind detection for its mobility status, such as its instantaneous Doppler shift. The blind detection procedure is fully UE implementation issue.

Moreover, it has been shown that the throughput of a UE using legacy channel estimation approach (“legacy UE”) may degrade by about 30% when it is running at a speed as high as 350km/h in SFN deployment [5]. Therefore, a number of modified/enhanced channel estimation approaches have been developed in this WI tailored for high speed SFN scenarios. For example, in [11], it shows that modified channel estimation approach (“modified UE”) can considerably improve demodulation performance in HST SFN scenarios. Therefore, if a network-assistant “HST” signalling is provided, the UE can accordingly switch between “legacy” and “modified” channel estimation approaches, and know when to start/stop blind detection for its mobility status.

Proposal: A single HST scenario indicator is necessary and useful for demodulation performance enhancement purpose.

- Conclusion

In this contribution, we continue the discussion on the network-assisted signalling to indicate that a train is within the HST SFN area, and to help UE achieve satisfactory demodulation performance.

Proposal: A single HST scenario indicator is necessary and useful for demodulation performance enhancement purpose.

- Reference

[1] RP-152263, “New WI proposal: Performance enhancements for high speed scenario”, NTT DOCOMO INC., Huawei, HiSilicon, Dec. 2015.

[2] R4-161635, “Network assistance for performance enhancement in high speed scenarios”, Intel Corporation, April. 2016.

[3] R4-156999, “Discussion on UE demodulation performances for high speed train scenario with SFN deployment”, Intel Corporation, Nov. 2015.

[4] R4-157944, “Improvement in UE demodulation performance in SFN channel”, Qualcomm Incorporated, Nov. 2015.

[5] R4-156001, “UE demodulation performance evaluation of the SFN channel”, Ericsson, Oct. 2015.

[6] R4-155657, “Evaluation of UE demodulation performance under the identified SFN scenario”, Huawei, HiSilicon, Oct. 2015.

[7] TR-36.878, “Study on performance enhancements for high speed scenario in LTE”, Dec. 2015.

[8] RP-161252, “Way forward on UE performance enhancement under SFN channel”, Huawei, HiSilicon, NTT DoCoMo, Ericsson, Qualcomm, Intel, Feb. 2016.

[9] R4-165032, “Discussion on the network assistant signaling for enhanced RRM requirements in HST”, Intel Corporation, August 2016.

[10] 3GPP TS 36.331 v13.0.0: "Evolved Universal Terrestrial Radio Access (E-UTRA): RRC".

[11] R4-165033, “Discussion on MCS and scenario selection, and UE demodulation performance for bidirectional HST SFN channels”, Intel Corporation, August 2016.