3GPP TSG-RAN WG4 Meeting #80	R4-165169

Gothenburg, Sweden, 22 – 26 August 2016

Agenda Item:	5.14.3

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	NB-IoT guard-band operation with 5 MHz E-UTRA channel bandwidth

Document for:	Discussion

# 1.	Introduction

The revised WI proposal to support Narrowband Internet of Things (NB-IoT) was approved at RAN#70 [1]. One objective of the WI is to specify the core requirements for Base Station (BS) radio transmission and reception for NB-IoT.

The NB-IoT BS core requirements were completed in RAN4#79. It was discussed in RAN4 that NB-IoT guard-band operation with ≤ 5 MHz E-UTRA channel bandwidth would be challenging [2-4], and it was agreed [5] that ‘For guard-band operation mode with 5 MHz CBW, in Release 13 it is decided manufacturer can declare any power boosting level which BS can perform, while in Release 14 specific boosting level may be specified. Guard-band operation mode with 1.4/3 MHz CBW is FFS.’

As power boosting is required to enhance the NB-IoT coverage, a smaller or zero (or even negative) power boosting level may negatively  impact the practical network deployment of NB-IoT. In this paper, we discuss other alternatives to allow NB-IoT guard-band operation with 5 MHz E-UTRA channel bandwidth, instead of using a manufacturer declared level of power boosting.

# 2.	Discussion

The restrictions for guard band NB-IoT’s filter pass-band extension are no serious problem with 10, 15 or 20 MHz E-UTRA channel bandwidth. But other E-UTRA options with lower channel bandwidth have too little room to host one NB-IoT channel inside its guard band. For instance, nominal 1.4 and 3 MHz E-UTRA options only have, respectively, 160 kHz and 150 kHz guard-band at each side of the Transmission Bandwidth Configuration, thus won’t have sufficient guard-band to host the 180 kHz NB-IoT carrier.

Figure 1 illustrates the situation for the 5 MHz E-UTRA DL option. The outermost E-UTRA subcarrier is centered at 2250 kHz off the E-UTRA center frequency. In order to be aligned with the 100 kHz channel raster (-7.5 kHz offset is tolerated), NB-IoT is centered at 2392.5 kHz from the E-UTRA center. The furthest distant 15 kHz NB-IoT tone is 2475 kHz off the E-UTRA center. Thus, the upper pass band edge is 2.4825 MHz relative to E-UTRA center.

Since filter stop-band requirements shall be fulfilled at 2.5 MHz, only 17.5 kHz are left for filter roll-off. This would require extremely a long filter impulse response with a lot of filter taps. Even if this huge effort was spent in hardware, the long impulse response would eat up the cyclic prefix and create multiple inter-symbol-interference.

|  | E-UTRA 5 MHz |  |
| --- | --- | --- |
| subcarrier |  |  |
| frequency |  |  |
| offset in kHz |  |  |
| to the center |  |  |
| frequency of |  |  |
| the 5 MHz E-UTRA | end of last |  |
| carrier | PRB of |  |
| 2250 | 5 MHz E-UTRA |  |
| 2265 | 3 unused |  |
| 2280 | subcarriers |  |
| 2295 | (45 kHz) |  |
| 2310 |  |  |
| 2325 |  |  |
| 2340 | NB-IOT |  |
| 2355 | subcarriers |  |
| 2370 |  |  |
| 2385 |  | 2392.5 |
| 2400 | Offset of |  |
| 2415 | -7.5 kHz to |  |
| 2430 | the channel |  |
| 2445 | raster |  |
| 2460 |  |  |
| 2475 |  |  |

Figure 1: Closest NB-IoT guard-band channel location with 5 MHz E-UTRA DL

To solve this problem for DL, a frequency shift of -100 kHz can be applied to the 5 MHz E-UTRA Transmission Bandwidth Configuration and consequently also to the NB-IoT carrier [4]. As a result, both carriers are still centered on the 100 kHz channel raster. Instead of 17.5 kHz before, now 117.5 kHz are available for filter roll-off, which is a feasible value. On the other side of the 5 MHz E-UTRA channel without NB-IoT carrier, the -100 kHz shift decreases the range for roll-off from 242.5 kHz to 142.5 kHz. Consequently, even a symmetric filter with 117.5 kHz filter roll-off can be applied.

Though this option constitutes a valid solution for DL, the situation for UL is not satisfactory. A shift of -100 kHz may cause significant interference to the adjacent frequency block. Since the legacy E-UTRA UEs are allowed to occupy a 5MHz block, no frequency shift can be applied to them.

Usually UL is set up with a fixed duplex distance. This duplex distance applies to all carriers of one band in the same way. However, in the case of NB-IoT guard-band operation with 5 MHz E-UTRA, the gap of 4 unused subcarriers (as shown in Figure 2) is a waste of spectrum. On the other hand, eNodeB may signal a different duplex distance, e.g. via E-UTRA system information, to the UE for the UL which compensates the frequency shift of -100 kHz for the 5 MHz E-UTRA UL and/or closes above gap between the E-UTRA and NB-IoT carriers. Consequently, the E-UTRA UL would be centered in the middle of its original 5 MHz block and/or may sit adjacent to the E-UTRA UL according to the frequency offset. This extends the roll-off region from 10 kHz to 70 kHz. If PUCCH over-provisioning is enabled for the E-UTRA UL, the NB-IoT UL may shift even further towards the E-UTRA center.

|  | E-UTRA |  |
| --- | --- | --- |
|  | 5 MHz |  |
| subcarrier |  |  |
| frequency |  |  |
| offset in kHz |  |  |
| to the center |  |  |
| frequency of the |  |  |
| 5 MHz E-UTRA | end of last |  |
| UL carrier | PRB of |  |
| 2242.5 | 5 MHz E-UTRA |  |
| 2257.5 | 4 unused |  |
| 2272.5 | subcarriers |  |
| 2287.5 | (60 kHz) |  |
| 2302.5 |  |  |
| 2317.5 |  |  |
| 2332.5 |  |  |
| 2347.5 | NB-IOT |  |
| 2362.5 | subcarriers |  |
| 2377.5 |  |  |
| 2392.5 |  | 2400 |
| 2407.5 | Offset of |  |
| 2422.5 | 0 kHz to |  |
| 2437.5 | the channel |  |
| 2452.5 | raster |  |
| 2467.5 |  |  |
| 2482.5 |  |  |

Figure 2: Closest NB-IoT guard-band channel location with 5 MHz E-UTRA UL

Alternatively, the offset in the duplex distance may be utilized to cause overlapping between the 5 MHz E-UTRA UL and NB-IoT UL frequency region. Because of asymmetric load in the hosting E-UTRA system, there may be unused E-UTRA UL resource blocks, or even NB-IoT in-band operation may be performed instead of NB-IoT guard-band operation for the UL. For example, if three 15 kHz subcarriers from the E-UTRA and/or NB-IoT carrier are blanked out, the NB-IoT UL would sit adjacent to E-UTRA UL, and the NB-IoT UL channel can be shifted -100 kHz which extends the roll-off region from 10 kHz to 110 kHz.

# 3.	Conclusion

In this paper, we have discussed other alternatives to allow NB-IoT guard-band operation with 5 MHz E-UTRA channel bandwidth, instead of using a manufacturer declared level of power boosting which may negatively  impact the practical network deployment of NB-IoT.

# References

[1]	RP-152284, “Revised Work Item: Narrowband IoT (NB-IoT)”, Huawei, HiSilicon.

[2]	R4-78AH-0007, 	“Proposal on BS emission masks for NB-IoT guard-band operation with LTE channel bandwidth of 1.4/3/5 MHz”, Nokia, Alcatel-Lucent Shanghai Bell.

[3]	R4-78AH-0204, 	“TP for TR36.802: BS OBUE for guard-band operation”, NTT DOCOMO, INC.

[4]	R4-162098, “NB-IoT guard band operation for small LTE bandwidth”, Ericsson.