3GPP TSG-RAN WG4 (Radio) Meeting #80	R4-165179

Gothenburg, Sweden, 22 – 26 August 2016

Agenda Item:	7.9.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	TP for TR 37.871: Necessary changes to the core requirements for Multi-Band Base Station testing with three or more bands

Document for:	Approval

# 1.	Introduction

The Multi-Band (MB) Base Station (BS) testing with three or more bands work item was approved at RAN#70 [1]. The necessary changes to the core requirements were discussed at RAN4#78 [2, 3] and RAN4#78bis [4-6] where a text proposal was approved [7], and further discussed in RAN4#79 [8, 9] where a text proposal was approved [10].

During the discussion at RAN4#79, another potentially necessary change to the core requirement was identified. In this contribution, we discuss how to implement the identified changes (if necessary) in the specifications, and provide a text proposal to include the proposals into the TR [11] of this work item.

# 2.	Discussion

It was discussed at RAN4#79 that the statement (extracted from clause 4.8 of TS 37.104 [12]) below means that the listed single-band requirements apply to each antenna connector, even when multiple bands (e.g. two out of three supported bands) are mapped on one antenna connector while the other supported band is mapped on another connector. Indeed in this case exclusions or provisions for multi-band capable BS are applicable to the antenna connector that is mapped to multiple bands.

In the case where multiple bands are mapped on separate antenna connectors, the following applies:

-	Single-band transmitter spurious emissions, operating band unwanted emissions, ACLR, transmitter intermodulation and receiver spurious emissions requirements apply to each antenna connector.

Note that this would only be an issue for BS that supports more than two bands, otherwise only one of the two supported bands is mapped on each antenna connector.

To solve the identified issue, it is proposed to update the statement above into the following to clarify that the listed single-band requirements apply only to each antenna connector that is mapped to single-band:

In the case where multiple bands are mapped on separate antenna connectors, the following applies:

-	Single-band transmitter spurious emissions, operating band unwanted emissions, ACLR, transmitter intermodulation and receiver spurious emissions requirements apply to each antenna connector that is mapped to single-band.

# 3.	Conclusion

In this contribution, we have discussed how to implement the identified changes (if necessary) in the specifications. It is proposed to include the text proposal below into TR 37.871.

# 4.	Text proposal

<Start of change>

5.3	Operating band unwanted emissions

It was discussed at RAN4#79 that the statement (extracted from clause 4.8 of TS 37.104 [3]) below means that the listed single-band requirements apply to each antenna connector, even when multiple bands (e.g. two out of three supported bands) are mapped on one antenna connector while the other supported band is mapped on another connector. Indeed in this case exclusions or provisions for multi-band capable BS are applicable to the antenna connector that is mapped to multiple bands.

In the case where multiple bands are mapped on separate antenna connectors, the following applies:

-	Single-band transmitter spurious emissions, operating band unwanted emissions, ACLR, transmitter intermodulation and receiver spurious emissions requirements apply to each antenna connector.

Note that this would only be an issue for BS that supports more than two bands, otherwise only one of the two supported bands is mapped on each antenna connector.

To solve the identified issue, it is proposed to update the statement above into the following to clarify that the listed single-band requirements apply only to each antenna connector that is mapped to single-band:

In the case where multiple bands are mapped on separate antenna connectors, the following applies:

-	Single-band transmitter spurious emissions, operating band unwanted emissions, ACLR, transmitter intermodulation and receiver spurious emissions requirements apply to each antenna connector that is mapped to single-band.

<End of change>

# References

[1]	RP-152205, “New Work Item proposal: Multi-Band Base Station testing with three or more bands”, Alcatel-Lucent.

[2]	R4-160430, “Impact on core requirements”, Ericsson.

[3]	R4-160579, “Discussion on Multi-band definition”, Huawei.

[4]	R4-161599, “TP for TR 37.871: Necessary changes to the core requirements for Multi-Band Base Station testing with three or more bands”, Nokia, Alcatel-Lucent Shanghai Bell.

[5]	R4-162368, “Correction on multi-band definition”, Huawei.

[6]	R4-162513, “Impact on core requirements”, Ericsson.

[7]	R4-163133, “TP for TR 37.871: Necessary changes to the core requirements for Multi-Band Base Station testing with three or more bands”, Nokia, Alcatel-Lucent Shanghai Bell, Huawei.

[8]	R4-164120, TP for TR 37.871: Impact on core requirements, Ericsson.

[9]	R4-164193, TP for TR 37.871: Necessary changes to the core requirements for Multi-Band Base Station testing with three or more bands, Nokia, Alcatel-Lucent Shanghai Bell.

[10]	R4-164702, TP for TR 37.871: Necessary changes to the core requirements for Multi-Band Base Station testing with three or more bands, Nokia, Alcatel-Lucent Shanghai Bell, Huawei, Ericsson.

[11]	RP-160776, “TR 37.871 V1.0.0”, Nokia.

[12]	3GPP TR 36.104 v14.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.