3GPP TSG-RAN WG4 (Radio) Meeting #80	R4-165181

Gothenburg, Sweden, 22 – 26 August 2016

Agenda Item:	9.3.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	System level simulation methodology and assumptions for coexistence study for WP5D on new radio access technology

Document for:	Approval

# 1.	Introduction

The revised study item on New Radio Access Technology was approved at TSG RAN#72 [1]. The objectives of this study item include to identify relevant RF parameters to be used for sharing and co-existence studies. Also a LS was received at TSG RAN#72 [2] from ITU-R WP5D asking for characteristics of terrestrial IMT systems for frequency sharing/interference analysis in 24.25 - 86 GHz. This topic has been discussed at RAN4#78b and RAN4#79 meetings, where two way forward proposals were approved [3, 4]. ITU-R WP5D sent another LS on “Updated characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz” in [11]. On the other hand, a LS has been sent from ITU-R WP5D to 3GPP describing detailed modelling and simulation of IMT networks for use in sharing and compatibility studies [5].

This contribution proposes the system level simulation methodology and assumptions for coexistence study for WP5D on new radio access technology, and provide a text proposal to record the assumptions and methodology into the TR 38.803 [6] for this study item.

# 2.	Discussion

The simulation methodology and assumptions shall mainly be based on the recommendations from ITU-R WP5D [5], the Radio Frequency (RF) system scenarios for macro cellular network in 3GPP TR 36.942 [7] and for micro-cellular network in 3GPP TR 37.809 [8], the channel model for frequency spectrum above 6 GHz in 3GPP TR 38.900 [9], and the antenna array model in 3GPP TR 37.842 [10], with slight modifications to better represent scenarios and requirements for new radio access technology.

#### 2.1	Cell layouts

#### 2.1.1	Macro cellular network [7]

Base stations with 3 sectors per site are placed on a hexagonal grid with distance of 3*R, where R is the cell radius (see Figure 2.1.1-1), with wrap around. The number of sites shall be equal to or higher than 19. Uncoordinated macro cellular deployment is assumed, where interfering UE may be at cell edge of the serving base station but close to the victim base station (hence transmitting with highest power and causing highest interference). The inter-site distance (ISD) is 500m or 1732 m.

Figure 2.1.1-1: Uncoordinated macro cellular deployment

#### 2.1.2	Micro cellular network [8]

A modified version of the Manhattan grid pattern from the ITU-R WP5D recommendations is used as a micro cell layout. The proposed modification produces a symmetrical grid pattern that can be replicated into a larger grid. An additional column with 3 new nodes has been added resulting in a 12 by 12 block grid, as shown in Figure 2.1.2-1.

In addition, 3 nodes on the top of the grid have been removed since they are redundant with the nodes on the bottom of the original grid when wrap-around is employed. As per the Manhattan grid definition in 3GPP TR 37.809, each block is 75m x 75m and each street is 15m wide. This results in the modified grid having a dimension of 1080m x 1080m.

Figure 2.1.2-1: Micro cellular Manhattan grid pattern

To ensure that at least one macro cell is covered when the micro cells are overlaid on the larger macro cell layout with ISD 1732 m, a larger Manhattan is grid used, formed by a four-fold replication of the Manhattan grid in Figure 2.1.2-1. This extended grid is shown below in Figure 2.1.2-2.

Figure 2.1.2-2: Four-fold replication of modified grid pattern of Figure 2.1.2-1 above.

#### 2.1.3	Indoor hotspot [9]

In this scenario, the base stations are deployed indoors. The indoor hotspot scenario consists of one floor of a building. The detailed topology of indoor office scenarios is shown in Figure 2.1.3-1.

Figure 2.1.3-1: Layout of indoor office scenarios

#### 2.1.4	Heterogeneous network [8]

For micro-to-macro and macro-to-micro simulations, the following layouts are used for the overlaid patterns:

-	In case of an ISD of 500 m for the macro network, the layout of Figure 2.1.4-1 is used, with the 12 column micro grid pattern centred over the corner between cell#3/19/20 of the macro network.

-	In case of an ISD of 1732 m for the macro network, the layout of Figure 2.1.4-2 is used, with the extended (four-fold) micro grid pattern centred over the corner between cell#3/19/20 of the macro network.

For these simulations, in the area that is overlapped by the Micro network’s Manhattan grid, UEs should only be dropped in the streets. In order to maintain the same Macro UE density per cell as in the area not covered by the Micro network, the following principle should be used:

1.	Macro UEs are first dropped with a uniform distribution over the whole Macro grid area, both in the part not covered by the micro network and in the part that is overlapped by the micro network.

2.	In the area overlapping with the micro network, UEs that were dropped indoor are identified. These UEs are then dropped again, uniformly over the outdoor “half-street” area surrounding the indoor block, as shown in Figure 2.1.4-3.

Figure 2.1.4-1: Manhattan grid micro node network overlaid on a Case 1 (ISD = 500 m) macro network. The Manhattan grid is centred on the corner between cell#3/19/20 in the macro network.

Figure 2.1.4-2: Four-fold replication of the Manhattan grid micro node network overlaid on a Case 3 (ISD = 1732 m) macro network. The overlaid grid is centered on corner between cell#3/19/20 of the macro network.

Figure 2.1.4-3: Method of re-distributing UEs from the indoor to the street area.

#### 2.2	Path loss models [9]

The path loss models are summarized in Table 2.2-1 and the distance definitions are indicated in Figure 2.2-1 and Figure 2.2-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 2.2-1.

| Figure 2.2-1: Definition of d2D and d3D for outdoor UEs | Figure 2.2-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UEs. Note that d3D-out+d3D-in=d2D-out+d2D-in2+hBS-hUT2 |
| --- | --- |

Table 2.2-1: Path loss models

| Scenario | Path loss [dB], fc is in GHz and d is in meters (6) |  | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| RMa LOS |  |  |  | 10 m < d2D < dBP (5) dBP < d2D < 10 000 m, hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 5 m h = avg. building height W = avg. street width The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1 m < hUT < 10 m |
| RMa NLOS |  |  |  | 10 m < d2D < 5 000 m, hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 5 m h = avg. building height W = avg. street width The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1 m < hUT < 10 m |
| UMa LOS |  |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 25 m |
| UMa NLOS |  |  | σSF =6 | 10 m < d2D < 5 000 m 1.5 m ≦ hUT ≦ 22.5 m hBS = 25 m Explanations: see note 3 |
|  | Optional |  | σSF =7.8 |  |
| UMi - Street Canyon LOS |  |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
|  | Optional |  | σSF=8.2 |  |
| InH - Office LOS |  |  | σSF=3.0 | 1<d3D<100m |
| InH - Office NLOS |  |  | σSF=8.03 | 1<d3D<86m |
|  | Optional |  | σSF=8.29 | 1< d3D <86m |
| InH - Shopping mall LOS |  |  | σSF=2.0 | 1<d3D<150m |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa path loss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS path loss is from TR36.873 with simplified format and and PLUMa-LOS = Path loss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Path loss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

For UMa outdoor users following terms are defined in 3GPP TR 38.900:

and

#### 2.3	Beamforming antenna pattern [10]

The beamforming antenna is based on an antenna array and consists of a number of identical radiating elements located in the yz-plane with a fix separation distance (e.g. /2), all elements having identical radiation patterns and “pointing” (having maximum directivity) along the x-axis. A weighting function is used to direct the beam in other directions. Total antenna gain is the sum (logarithmic scale) of the array gain and the element gain. Such a model is described in 3GPP TR 37.842 as follows.

The Advanced Antenna System (AAS) antenna array model is determined by array element pattern, array factor and signals applied to the system. The element pattern and composite antenna pattern are in the following sections. The  and definition is based on the coordinate system in Section 5.4.4 .1 of 3GPP TR 37.840 and are illustrated in Figure 2.3-1.

dH

z

u

dv

φ

θ

x

y

……

Figure 2.3-1: Antenna Model Geometry: : elevation, range from 0 to 180 degree
 : Azimuth, range from -180 to 180 degree

The element pattern and the composite antenna pattern are given in Section 5.3.3 of 3GPP TR 37.842 and are provided Table 2.3-1.

Table 2.3-1: Antenna pattern

(a) Element pattern for antenna array model

| Horizontal Radiation Pattern |  |
| --- | --- |
| Horizontal 3dB bandwidth of single element / deg | For single column antenna: 65º For multi-column antenna: 80º |
| Front-to-back ratio | Am = 30dB |
| Vertical Pattern method | , SLAv =30 dB |
| Vertical 3dB bandwidth of single element / deg | 65º |
| Element Pattern |  |
| Element Gain without antenna losses | For single column antenna: GE,max= 9 dBi For multi-column antenna: GE,max= 7.5 dBi* |
| Note: *GE,max is obtained from TR37.840 Table 5.4.4.2.1-1. |  |

(b) Composite antenna pattern for BS and UE beam forming

| Configuration | Multiple columns (NVxNH elements) |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the super position vector is given by: the weighting is given by: |
| Antenna array configuration (Row×Column) | 10×4 |
| Horizontal radiating element spacing d/ | 0.5 |
| Vertical radiating element spacing d/ | 0.9 |
| Down-tilt angle (deg) | 0 degrees |
| Power allocated to each beam | 6dB below the maximum transmitted power |

#### 2.4	Power control modelling [7]

In 3GPP TR 36.942, the following power control equation is used for the uplink coexistence simulations:

(1)

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadow fading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax. Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel.

The parameter sets for power control are specified in 3GPP TR 36.942 (copied in Table 2.4-1 below).

Table 2.4-1: Power control algorithm parameter (for 2 GHz carrier frequency)

| Parameter set | Gamma | CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 109 | 110 | 112 | 115 |
| Set 2 | 0,8 | TBD | TBD | 129 | 133 |

Note that the power control parameters were specified for urban area with a cell range of 500 m, a carrier frequency of 2 GHz and a base station antenna height of 15 m above average rooftop level. The path loss model is based on the equation below:

(2)

where:

R is the base station-UE separation in kilometres

f is the carrier frequency in MHz

Dhb is the base station antenna height in metres, measured from the average rooftop level

Based on equations (2), the power control parameters can be modified to account for different path loss model and carrier frequency. For example, in Table 2.4-1, CLx-ile = 109 dB for Set 1and 20 MHz bandwidth, which corresponds to R = 0.31 km that is (0.31/0.5 =) 62% of the cell range according to equation (2) assuming total antenna gain of 0 dBi.

Considering a carrier frequency of 30 GHz and a base station-UE separation of (500*2/3*62% =) 207 m and a base station antenna height of 15 m above average rooftop level, the path loss using the UMa LOS PL1 formula in Table 2.4-1 will result in a path loss of 32.4+20log10(sqrt(2072+152))+20log10(30) = 108 dB. Similarly, the modified power control parameters for other cases with carrier frequency of 30 GHz and 70 GHz can be calculated as shown in Table 2.4-2 below.

Table 2.4-2: Modified power control algorithm parameter

(a) For 30 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1a | 1 | 108 | 109 | 110 | 111 |
| Set 2a | 0,8 | TBD | TBD | 119 | 121 |

(b) For 70 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1a | 1 | 116 | 116 | 117 | 119 |
| Set 2a | 0,8 | TBD | TBD | 126 | 128 |

#### 2.5	Other simulation assumptions and parameters [4]

Other simulation assumptions and parameter are summarized in Table 2.5-1 below:

Table 2.5-1: Simulation assumptions and parameters

(a) Main simulation assumptions

|  | Way forward | Remarks |
| --- | --- | --- |
| Methodology | Static MonteCarlo simulation |  |
| Usage scenario (eMBB, mMTC, URLLC) | eMBB | Start with eMBB as 1st step. We shall consider the other scenarios in SI. |
| Deployment scenario | At least, Indoor hotspot, Urban macro and Dense urban are the scenarios for WP5D co-existence. | The other scenarios are FFS. |
| Victim system | NR |  |
| PathLoss Model | Follow RAN1 channel modelling SI outcome |  |
| Carrier frequency | 30 and 70 GHz | Using 28GHz is not precluded in addition to 30 and 70GHz. |
| Channel Bandwidth | 200 MHz | Even wider channel bandwidth for example 1GHz can be considered later after understanding the baseline in SI |
| Channel arrangement | Symmetrical (200MHz vs 200MHz) | Start with symmetrical as 1st step |
| UL power control | The same as TR 36.942, adopting PC set parameters to the scenario |  |
| DL power control | No power control |  |
| The number of antenna element | For 30GHz: UE: 16, eNB: 256 For 70GHz: UE: 32, eNB: 256 | Companies are encouraged to provide technical justification on these values. |
| UE antenna | Use Table 2.3-1 (a), element gain and antenna aperture TBD. |  |
| BS antenna | Use Table 2.3-1 (a). |  |
| Beamforming | Use Table 2.3-1 (b). |  |
| SINR to throughput mapping | Adopt scaled Shannon's formula with update truncation and attenuation parameters until we get RAN1, we will use 36.942 Scaled Shannon's formula. |  |
| Co-existence scenario in terms of interference | UL to UL and DL to DL | Not consider UE to DL and DL to UL for the moment. Specifically for UE to UE, this can be discussed after understanding specific mitigation techniques. |
| Traffic | Full buffer |  |
| ACI leakage model | In addition, model in ACLR is two step models in frequency use the different correlation levels to study the spatial patterns for ACLR. ACLR metric defines integrated over the whole space. If study should ACLR can be modelled as flat in space, then, we just use integrated ACLR model. Assume flat ACS in frequency. In space, study if ACS model is flat. |  |
| Performance metric | Throughput degradation compared to single operator case, i.e. no ACI. Both mean and 5%-tile are considered. |  |
| NF | For BS: 8 dB For UE: 13 dB for baseline performance, 10 dB for high performance | Companies are encourage to provide more analysis data on this aspect in the future meeting. |

(b) Other simulation parameters

|  | BS |  | UE |
| --- | --- | --- | --- |
| Lognormal fading | 10 dB |  |  |
| Shadowing correlation | Between sites: 0.5, between sectors of the same site: 1.0 |  |  |
| Minimum distance UE<->BS (2D) | RMa, UMa: 35 m UMi: 10 m InH: 0 m |  |  |
| Number of active UEs | DL: 1 UE, UL: 4 UEs |  |  |
| Handover margin | 3 dB |  |  |
| Cable loss | 1 dB | 0 dB |  |
| Transmit power | RMa, UMa: 46 dBm UMi: 38 dBm InH: 20 dBm |  | 23 dBm |
| Antenna height | RMa: 45 m UMa: 25 m UMi: 10 m InH: 3 m |  | 1.5 m |

# 3.	Conclusion

This contribution proposes the system level simulation methodology and assumptions for coexistence study for WP5D on new radio access technology. It is proposed to include the text proposal below into TR 36.8xx.

# 4.	Text proposal

<Start of change>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP RP-160671: "New SID Proposal: Study on New Radio Access Technology".

[2]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[3]	3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

[4]	3GPP TR 37.809 v11.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA) medium range and Multi-Standard Radio (MSR) medium range / local area Base Station (BS) class requirements”.

[5]	RP-161301, “TR38.900, Version 2.0.0”, Samsung.

[6]	3GPP TR 37.809 v11.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA) medium range and Multi-Standard Radio (MSR) medium range / local area Base Station (BS) class requirements”.

[7]	R4-157516, “TR 37.842 v1.8.0”, Huawei.

[8]	R4-164528, “WF on WP 5D co-existence simulation assumptions”, NTT DOCOMO, INC.

<Next change>

## 5.1	Co-existence simulation scenario

Editor’s note: intended to capture specific scenarios such as operation system (eMBB, Massive MTC etc) systems in terms of aggressor system, victim system and their directions

#### 5.1.1	Macro cellular network [3]

Base stations with 3 sectors per site are placed on a hexagonal grid with distance of 3*R, where R is the cell radius (see Figure 5.1.1-1), with wrap around. The number of sites shall be equal to or higher than 19. Uncoordinated macro cellular deployment is assumed, where interfering UE may be at cell edge of the serving base station but close to the victim base station (hence transmitting with highest power and causing highest interference). The inter-site distance (ISD) is 500m or 1732 m.

Figure 5.1.1-1: Uncoordinated macro cellular deployment

#### 5.1.2	Micro cellular network [4]

A modified version of the Manhattan grid pattern from the ITU-R WP5D recommendations is used as a micro cell layout. The proposed modification produces a symmetrical grid pattern that can be replicated into a larger grid. An additional column with 3 new nodes has been added resulting in a 12 by 12 block grid, as shown in Figure 5.1.2-1.

In addition, 3 nodes on the top of the grid have been removed since they are redundant with the nodes on the bottom of the original grid when wrap-around is employed. As per the Manhattan grid definition in 3GPP TR 37.809, each block is 75m x 75m and each street is 15m wide. This results in the modified grid having a dimension of 1080m x 1080m.

Figure 5.1.2-1: Micro cellular Manhattan grid pattern

To ensure that at least one macro cell is covered when the micro cells are overlaid on the larger macro cell layout with ISD 1732 m, a larger Manhattan is grid used, formed by a four-fold replication of the Manhattan grid in Figure 5.1.2-1. This extended grid is shown below in Figure 5.1.2-2.

Figure 5.1.2-2: Four-fold replication of modified grid pattern of Figure 5.1.2-1 above.

#### 5.1.3	Indoor hotspot [5]

In this scenario, the base stations are deployed indoors. The indoor hotspot scenario consists of one floor of a building. The detailed topology of indoor office scenarios is shown in Figure 5.1.3-1.

Figure 5.1.3-1: Layout of indoor office scenarios

#### 5.1.4	Heterogeneous network [6]

For micro-to-macro and macro-to-micro simulations, the following layouts are used for the overlaid patterns:

-	In case of an ISD of 500 m for the macro network, the layout of Figure 5.1.4-1 is used, with the 12 column micro grid pattern centred over the corner between cell#3/19/20 of the macro network.

-	In case of an ISD of 1732 m for the macro network, the layout of Figure 5.1.4-2 is used, with the extended (four-fold) micro grid pattern centred over the corner between cell#3/19/20 of the macro network.

For these simulations, in the area that is overlapped by the Micro network’s Manhattan grid, UEs should only be dropped in the streets. In order to maintain the same Macro UE density per cell as in the area not covered by the Micro network, the following principle should be used:

1.	Macro UEs are first dropped with a uniform distribution over the whole Macro grid area, both in the part not covered by the micro network and in the part that is overlapped by the micro network.

2.	In the area overlapping with the micro network, UEs that were dropped indoor are identified. These UEs are then dropped again, uniformly over the outdoor “half-street” area surrounding the indoor block, as shown in Figure 5.1.4-3.

Figure 5.1.4-1: Manhattan grid micro node network overlaid on a Case 1 (ISD = 500 m) macro network. The Manhattan grid is centred on the corner between cell#3/19/20 in the macro network.

Figure 5.1.4-2: Four-fold replication of the Manhattan grid micro node network overlaid on a Case 3 (ISD = 1732 m) macro network. The overlaid grid is centered on corner between cell#3/19/20 of the macro network.

Figure 5.1.4-3: Method of re-distributing UEs from the indoor to the street area.

## 5.2	Co-existence simulation assumption

Editor’s note: intended to capture specific simulation parameters such as frequencies, antenna pattern cell layouts and so on.

#### 5.2.1	Path loss models [5]

The path loss models are summarized in Table 5.2.1-1 and the distance definitions are indicated in Figure 5.2.1-1 and Figure 5.2.1-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 5.2.1-1.

| Figure 5.2.1-1: Definition of d2D and d3D for outdoor UEs | Figure 5.2.1-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UEs. Note that d3D-out+d3D-in=d2D-out+d2D-in2+hBS-hUT2 |
| --- | --- |

Table 5.2.1-1: Path loss models

| Scenario | Path loss [dB], fc is in GHz and d is in meters (6) |  | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| RMa LOS |  |  |  | 10 m < d2D < dBP (5) dBP < d2D < 10 000 m, hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 5 m h = avg. building height W = avg. street width The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1 m < hUT < 10 m |
| RMa NLOS |  |  |  | 10 m < d2D < 5 000 m, hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 5 m h = avg. building height W = avg. street width The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1 m < hUT < 10 m |
| UMa LOS |  |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 25 m |
| UMa NLOS |  |  | σSF =6 | 10 m < d2D < 5 000 m 1.5 m ≦ hUT ≦ 22.5 m hBS = 25 m Explanations: see note 3 |
|  | Optional |  | σSF =7.8 |  |
| UMi - Street Canyon LOS |  |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
|  | Optional |  | σSF=8.2 |  |
| InH - Office LOS |  |  | σSF=3.0 | 1<d3D<100m |
| InH - Office NLOS |  |  | σSF=8.03 | 1<d3D<86m |
|  | Optional |  | σSF=8.29 | 1< d3D <86m |
| InH - Shopping mall LOS |  |  | σSF=2.0 | 1<d3D<150m |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa path loss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS path loss is from TR36.873 with simplified format and and PLUMa-LOS = Path loss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Path loss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

For UMa outdoor users following terms are defined in 3GPP TR 38.900:

and

#### 5.2.2	Beamforming antenna pattern [7]

The beamforming antenna is based on an antenna array and consists of a number of identical radiating elements located in the yz-plane with a fix separation distance (e.g. /2), all elements having identical radiation patterns and “pointing” (having maximum directivity) along the x-axis. A weighting function is used to direct the beam in other directions. Total antenna gain is the sum (logarithmic scale) of the array gain and the element gain. Such a model is described in 3GPP TR 37.842 as follows.

The Advanced Antenna System (AAS) antenna array model is determined by array element pattern, array factor and signals applied to the system. The element pattern and composite antenna pattern are in the following sections. The  and definition is based on the coordinate system in Section 5.4.4 .1 of 3GPP TR 37.840 and are illustrated in Figure 5.2.2-1.

dH

z

u

dv

φ

θ

x

y

……

Figure 5.2.2-1: Antenna Model Geometry: : elevation, range from 0 to 180 degree
 : Azimuth, range from -180 to 180 degree

The element pattern and the composite antenna pattern are given in Section 5.3.3 of 3GPP TR 37.842 and are provided Table 5.2.2-1.

Table 5.2.2-1: Antenna pattern

(a) Element pattern for antenna array model

| Horizontal Radiation Pattern |  |
| --- | --- |
| Horizontal 3dB bandwidth of single element / deg | For single column antenna: 65º For multi-column antenna: 80º |
| Front-to-back ratio | Am = 30dB |
| Vertical Pattern method | , SLAv =30 dB |
| Vertical 3dB bandwidth of single element / deg | 65º |
| Element Pattern |  |
| Element Gain without antenna losses | For single column antenna: GE,max= 9 dBi For multi-column antenna: GE,max= 7.5 dBi* |
| Note: *GE,max is obtained from TR37.840 Table 5.4.4.2.1-1. |  |

(b) Composite antenna pattern for BS and UE beam forming

| Configuration | Multiple columns (NVxNH elements) |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the super position vector is given by: the weighting is given by: |
| Antenna array configuration (Row×Column) | 10×4 |
| Horizontal radiating element spacing d/ | 0.5 |
| Vertical radiating element spacing d/ | 0.9 |
| Down-tilt angle (deg) | 0 degrees |
| Power allocated to each beam | 6dB below the maximum transmitted power |

#### 5.2.3	Power control modelling [3]

In 3GPP TR 36.942, the following power control equation is used for the uplink coexistence simulations:

(1)

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadow fading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax. Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel.

The parameter sets for power control are specified in 3GPP TR 36.942 (copied in Table 5.2.3-1 below).

Table 5.2.3-1: Power control algorithm parameter (for 2 GHz carrier frequency)

| Parameter set | Gamma | CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1 | 1 | 109 | 110 | 112 | 115 |
| Set 2 | 0,8 | TBD | TBD | 129 | 133 |

Note that the power control parameters were specified for urban area with a cell range of 500 m, a carrier frequency of 2 GHz and a base station antenna height of 15 m above average rooftop level. The path loss model is based on the equation below:

(2)

where:

R is the base station-UE separation in kilometres

f is the carrier frequency in MHz

Dhb is the base station antenna height in metres, measured from the average rooftop level

Based on equations (2), the power control parameters can be modified to account for different path loss model and carrier frequency. For example, in Table 5.2.3-1, CLx-ile = 109 dB for Set 1and 20 MHz bandwidth, which corresponds to R = 0.31 km that is (0.31/0.5 =) 62% of the cell range according to equation (2) assuming total antenna gain of 0 dBi.

Considering a carrier frequency of 30 GHz and a base station-UE separation of (500*2/3*62% =) 207 m and a base station antenna height of 15 m above average rooftop level, the path loss using the UMa LOS PL1 formula in Table 5.2.3-1 will result in a path loss of 32.4+20log10(sqrt(2072+152))+20log10(30) = 108 dB. Similarly, the modified power control parameters for other cases with carrier frequency of 30 GHz and 70 GHz can be calculated as shown in Table 5.2.3-2 below.

Table 5.2.3-2: Modified power control algorithm parameter

(a) For 30 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1a | 1 | 108 | 109 | 110 | 111 |
| Set 2a | 0,8 | TBD | TBD | 119 | 121 |

(b) For 70 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1a | 1 | 116 | 116 | 117 | 119 |
| Set 2a | 0,8 | TBD | TBD | 126 | 128 |

#### 5.2.4	Other simulation assumptions and parameters [8]

Other simulation assumptions and parameter are summarized in Table 5.2.4-1 below:

Table 5.2.4-1: Simulation assumptions and parameters

(a) Main simulation assumptions

|  | Way forward | Remarks |
| --- | --- | --- |
| Methodology | Static MonteCarlo simulation |  |
| Usage scenario (eMBB, mMTC, URLLC) | eMBB | Start with eMBB as 1st step. We shall consider the other scenarios in SI. |
| Deployment scenario | At least, Indoor hotspot, Urban macro and Dense urban are the scenarios for WP5D co-existence. | The other scenarios are FFS. |
| Victim system | NR |  |
| PathLoss Model | Follow RAN1 channel modelling SI outcome |  |
| Carrier frequency | 30 and 70 GHz | Using 28GHz is not precluded in addition to 30 and 70GHz. |
| Channel Bandwidth | 200 MHz | Even wider channel bandwidth for example 1GHz can be considered later after understanding the baseline in SI |
| Channel arrangement | Symmetrical (200MHz vs 200MHz) | Start with symmetrical as 1st step |
| UL power control | The same as TR 36.942, adopting PC set parameters to the scenario |  |
| DL power control | No power control |  |
| The number of antenna element | For 30GHz: UE: 16, eNB: 256 For 70GHz: UE: 32, eNB: 256 | Companies are encouraged to provide technical justification on these values. |
| UE antenna | Use Table 5.2.2-1 (a), element gain and antenna aperture TBD. |  |
| BS antenna | Use Table 5.2.2-1 (a). |  |
| Beamforming | Use Table 5.2.2-1 (b). |  |
| SINR to throughput mapping | Adopt scaled Shannon's formula with update truncation and attenuation parameters until we get RAN1, we will use 36.942 Scaled Shannon's formula. |  |
| Co-existence scenario in terms of interference | UL to UL and DL to DL | Not consider UE to DL and DL to UL for the moment. Specifically for UE to UE, this can be discussed after understanding specific mitigation techniques. |
| Traffic | Full buffer |  |
| ACI leakage model | In addition, model in ACLR is two step models in frequency use the different correlation levels to study the spatial patterns for ACLR. ACLR metric defines integrated over the whole space. If study should ACLR can be modelled as flat in space, then, we just use integrated ACLR model. Assume flat ACS in frequency. In space, study if ACS model is flat. |  |
| Performance metric | Throughput degradation compared to single operator case, i.e. no ACI. Both mean and 5%-tile are considered. |  |
| NF | For BS: 8 dB For UE: 13 dB for baseline performance, 10 dB for high performance | Companies are encourage to provide more analysis data on this aspect in the future meeting. |

(b) Other simulation parameters

|  | BS |  | UE |
| --- | --- | --- | --- |
| Lognormal fading | 10 dB |  |  |
| Shadowing correlation | Between sites: 0.5, between sectors of the same site: 1.0 |  |  |
| Minimum distance UE<->BS (2D) | RMa, UMa: 35 m UMi: 10 m InH: 0 m |  |  |
| Number of active UEs | DL: 1 UE, UL: 4 UEs |  |  |
| Handover margin | 3 dB |  |  |
| Cable loss | 1 dB | 0 dB |  |
| Transmit power | RMa, UMa: 46 dBm UMi: 38 dBm InH: 20 dBm |  | 23 dBm |
| Antenna height | RMa: 45 m UMa: 25 m UMi: 10 m InH: 3 m |  | 1.5 m |

<End of change>

# References

[1]	RP-161214, “Revision of SI: Study on New Radio Access Technology”, NTT DOCOMO, INC.

[2]	RP-160508, “LS on Characteristics of terrestrial IMT systems for frequency sharing/interf. analysis in 24.25 - 86 GHz”, ITU-R WP5D.

[3]	R4-162879, “WF on RF parameters requested by WP 5D”, NTT DOCOMO, INC., Qualcomm, CMCC, Nokia, Huawei, Ericsson.

[4]	R4-164528, “WF on WP 5D co-existence simulation assumptions”, NTT DOCOMO, INC.

[5]	R4-165141, “Modelling and simulation of IMT networks for use in sharing and compatibility studies”, ITU-R WP5D.

[6]	R4-164535, “Draft TR skeleton for New Radio SI (TR 38.803)”, NTT DOCOMO, INC.

[7]	3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

[8]	3GPP TR 37.809 v11.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA) medium range and Multi-Standard Radio (MSR) medium range / local area Base Station (BS) class requirements”.

[9]	RP-161301, “TR38.900, Version 2.0.0”, Samsung.

[10]	R4-157516, “TR 37.842 v1.8.0”, Huawei.

[11]	R4-165140, Updated characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz, ITU-R WP5D