3GPP TSG-RAN WG4 #80	R4-165554

Gothenburg, Sweden, 22 – 26 Aug, 2016

Source: 	Ericsson

Title:	Simulation results for LAA PDSCH and PDCCH

Agenda Item:	6.6.4.2

Document for:	Discussion

# Introduction

In Hongkong Ad Hoc meeting, simulation assumption for PDSCH was agreed in [1] and simulation assumption for PDCCH proposed in [2] also got some consensus. In this contribution, we provide simulation results based on the given simulation assumption.

# Simulation results for TM4

The simulation assumption is shown in Table 1. The simulation results are shown in Figure 1 and Figure 2.

Figure 1: TM4 results for 4x2 16QAM coderate=0.5

Figure 2: TM4 results for 64 QAM coderate=0.6

Table 1: Simulation assumption for TM4

The target SNR @70% throughput is tabulated in Table 2.

Table 2: Target SNR for TM4 4x2

|  | Ideal simulation results (dB) | Impairment results (dB) |
| --- | --- | --- |
| 16QAM coderate =0.5 | 8.2 | 9.7 |
| 64QAM coderate =0.6 | 15.7 | 17.2 |

# Simulation results for TM9

Simulation assumption is shown in Table 3. The simulation results are shown in Figure 3 and Figure 4.

Table 3: Simulation assumption for TM9

Figure 3: TM9 results for 16 QAM coderate=0.5

Figure 4: TM9 results for 64 QAM coderate=0.6

The target SNR @70% throughput is tabulated in Table 4.

Table 4: Target SNR for TM9 2x2

|  | Ideal simulation results (dB) | Impairment results (dB) |
| --- | --- | --- |
| 16QAM coderate =0.5 | 11.5 | 13.0 |
| 64QAM coderate =0.6 | 20.3 | 21.8 |

# Discussion on the MCS selection for PDSCH

Since the discontinuous CRS impact on the PDSCH performance is more visible for higher modulation, and further low SNR point is checked via PDCCH test to some extended, Hence, we slightly prefer 64QAM coderate=0.6 as the final MCS.

Proposal 1: 64QAM coderate=0.6 is adopted as the MCS for PDSCH Test case

# Simulation results for PDCCH

Simulation assumption is shown in Table 5. The results for LAA Scell with FDD Pcell and LAA Scell with TDD Pcell are shown in Figure 5 and Figure 6. The target SNR @1% BLER is shown in

Table 5: Simulation assumption for PDCCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Propagation condition |  |  | EVA70 |
| Correlation matrix |  |  | 2x2 Low |
| Number of CCE |  |  | 4 |
| Format 2A |  | bits | 48 (not included CRC) (FDD Pcell) 51 (not included CRC) (TDD Pcell) |
| PDSCH transmission mode |  |  | 2 |
| NOTE 1: . NOTE 2: PUCCH format 1b with channel selection is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-4, PUCCH format 3 is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-6. NOTE 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Figure 5: PDCCH performance for LAA Scell with FDD Pcell

Figure 6: PDCCH performance for LAA Scell with TDD Pcell

Table 6: Target SNR for PDCCH

|  | Ideal simulation results (dB) | Impairment results (dB) |
| --- | --- | --- |
| Target SNR @1% BLER for LAA Scell with FDD Pcell | -1.8 | -0.3 |
| Target SNR @1% BLER for LAA Scell with TDD Pcell | -1.5 | 0 |

# Conclusion

In this paper, we provide our simulation results for PDSCH and PDCCH for LAA scell. Based on the results, we have the following proposal:

Proposal 1: 64QAM coderate=0.6 is adopted as the MCS for PDSCH Test case

We hope the group can consider the simulation results for the performance definition of LAA Scell.

# Reference

- R4-79AH-0232, Simulation assumptions for PDSCH, Ericsson, June, 2016

- R4-165555, Simulation assumption for LAA PDCCH, Ericsson, Aug, 2016.