3GPP TSG-RAN WG4 RAN4 #80                                        R4-165931

Gothenburg, Sweden, 22 – 26 Aug, 2016

Agenda item:			9.3.1

Source:			Samsung

Title:			Discussion on General Coexistence Scenarios and Simulation Methodology

Document for:			Discussion

# 1	Introduction

The study item on NB-IoT RF requirement for coexistence with CDMA was approved at RAN #72. According to WID [1], the study item has set up objectives as follows:

- Identify operating bands for interference evaluation on NB-IoT coexistence with CDMA, such as bands around 800M (B5, B26 etc.) - For the NB-IoT stand-alone operation mode, evaluate coexistence requirement between NB-IoT and CDMA, and further evaluate whether R13 NB-IoT RF requirements could be reused or not. - Identify necessary additional RF requirements to ensure NB-IoT for co-existence with CDMA.

In this contribution, we provide our understanding for related issues for this coexistence analysis, including the operating bands for NB-IoT and CDMA, coexistence simulation cases to be researched in this study item. Furthermore, the general simulation methodology is proposed, while the detailed discussion on simulation assumptions and parameters are provided in our accompanying contributions [5][6].

# Operating Bands for Coexistence Study

According to TS36.101, NB-IoT is designed to operate in the E-UTRA operating bands 1, 2, 3, 5, 8, 12, 13, 17, 18, 19, 20, 26, 28, 66 which are defined in Table 5.5-1 [2]. Based on the description in WID [1], the frequency bands around 800MHz is highlighted to be the research objective: Currently, 800MHz is widely used for deployment of CDMA technologies around the world, and operators’ plan to re-farm this band for NB-IoT deployment which potentially means that a relative long-term coexistence between NB-IoT and legacy CDMA technologies.

Firstly, the E-UTRA bands for NB-IoT around 800MHz are

Table 2.1 3GPP NB-IOT frequency bands for 800MHz

| NB-IOT Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |
| 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |
| 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz |
| 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |
| 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz |

On the other hand, based on 3GPP2 specification on CDMA2000 air interface, the following bands will be the ones to be used around 800MHz:

Table 2.3 3GPP2 CDMA2000 frequency bands for 800MHz

| CDMA2000 Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |
| Band Class 0 (800MHz) | 824.025 844.995 | – – | 835.005 846.495 | 869.025 889.995 | – – | 880.005 891.495 |
| Band Class 10 (Secondary 800MHz Band) | 806.000 811.000 816.000 821.000 | – – – – | 810.975 815.975 820.975 823.975 | 851.000 856.000 861.000 866.000 | – – – – | 855.975 860.975 865.975 868.975 |

From our information updated till now, China Telecom has the plan to re-farm 800MHz band for NB-IoT usage coexisting with legacy CDMA deployment. Therefore, the current spectrum allocation around 800MHz in China is provided here for information:

Figure 2.1 Spectrum Allocation in China for 800MHz

Based on above spectrum allocation status, the CDMA 2000 band class 0 will be re-farmed for NB-IoT usage, which is corresponding to E-UTRA band 5 and 26. Therefore, 850MHz can be utilized as carrier frequency in the coexistence study.

- Proposal 1: 850MHz can be utilized as carrier frequency in the coexistence study.

# Coexistence Simulation Cases

Another question to be answer is which CDMA technology should be considered in this coexistence study, which is of importance in determining coexistence simulation assumption and parameters.

For the above-mentioned 800MHz CDMA deployment in China, the CDMA2000 deployment will be re-farmed by LTE and NB-IoT usage, which will be neighboring to legacy CDMA 2000 1x deployment in this upper-band. Especially considering the deployment CDMA2000 1x EV-DO deployment will be replaced by LTE system for providing data service, we propose to consider CDMA2000 1x system in the coexistence study:

- Proposal 2: Rather than CDMA2000 1x EV-DO system, CDMA2000 1x system should be utilized in this coexistence study.

As requested in WID [1], only NB-IoT stand-alone mode needs to be considered in the coexistence study. Similar to the analysis in Rel-13 NB-IoT study [4], we can reach the following proposal:

- Proposal 3: The following cases for coexistence between NB-IoT and legacy CDMA2000 1x system should be considered:

Table 3.1 Simulation cases of coexistence study for NB-IoT with CDMA

| Cases | Operation mode | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- |
| 1 | Stand-alone | NB-IoT | CDMA2000 1x | Downlink |
| 2 | Stand-alone | CDMA2000 1x | NB-IoT | Downlink |
| 3 | Stand-alone | NB-IoT | CDMA2000 1x | Uplink |
| 4 | Stand-alone | CDMA2000 1x | NB-IoT | Uplink |

# Coexistence Analysis Methodology

According to WID [1], RAN4 should evaluate coexistence requirement between NB-IoT and CDMA, and further evaluate whether Rel-13 NB-IoT RF requirements could be reused or not. The following methodology for coexistence analysis is proposed:

Step-1: RAN4 get agreement on RF performance assumptions for legacy CDMA system.

- The RF performance assumptions for legacy CDMA system should be determined firstly, e.g., ACLR/ACS performance for both CDMA UE and BS. In another contribution from us, we provide our understanding and proposals on CDMA2000 1x RF performance assumptions [6].

Step-2: Based upon operators’ input on the NB-IoT deployment scenarios, the carrier separation between NB-IoT and CDMA systems are determined.

- The carrier separation should be determined based on practical operators’ deployment needs.

- For evaluation purpose, several candidate options for carrier separation could be proposed.

Step-3: Conduct coexistence simulation based on agreed simulation assumptions, under certain agreed RF performance assumptions for legacy CDMA system and certain carrier separation options between NB-IoT and CDMA systems.

- Similar to Rel-13 coexistence analysis, the same range of NB-IoT ACLR/ACS values (listed in the table provided in [5]) will be applied.

- After obtaining coexistence simulation results, RAN4 should compare the simulation results with Rel-13 NB-IoT RF requirement to determine it could be reused or not.

- If the aligned simulation results indicate that Rel-13 NB-IoT RF requirements could be reused to guarantee the coexistence between NB-IoT and legacy system, go to step 4.

- If the aligned simulation results indicate that Rel-13 NB-IoT RF requirements is not enough to guarantee the coexistence between NB-IoT and legacy system, go to step 5.

Step-4: Conclusion can be reached that Rel-13 NB-IoT RF requirements could be reused to make sure a proper coexistence with CDMA system.

Step-5: Identify the requirement to restrict NB-IoT RF requirement to make sure a proper coexistence with CDMA system.

- Proposal 4: The following coexistence analysis methodology is proposed for this study:

- Step-1: RAN4 get agreement on RF performance assumptions for legacy CDMA system.

- Step-2: Based upon operators’ input on the NB-IoT deployment scenarios, the carrier separation between NB-IoT and CDMA systems are determined.

- Step-3: Conduct coexistence simulation based on agreed simulation assumptions, under certain agreed RF performance assumptions for legacy CDMA system and certain carrier separation options between NB-IoT and CDMA systems.

  - If the aligned simulation results indicate that Rel-13 NB-IoT RF requirements could be reused to guarantee the coexistence between NB-IoT and legacy system, go to step 4.

  - If the aligned simulation results indicate that Rel-13 NB-IoT RF requirements is not enough to guarantee the coexistence between NB-IoT and legacy system, go to step 5.

- Step-4: Conclusion can be reached that Rel-13 NB-IoT RF requirements could be reused to make sure a proper coexistence with CDMA system.

- Step-5: Identify the requirement to restrict NB-IoT RF requirement to make sure a proper coexistence with CDMA system.

# Conclusion

In this contribution, we provided our understanding for related issues for this coexistence analysis, including the operating bands for NB-IoT and CDMA, coexistence simulation cases to be researched in this study item. Furthermore, the general simulation methodology was proposed. Specifically, the following proposals are given:

- Proposal 1: 850MHz can be utilized as carrier frequency in the coexistence study.

- Proposal 2: Rather than CDMA2000 1x EV-DO system, CDMA2000 1x system should be utilized in this coexistence study.

- Proposal 3: The following cases for coexistence between NB-IoT and legacy CDMA2000 1x system should be considered:

Table 3.1 Simulation cases of coexistence study for NB-IoT with CDMA

| Cases | Operation mode | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- |
| 1 | Stand-alone | NB-IoT | CDMA2000 1x | Downlink |
| 2 | Stand-alone | CDMA2000 1x | NB-IoT | Downlink |
| 3 | Stand-alone | NB-IoT | CDMA2000 1x | Uplink |
| 4 | Stand-alone | CDMA2000 1x | NB-IoT | Uplink |

- Proposal 4: The following coexistence analysis methodology is proposed for this study:

- Step-1: RAN4 get agreement on RF performance assumptions for legacy CDMA system.

- Step-2: Based upon operators’ input on the NB-IoT deployment scenarios, the carrier separation between NB-IoT and CDMA systems are determined.

- Step-3: Conduct coexistence simulation based on agreed simulation assumptions, under certain agreed RF performance assumptions for legacy CDMA system and certain carrier separation options between NB-IoT and CDMA systems.

  - If the aligned simulation results indicate that Rel-13 NB-IoT RF requirements could be reused to guarantee the coexistence between NB-IoT and legacy system, go to step 4.

  - If the aligned simulation results indicate that Rel-13 NB-IoT RF requirements is not enough to guarantee the coexistence between NB-IoT and legacy system, go to step 5.

- Step-4: Conclusion can be reached that Rel-13 NB-IoT RF requirements could be reused to make sure a proper coexistence with CDMA system.

- Step-5: Identify the requirement to restrict NB-IoT RF requirement to make sure a proper coexistence with CDMA system.

# Reference

[1] 3GPP RP-161187, “WID: Study on NB-IoT RF requirement for coexistence with CDMA”, China Telecom.

[2] 3GPP TS 36.101 V13.4.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”.

[3] 3GPP2 C.S0024 V4.0, “cdma2000 High Rate Packet Data Air Interface Specification”.

[4] 3GPP TR 36.802 V1.0.0, “Narrowband Internet of Things (NB-IoT); Technical Report for BS and UE radio transmission and reception”.

[5] 3GPP R4-165933, “Discussion on Detailed Coexistence Simulation Assumptions and Parameters”, Samsung, China Telecom.

[6] 3GPP R4-165932, “Discussion on CDMA2000 1x RF Performance”, Samsung.