3GPP TSG-RAN WG4 RAN4 #80                                        R4-165934

Gothenburg, Sweden, 22 – 26 Aug, 2016

Agenda item:			9.3.3

Source:			Samsung

Title:	Simulation Results on NB-IoT Coexistence Study with CDMA system for Uplink Scenarios

Document for:			Discussion

# 1	Introduction

The study item on NB-IoT RF requirement for coexistence with CDMA was approved at RAN #72. According to WID [1], the study item has set up objectives as follows:

- Identify operating bands for interference evaluation on NB-IoT coexistence with CDMA, such as bands around 800M (B5, B26 etc.) - For the NB-IoT stand-alone operation mode, evaluate coexistence requirement between NB-IoT and CDMA, and further evaluate whether R13 NB-IoT RF requirements could be reused or not. - Identify necessary additional RF requirements to ensure NB-IoT for co-existence with CDMA.

In this contribution, we provide the initial simulation results for the uplink coexistence simulation scenarios between NB-IoT and CDMA system. The related coexistence simulation scenarios and methodology are described in our accompanying contribution [2], and the detailed simulation assumptions and parameters are provided in another accompanying contribution [3].

# Coexistence Simulation Results

As described in [2], there are total 4 coexistence simulation cases between NB-IoT and CDMA2000 1x system. In this paper, we only provide the simulation results for the two uplink cases, i.e. Case 3 and Case 4 in below table.

Table 2.1 Simulation cases of coexistence study for NB-IoT with CDMA

| Cases | Operation mode | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- |
| 1 | Stand-alone | NB-IoT | CDMA2000 1x | Downlink |
| 2 | Stand-alone | CDMA2000 1x | NB-IoT | Downlink |
| 3 | Stand-alone | NB-IoT | CDMA2000 1x | Uplink |
| 4 | Stand-alone | CDMA2000 1x | NB-IoT | Uplink |

The detailed simulation assumptions and parameters can be referred in our accompanying contribution [3].

The simulation results are provided in Table 2.2 and Table 2.3 for Case 3 and Case 4 respectively. In below simulation results, 3 types of NB-IoT UE configurations are provided, i.e. 3/12/48 UEs per cell with 60/15/3.75 KHz bandwidth per UE.

Table 2.2 Simulation results for Case 3: Standalone uplink, NB-IoT aggressor, CDMA victim

| # of NB-IoT UE | CDMA capacity loss (%) vs. NB-IoT UE ACLR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| 3 UEs – 60 KHz | 95.53 | 89.11 | 61.86 | 31.25 | 13.06 | 4.98 | 1.86 |
| 12 UEs – 15 KHz | 95.61 | 95.38 | 79.09 | 39.52 | 14.84 | 5.25 | 1.98 |
| 48 UEs – 3.75 KHz | 95.61 | 95.61 | 90.77 | 45.63 | 15.94 | 5.40 | 2.10 |

Table 2.3 Simulation results for Case 4: Standalone uplink, CDMA aggressor, NB-IoT victim

| # of NB-IoT UE | # of NB-IoT UE | NB-IoT SINR loss (dB) vs. NB-IoT BS ACS |  |  |
| --- | --- | --- | --- | --- |
|  |  | 40 | 45 | 50 |
| 3 UEs – 60 KHz | SINR loss 5%-tile | 0.01 | 0.01 | 0.01 |
|  | SINR loss 50%-tile | 0.03 | 0.02 | 0.02 |
|  | SINR loss 95%-tile | 0.11 | 0.10 | 0.09 |
|  | SINR loss 99%-tile | 0.12 | 0.10 | 0.09 |
| 12 UEs – 15 KHz | SINR loss 5%-tile | 0.01 | 0.01 | 0.01 |
|  | SINR loss 50%-tile | 0.03 | 0.03 | 0.03 |
|  | SINR loss 95%-tile | 0.10 | 0.09 | 0.08 |
|  | SINR loss 99%-tile | 0.14 | 0.12 | 0.11 |
| 48 UEs – 3.75 KHz | SINR loss 5%-tile | 0.01 | 0.01 | 0.01 |
|  | SINR loss 50%-tile | 0.03 | 0.03 | 0.03 |
|  | SINR loss 95%-tile | 0.10 | 0.09 | 0.08 |
|  | SINR loss 99%-tile | 0.10 | 0.09 | 0.09 |

Based on the above results, it can be observed that:

- For Case 3, i.e. NB-IoT interfering CDMA in uplink, when NB-IoT UE ACLR is 45 dB (defined on 180 KHz bandwidth), the CDMA system average capacity loss is around 5%. So, to satisfy the CDMA performance loss criteria that capacity loss is less or equal to 5%, the requirement for NB-IoT UE ACLR should be larger than 45 dB for its own bandwidth.

- For Case 4, i.e. CDMA interfering NB-IoT in uplink, since the legacy CDMA UE ACLR is fixed to 33.8 dB (defined on 180 KHz bandwidth), and the input of NB-IoT BS ACS is 40/45/50 dB (defined on 180 KHz bandwidth), we can see that the major ACIR interference is from CDMA UE ACLR. The results show that even when the NB-IoT BS ACS is 40 dB (defined on 180 KHz bandwidth), it can well satisfy the NB-IoT performance loss criteria that average of SNR value loss is less or equal to 1 dB.

The above observations can be summarized as below:

Observation 1: To satisfy CDMA performance loss criteria, the requirement for NB-IoT UE ACLR should be larger than 45 dB for its own bandwidth.

Observation 2: The NB-IoT performance loss criteria can be well satisfied even when NB-IoT BS ACS is 40 dB for its own bandwidth.

# Conclusion

In this contribution, we provide our results for the uplink scenarios based on the simulation assumptions and parameters provided in our accompanying contribution [3]. Based on the simulation results, the following observations are given:

Observation 1: To satisfy CDMA performance loss criteria, the requirement for NB-IoT UE ACLR should be larger than 45 dB for its own bandwidth.

Observation 2: The NB-IoT performance loss criteria can be well satisfied even when NB-IoT BS ACS is 40 dB for its own bandwidth.

# Reference

[1] 3GPP RP-161187, “WID: Study on NB-IoT RF requirement for coexistence with CDMA”, China Telecom.

[2] 3GPP R4-165931, “Discussion on General Coexistence Scenarios and Simulation Methodology”, Samsung.

[3] 3GPP R4-165933, “Discussion on Detailed Coexistence Simulation Assumptions and Parameters”, Samsung, China Telecom.

# Appendix

The detailed simulation parameters are provided as below:

Table 2.1 Simulation assumption for coexistence study between NB-IoT and CDMA2000 1x system

|  | NB-IoT standalone | CDMA2000 1x Refer to TR25.942 |
| --- | --- | --- |
| Carrier frequency in GHz | 0.85 | 0.85 |
| Size of each nominal channel BW in MHz | 0.2 | 1.23 |
| Transmission bandwidth in MHz | 0.18 | 1.23 |
| Environment | Urban macro | Urban macro |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around |
| Inter-site distance in meter | 750m (Same for aggressor and victim) | 750m (Same for aggressor and victim) |
| System loading and activity | Full buffer 100% | All users with 9.6kbps speech |
| Network location | Non co-located (at cell edge of legacy) | Non co-located with IoT |
| UL | See RP-152284(both multi-tone and single-tone) | n/a |
| UL power control | 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB | TR25.942 (reuse UMTS PC) |
| Frequency reuse | 1 | 1 |
| Number of scheduled UE per cell (UL) | 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone | according to 5.5dB noise rise; target Eb/N0=4.0dB no MUD utilized, beta=0 |
| BS antenna height in meter | 30 | 30 |
| BS antenna gain including feeder loss in dBi | 15 | 15 |
| BS antenna pattern | Horizontal (TR36.942) | Horizontal (65deg for -3dB) |
| BS antenna front-back ratio in dB | 20 | 20 |
| UE antenna height in meter | 1.5 | 1.5 |
| UE TX power in dBm | -40 to 23 | -52 to 23 |
| UE antenna gain in dBi | 0 | 0 |
| Building penetration loss | 45.820 Annex D.1 | n/a |
| Cell selection margin in dB | 3 | 3 |
| BS-MS min couple loss in dB | 70 | 70 |
| BS ACLR in dB | 40 to 60* | 36.8 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |
| BS ACS in dB | 40 to 50* | 81 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |
| UE ACLR in dB | 20 to 50* | 33.8 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |
| UE ACS in dB | 20 to 40* | 49.5 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |
| BS noise figure in dB | 5 | 5 |
| BS-UE path-loss model | TR36.942 macro urban | TR36.942 macro urban |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 | 10 |
| Shadowing correlation | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 |
| Evaluation metrics | SINR vs ACS (as victim) | Capacity vs NB-IoT ACLR (as victim). For DL, capacity is the number of satisfied users. For UL, capacity is the number of users when 5.5dB noise rise is reached |

*Similar to coexistence study in TR36.802, NB-IoT ACLR and ACS are defined for its own bandwidth size.