3GPP TSG-RAN WG4 Meeting #80                                                                                                  R4-15937

# Gothenburg, Sweden, 22 - 26 Aug, 2016

Agenda Item:	6.7.4.2.2

Source: 	Samsung

Title: 	Simulation results for NB-IoT PRACH

Document for:	Discussion

Introduction

In this contribution, simulation results for NB-IoT PRACH were given.

Discussion

| Simulation assumptions |  |  |
| --- | --- | --- |
|  | Parameter | Values |
|  | Preamble format | 0 |
|  | Number of Repetitions | 2, 8, 32 |
|  | Number of subcarriers | 12 |
|  | Cell ID | 0 |
|  | Antenna configuration | 1x2 |
|  | Frequency offset | 0Hz (AWGN); [200]Hz (EPA1) |
|  | Propagation channels | AWGN, [EPA1] |
|  | Detection performance | Missed detection rate (1%) with false alarm rate (0.1%) |
|  | Timing estimation | Timing error probability with limits 2.5us, [5us] |

Simulation Results for AWGN

Table1. Simulation results for detection performance of missed detection=1% with false alarm rate 0.1% for AWGN[frequency offset=0Hz]

| Repetition level | SNR@1% Miss. |
| --- | --- |
| 2 | -1.8 |
| 8 | -5.9 |
| 32 | -9.6 |

Table2. Simulation results for timing error probability [=1%] with limits [=2.5us, 5us] for AWGN [frequency offset=0Hz]

| Repetition level | SNR@2.5us (1%) | SNR@5us (1%) |
| --- | --- | --- |
| 2 | 7.3 | 1.7 |
| 8 | -1.5 | -5.2 |
| 32 | -6.6 | -9.5 |

Simulation results for EPA1Hz

Table3. Simulation results for detection performance of missed detection=1% with false alarm rate 0.1% for EPA1 [frequency offset=200Hz]

| Repetition level | SNR@1% Miss |
| --- | --- |
| 2 | 6.7 |
| 8 | 2.6 |
| 32 | -2.3 |

Table4. Simulation results for timing error probability [=1%] with limits [=2.5us, 5us] for EPA1 [frequency offset=200Hz]

| Repetition level | SNR@2.5us(1%) | SNR@5us (1%) |
| --- | --- | --- |
| 2 | 11.6 | 6.6 |
| 8 | 4.2 | 1.1 |
| 32 | -1.6 | -3.9 |

Conclusion

In this contribution, simulation results for NB-IoT PRACH were given.

Reference