3GPP TSG-RAN Working Group 4 Meeting #80	R4- 166141

Gothenburg, Sweden, 22nd August – 26th August 2016

Source:	DISH Network, Nokia

Title:	TP for TR 36.714-03-01:  Operating bands, Channel bandwidths, Co-existence study and delta Tib/Rib values for CA_29A-66C and CA_29A-66A-66A

Agenda item:	8.3.2

Document for:	Approval

## 1	Introduction

This contribution is a text proposal for CA_29A-66C and CA_29A-66A-66A which were approved in RAN #72 [1].  Co-existence studies for both configurations are also included.

## 2	Text Proposal for TR 36.714-03-01

----- Start of Text proposal -----

## 6.X	CA_29A-66C_BCS0 and CA_29A-66A-66A_BCS0

### 6.X.1	Operating bands for CA

Table 6.10.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_29-66 | 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  |
| CA_29-66-66 | 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: E-UTRA CA configurations and bandwidth combination sets defined for CA_29A-66C and CA_29A-66A-66A

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_29A-66C | 29 |  |  | Yes | Yes |  |  | 50 | 0 |
|  | 66 | See the CA_66C Bandwidth combination set 0 in the Table 6.1.1-2 |  |  |  |  |  |  |  |
| CA_29A-66A-66A | 29 |  |  | Yes | Yes |  |  | 50 | 0 |
|  | 66 | See the CA_66A-66A Bandwidth combination set 0 in the Table 6.1.1-3 |  |  |  |  |  |  |  |

Table 6.X.2-2: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66C | 5 | 20 |  |  | 40 | 0 |
|  | 10 | 15, 20 |  |  |  |  |
|  | 15 | 10, 15, 20 |  |  |  |  |
|  | 20 | 5, 15, 10, 20 |  |  |  |  |

Table 6.X.2-3: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

| E-UTRACA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66A-66A | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 |  |

### 6.X.3	Co-existence studies

#### 6.X.3.1	CA_29A-66C

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 29 and Band 66 DL carriers can be calculated as shown in Table 6.10.3.1-1 below:

Table 6.X.3.1-1: Band 29 and Band 66 DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 717 | 728 | 2110 | 2200 |
| 2nd harmonics frequency limits (MHz) | 1434 | 1456 | 4220 | 4400 |
| 3rd harmonics frequency limits (MHz) | 2151 | 2184 | 6330 | 6600 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 1382 | 1483 | 2827 | 2928 |
| 3rd order IMD products | (f2-low – 2*f1-high) | (f2-high – 2*f1-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 654 | 766 | 3492 | 3683 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 3544 | 3656 | 4937 | 5128 |
| 3rd order IMD products | (f1-low – f2-high + f2-low) | (f1-high + f2-high – f2-low) | (f2-low – f1-high + f1-low) | (f2-high + f1-high – f1-low) |
| IMD frequency limits (MHz) | 627 | 818 | 2099 | 2211 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 677 | 768 | 2100 | 2210 |

It can be seen from Table 6.7.3-1 the 2nd harmonics of BS transmitting in Band 29 may fall into the BS receive band of Bands 11, 21 and 45, while the 2nd IMD products supporting CA of Band 29 and Band 66 may fall into the BS receive band of Bands 11, 21 and 45, and the 3rd IMD products may fall into the BS receive band of Bands 12, 13, 14, 17, 18, 26, 27, 28, 42, 43, 44 and 68. Note that the calculation in Table 6.x.3.1-1 (except the last row) assumes the BS is transmitting with the whole 11 MHz DL frequency of Band 29 and the whole 90 MHz DL frequency of Band 66. If the BS is only transmitting up to 10 MHz DL in Band 29 and up to 40 MHz DL in Band 66 as stated in the WIDS, then the 3rd IMD products will not fall into the BS receive band of Band 13, 14, 18, 26 or 27 as shown in the last row in Table 6.7.3-1.

Note that Bands 11, 21, 28, 44, 45 and 68 are not intended for use in the same geographical area as Bands 29 and 66. Therefore, the focus here is on the harmonics and IMD falling into Bands 12, 17, 42 and 43.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 12, 17, 42 or 43 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 29 and 66 BS transmitters do not share the same antenna with Band 12, 17, 42 or 43 BS receiver.

Therefore, in order to prevent BS receiver desensitization, it is recommended that Bands 29 and 66 BS transmitters should not share the same antenna with Band 12, 17, 42 or 43 BS receiver unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 12, 17, 42 or 43 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

#### 6.X.3.2 CA_29A-66A-66A

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of CA_29A-66A-66A DL carriers can be calculated as shown in Table 6.X.3.2-1 below:

Table 6.X.3.2-1: CA_29A-66A-66A DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 717 | 728 | 2110 | 2200 | 2110 | 2200 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 627 |  |  | 818 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 627 |  |  | 818 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 3492 |  |  | 3683 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 4937 |  |  | 5128 |  |  |

It can be seen from Table 6.X.3.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA_29A-66A-66A may fall into the BS receive band of Bands 12, 13, 14, 17, 18, 26, 27, 28, 42, 43, 44 and 68.

Note that Bands 18, 28, 44 and 68 are not intended for use in the same geographical area as Bands 29 and 66. Therefore, the focus here is on the IMD falling into Bands 12, 13, 14, 17, 26, 27, 42 and 43.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 12, 13, 14, 17, 26, 27, 42 or 43 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 29 and 66 BS transmitters do not share the same antenna with Band 12, 13, 14, 17, 26, 27, 42 or 43 BS receiver..

Therefore, in order to prevent BS receiver desensitization, it is recommended that Bands 29 and 66 BS transmitters should not share the same antenna with Band 12, 13, 14, 17, 26, 27, 42 or 43 BS receiver, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 12, 13, 14, 17, 26, 27, 42 or 43 receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

### 6.X.4	∆TIB and ∆RIB values

CA_29A-66C and CA_29A-66A-66A are formed by diplexing a carrier in Band 29 with the intra-band carriers in Band 66.  No harmonic terms overlap with downlink in Band 29 from uplink in Band 66.  The diplexer relaxations TIB = 0.3 dB, RIB = 0 dB apply with no MSD or reference sensitivity exceptions.

----- End of Text proposal -----

## References

RP-160945, “Revised WID: LTE Advanced inter-band CA Rel-14 for 3DL/1UL”, Huawei, Hisilicon, 3GPP RAN#72, Busan, Korea, June 2016