3GPP TSG RAN4 Meeting #80	R4-166142

Gothenburg, Sweden, 22 – 26 August, 2016

Title: 	MSD for 3+7+8 3DL 2UL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.7.2

Document for:	Discussion

# 1	Introduction

This contribution discusses MSD requirements for 3+7+8 3DL 2 UL CA.

# 2	Discussion

This CA combination has several intermodulation products falling on top of own RX, see table below. Please note that this combination is the very first example when two intermodulations, and in addition, third harmonic fall on top of each other.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B8 | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |
|  | B3 | IMD2 | fB3 + fB8 | 1735 | 5 | 25 | B7 | 2635.0 | 10 |
|  | B8 |  |  | 900 | 5 | 25 |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB8 | 1772.5 | 5 | 25 | B7 | 2635.0 | 10 |
|  | B8 |  |  | 910 | 5 | 25 |  |  |  |
|  | B3 | IMD2+IMD3+H3 | fB3 + fB8, 2*fB3 - fB8 | 1780 | 5 | 25 | B7 | 2670 | 10 |
|  | B8 |  |  | 890 | 5 | 25 |  |  |  |

Table 1 Example test points

We use the following RF front-end architecture in our analysis. Another alternative would be common triplexer based architecture. There is harmonic trap assumed in B8 path for this CA case because harmonic trap for was assumed for B8+B7 1UL/2DL CA and for B3+B7+B8 1UL/3DL CA.

Figure 1 Common diplexer -architecture

We analyze the amount of IMD using the following component linearity assumptions:

| Component | IP2 (dBm) | IP3 (dBm) |
| --- | --- | --- |
| Antenna switch | 112 | 68 |
| Diplexer | 115 | 86 |
| Quadplexer | 113 | 74 |
| PA forward mixing | 27 | 30 |
| PA reverse mixing | 38 | 28 |
| LNA | 5 | -5 |

Table 2 General linearity parameters

B8 duplexer TX rejection at B7		30

Quadplexer B8 TX rejection    		45

Quadplexer B3 TX rejection			45

Antenna isolation 					10

PCB isolation PA-PA 				60

PCB isolation PAout-RX				65

RFIC Pout B3/B8						0

Quadplexer X-band Iso B3->B7		50

Diplexer isolation						15

PA gain								28

Based on these parameters we get the following MSD. Please note that the CF is the same as in [1]. For the case where B3+B8 IMD2, IMD3, and B8 third harmonic fall on top of each other at B7 DL the impact of the third harmonic is negligible because it is more than 20dB below the MSD due to IMD2 and IMD3.

B3 forward mixing and B8 LNA are dominant in IMD3 for B8.

B3 and B8 forward mixing are dominant in IMD2 for B7.

B3 forward mixing and B7 LNA are dominant in IMD3 for B7.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B8 | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 | 0.62 | 19.9 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |  |  |
|  | B3 | IMD2 | fB3 + fB8 | 1735 | 5 | 25 | B7 | 2635.0 | 10 | 0 | 30.5 |
|  | B8 |  |  | 900 | 5 | 25 |  |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB8 | 1772.5 | 5 | 25 | B7 | 2635.0 | 10 | 0 | 23.1 |
|  | B8 |  |  | 910 | 5 | 25 |  |  |  |  |  |
|  | B3 | IMD2+IMD3+H3 | fB3 + fB8, 2*fB3 - fB8 | 1780 | 5 | 25 | B7 | 2670 | 10 | 0 | 31.3 |
|  | B8 |  |  | 890 | 5 | 25 |  |  |  |  |  |

# 3	Conclusion

Example test points and MSD for B3+B7+B8 3DL 2UL CA MSD is provided.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B8 | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 | 0.62 | 19.9 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |  |  |
|  | B3 | IMD2 | fB3 + fB8 | 1735 | 5 | 25 | B7 | 2635.0 | 10 | 0 | 30.5 |
|  | B8 |  |  | 900 | 5 | 25 |  |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB8 | 1772.5 | 5 | 25 | B7 | 2635.0 | 10 | 0 | 23.1 |
|  | B8 |  |  | 910 | 5 | 25 |  |  |  |  |  |
|  | B3 | IMD2+IMD3+H3 | fB3 + fB8, 2*fB3 - fB8 | 1780 | 5 | 25 | B7 | 2670 | 10 | 0 | 31.3 |
|  | B8 |  |  | 890 | 5 | 25 |  |  |  |  |  |

Discussion is needed on which cases are specified; for instance the difference between IMD2 on B7 and IMD2+IMD3+H3 on B7 is only 0.8dB (30.5 vs 31.3).

# 4	References

[1] R4-161889, “XDL 2UL IMDs, testpoints and CFs”, Qualcomm Inc., RAN4#78bis