3GPP TSG-WG4 Meeting #80	R4-166489

# Gothenburg, Sweden, 22nd – 26th August 2016

Agenda item: 	6.7.4.1.1

Source: 	Qualcomm Incorporated

Title: 	NPBCH Demodulation Simulation Results

Document for:	Discussion

# 1	Introduction

For NPBCH, RAN4 [1] agree with the following simulation assumptions for alignment, and related main configurations are listed as in Table 1. This paper provides the simulation result for SNR satisfying 1% Pm-bch.

# Table 1: Minimum performance for NPBCH

| Simulation number | Bandwidth | Deployment mode | Reference Channel | Propagation Condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1 | 180 KHz | Stand-alone | R.NB1.1 | EPA1 | 1 x 1 | 1 | TBD |
| 2 | 180kHz | In-band/Guard-band | R.NB1.2 | EPA1 | 2 x 1 | 1 | TBD |

Table 2	RMC table for NPBCH

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.NB1.1 | R.NB1.2 |
| Number of transmitter antennas |  | 1 | 2 |
| Channel bandwidth | MHz | 0.2 | 0.2 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 50/1600 | 50/1600 |
| Payload (without CRC) | Bits | 34 | 34 |

The simulation assumes that NRS is available in every subframe in the frame except subframe 5 which contains NPSS and subframe 9 in every other frame which contains NSSS. This enables channel estimation to be performed using time domain filtering over destaggered NRS tones. A low filter coefficient value is chosen since the channel model considered is a slow fading channel with 1Hz doppler.

# 2	Simulation Results

The following plot shows the NPBCH BLER performance with respect to SNR(dB) for simulation number 1 in Table 1.

Observation 1: For simulation number 1, 1% Pm-bch is achieved at -2dB SNR.

| Simulation number | Bandwidth | Deployment mode | Reference Channel | Propagation Condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1 | 180 KHz | Stand-alone | R.NB1.1 | EPA1 | 1 x 1 | 1 | -2 |

The following plot shows the NPBCH BLER performance with respect to SNR(dB) for simulation number 2 in Table 1.

Observation 2: For simulation number 2, 1% Pm-bch BLER is achieved at -4.5dB SNR.

| Simulation number | Bandwidth | Deployment mode | Reference Channel | Propagation Condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 2 | 180kHz | In-band/Guard-band | R.NB1.2 | EPA1 | 2 x 1 | 1 | -4.5dB |

# 3	Conclusions

This paper contains NPBCH simulation results for alignment and made the following observations:

Observation 1: For simulation number 1, 1% Pm-bch is achieved at -2dB SNR.

Observation 2: For simulation number 2, 1% Pm-bch is achieved at -4.5dB SNR.

# 4	References

- R4-79AH-0161, “Updated simulation assumptions for NB-IoT UE demodulation”, Huawei, HiSilicon, Ericsson, Qualcomm, Samsung, ZTE