3GPP TSG-WG4 Meeting #80	R4-166490

# Gothenburg, Sweden, 22nd – 26th August 2016

Agenda item: 	6.7.3.1

Source: 	Qualcomm Incorporated

Title: 	NRSRP and NRSRQ Measurement Accuracy

Document for:	Discussion

# 1	Introduction

RAN4 is agreed [1] to further analyze NRSRP and NRSRQ measurement accuracy and define accuracy requirements. This section provides an overview of the agreed approach taken to simulate and specify requirements.

Minimum number of sub frames containing NRS per L1 measurement period are to be used by the UE to define NRSRP/NRSRQ measurement accuracy requirements. The following options will be used:

- For L1 period of 400 ms: 10, 30 and 50 sub frames containing NRS

- For L1 period of 800 ms: 20, 60 and 100 sub frames containing NRS

Subset of the samples in the measurement period will be used for coherent averaging. Measurement accuracy requirements will be derived for maximum value obtained at the 5th and 95th percentiles of CDF.

RF margin of around 2-3 dB will be included when setting the requirement. The following were also agreed in RAN4:

- RAN4 will define different intra-frequency absolute accuracy requirements for NRSRP and NRSRQ under normal and extended coverage

- RAN4 will define different inter-frequency absolute accuracy requirements for NRSRP under normal and extended coverage

- RAN4 will not to define accuracy requirement for relative NRSRP measurement and will not define accuracy requirement for inter-frequency NRSRQ measurement

The following is the set of simulation assumptions for measurement accuracy study:

Table 1: Simulation parameters for NB-IOT measurement accuracy studies

The simulation assumed that only SF#0 and SF#4 in each frame contain NRS. One subframe coherent averaging implies that the NRS tones of the two slots in SF#0 are coherently averaged once every 40ms. Three subframe coherent averaging implies that the NRS tones of the 6 slots in SF#0, SF#4 and SF#10 are coherently averaged once every 40ms. Five subframe coherent averaging implies that the NRS tones of the 10 slots in SF#0, SF#4 and SF#10, SF#14 and SF#20 are coherently averaged once every 40ms. Measurement accuracy results shown below are reported at the 95th-%ile point on the CDF.

# 2	AWGN Simulation Results

## 2.1	One subframe coherent averaging

### 2.1.1	NRSRP Measurement Accuracy

The following table shows NRSRP Measurement Accuracy at 95-%tile point for AWGN:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | 4.1dB | 4dB | 4.2dB |
| SNR=-6dB | 10.5dB | 9.1dB | 11.2dB |
| SNR=-15dB | 19dB | 18.4dB | 18.2dB |

### 2.1.2	NRSRQ Measurement Accuracy

The following table shows NRSRQ Measurement Accuracy at 95-%tile point for AWGN:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | -4.9dB | -4.5dB | -4dB |
| SNR=-6dB | 8.8dB | 6.8dB | 8.8dB |
| SNR=-15dB | 16.7dB | 15.8dB | 16.6dB |

## 2.2	Three subframe coherent averaging

### 2.2.1	NRSRP Measurement Accuracy

The following table shows NRSRP Measurement Accuracy at 95-%tile point for AWGN:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | -3.7dB | -3.6dB | -3.5dB |
| SNR=-6dB | 6.3dB | 6.3dB | 7dB |
| SNR=-15dB | 14.8dB | 14.8dB | 13.8dB |

### 2.2.2	NRSRQ Measurement Accuracy

The following table shows NRSRQ Measurement Accuracy at 95-%tile point for AWGN:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | -5dB | -6.4dB | -6.4dB |
| SNR=-6dB | 4.3dB | 4.3dB | 4.3dB |
| SNR=-15dB | 12dB | 12dB | 11dB |

## 2.3	Five subframe coherent averaging

### 2.3.1	NRSRP Measurement Accuracy

The following table shows NRSRP Measurement Accuracy at 95-%tile point for AWGN:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | -15dB | -14.6 | -14.1dB |
| SNR=-6dB | -3.5dB | -4.3dB | -3.6dB |
| SNR=-15dB | 11.8dB | 12.8dB | 11.7dB |

### 2.3.2	NRSRQ Measurement Accuracy

The following table shows NRSRQ Measurement Accuracy at 95-%tile point for AWGN:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | -16.9dB | -17.2 | -16.89dB |
| SNR=-6dB | -6.8dB | -6.8dB | -6.8dB |
| SNR=-15dB | 10.3dB | 10.3dB | 9.4dB |

# 3	ETU5Hz Simulation Results

## 3.1	One subframe coherent averaging

### 3.1.1	NRSRP Measurement Accuracy

The following table shows NRSRP Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | 4.6dB | 3.85dB | 8.3dB |
| SNR=-6dB | 9dB | 9.2dB | 10.5dB |
| SNR=-15dB | 18.4dB | 18dB | 17.5dB |

### 3.1.2	NRSRQ Measurement Accuracy

The following table shows NRSRQ Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | 11.2dB | 3dB | 10.3dB |
| SNR=-6dB | 6.7dB | 6.8dB | 8.3dB |
| SNR=-15dB | 15.5dB | 15.5dB | 16dB |

## 3.2	Three subframe coherent averaging

### 3.2.1	NRSRP Measurement Accuracy

The following table shows NRSRP Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | 4.7dB | 3.6dB | 4.8dB |
| SNR=-6dB | 3.9dB | 5.6dB | 7dB |
| SNR=-15dB | 14.1dB | 15dB | 13.8dB |

### 3.2.2	NRSRQ Measurement Accuracy

The following table shows NRSRQ Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | -4.3dB | -4.2dB | -4.1dB |
| SNR=-6dB | -5dB | 3.2dB | 4dB |
| SNR=-15dB | 11.3dB | 12dB | 10.9dB |

## 3.3	Five subframe coherent averaging

### 3.3.1	NRSRP Measurement Accuracy

The following table shows NRSRP Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | -16.2dB | -18.1dB | -12.7dB |
| SNR=-6dB | -4.9dB | -4.9dB | -3.6dB |
| SNR=-15dB | 11.7dB | 11.7dB | 11.5dB |

### 3.3.2	NRSRQ Measurement Accuracy

The following table shows NRSRQ Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | -12.8dB | -15.3dB | -11.7dB |
| SNR=-6dB | -7dB | -7.7dB | -3.7dB |
| SNR=-15dB | 8.6dB | 8.6dB | 8.6dB |

# 4	EPA5Hz Simulation Results

## 4.1	One subframe coherent averaging

### 4.1.1	NRSRP Measurement Accuracy

The following table shows NRSRP Measurement Accuracy at 95-%tile poin:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | 7.6dB | 6.8dB | 7.6dB |
| SNR=-6dB | 11.2dB | 11.2dB | 11.2dB |
| SNR=-15dB | 17.2dB | 17.2dB | 18.2dB |

### 4.1.2	NRSRQ Measurement Accuracy

The following table shows NRSRQ Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | 10.5dB | 10.5dB | 12.2dB |
| SNR=-6dB | 6.2dB | 9dB | 9.4dB |
| SNR=-15dB | 16.4dB | 15dB | 16.3dB |

## 4.2	Three subframe coherent averaging

### 4.2.1	NRSRP Measurement Accuracy

The following table shows NRSRP Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | -5.8dB | 4.3dB | 4.3dB |
| SNR=-6dB | 7.7dB | 7.7dB | 8dB |
| SNR=-15dB | 13.7dB | 13.6dB | 13.7dB |

### 4.2.2	NRSRQ Measurement Accuracy

The following table shows NRSRQ Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | -9dB | -5dB | -4.9dB |
| SNR=-6dB | 6.7dB | 6.7dB | 5.1dB |
| SNR=-15dB | 12.5dB | 12.5dB | 11dB |

## 4.3	Five subframe coherent averaging

### 4.3.1	NRSRP Measurement Accuracy

The following table shows NRSRP Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | -12.3dB | -12.3dB | -11.1dB |
| SNR=-6dB | -3.4dB | -3.4dB | -3.4dB |
| SNR=-15dB | 11.5dB | 11.5dB | 11.4dB |

### 4.3.2	NRSRQ Measurement Accuracy

The following table shows NRSRQ Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms | Meas Period = 4sec |
| --- | --- | --- | --- |
| SNR=10dB | -11.4dB | -10dB | -8.3dB |
| SNR=-6dB | -6.4dB | -6.4dB | -6.9dB |
| SNR=-15dB | 9.1dB | 9.1dB | 8.6dB |

# 5	Conclusions

This paper contains simulation results for NRSRP and NRSRQ measurement accuracy for the purpose of alignment and subsequent requirement specification. Based on simulation results, we observe that three sub frame coherent averaging provides much better accuracy compared to one subframe coherent averaging and five subframe coherent averaging.

Proposal: Measurement accuracy requirements should be defined assuming three sub frame coherent averaging for NRSRP and NRSRQ measurements

# 6	References

- R4-79AH-0274, “Wayforward on NB-IoT RRM measurement accuracy simulation assumption”, Huawei, HiSilicon