3GPP TSG-RAN WG4 Meeting #80	R4-166615

# Gothenburg, Sweden, 22 - 26 Aug, 2016

Source:	Nokia

Title:	Simulation results of NPDCCH

Agenda item:	6.7.4.1

Document for:	Discussion

### 1	Introduction

With the completed Rel-13 NB-IoT work in RAN1 and RAN2 [1][2][3][4], RAN4 has started the performance specification work.

This contribution provides NPDCCH simulation results based on the agreed simulation [5] in the RAN4-79 ad hoc meeting.

### 2	Simulation assumptions

The simulation assumptions of NPDCCH are copied here from [5]:

# Table 3: Minimum performance for NPDCCH

| Simulation number | BW | Deployment Mode | Reference Channel | Repetition number | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) (Note 1) | Target SNR (dB |
| 1 | 180KHz | In-band | R.NB1.3 | Note 1 | [EPA5] | 2 | 1 | [-6] |
| 2 | 180KHz | In-band | R.NB1.4 | Note 1 | [ETU1] | 2 | 1 | [-12] |
| 3 | 180KHz | Standalone/Guard-band | R.NB1.3 | Note 1 | [EPA5] | 1 | 1 | [-6] |
| 4 | 180KHz | Standalone/Guard-band | R.NB1.4 | Note 1 | [ETU1] | 1 | 1 | [-12] |
| Note 1: Choose one repetition level from {16,32,64,128,256,512} to satisfy 1% BLER at target SNR=-6dB for tests 1 and 3, and SNR=-12dB for tests 2 and 4. Note 2: Assumption of In-band configuration: CFI=3 and the number of LTE CRS ports is 4. Demodulation is assumed to be based only NRS. |  |  |  |  |  |  |  |  |

# Table 4: FRC for NPDCCH

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB1.3 | R.NB1.4 |
| Number of transmitter antennas |  | 1 | 2 |
| Channel bandwidth | kHz | 180 | 180 |
| Aggregation level | NCCE | 2 | 2 |
| DCI Format |  | N1 | N1 |
| Cell ID |  | 0 | 0 |
| Payload (without CRC) | Bits | 23 | 23 |

### 3	Simulation results

The simulation results are listed in the table:

| Simulation number | BW | Deployment Mode | Reference Channel | Repetition number | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) (Note 1) | Target SNR (dB |
| 1 | 180KHz | In-band | R.NB1.3 | Note 1 | [EPA5] | 2 | 1 |  |
| 2 | 180KHz | In-band | R.NB1.4 | Note 1 | [ETU1] | 2 | 1 |  |
| 3 | 180KHz | Standalone/Guard-band | R.NB1.3 | Note 1 | [EPA5] | 1 | 1 |  |
| 4 | 180KHz | Standalone/Guard-band | R.NB1.4 | Note 1 | [ETU1] | 1 | 1 |  |
| Note 1: Choose one repetition level from {16,32,64,128,256,512} to satisfy 1% BLER at target SNR=-6dB for tests 1 and 3, and SNR=-12dB for tests 2 and 4. Note 2: Assumption of In-band configuration: CFI=3 and the number of LTE CRS ports is 4. Demodulation is assumed to be based only NRS. |  |  |  |  |  |  |  |  |

# Reference

[1]	R1-163593	“Introduction of NB-IoT”, Ericsson, RAN1 #84bis

[2]	R1-165971	36.211 CR, “Intorduction of NB-IoT”, Ericsson, RAN1 #85

[3]	R1-163409	“Draft 36213 for NB-IoT”, RAN1 #84bis

[4]	R2-163221	“36.331 Running CR to capture agreements on NB-IoT”, Huawei, RAN2 NB-IoT ad hoc

[5]	R4-79AH-0161	“Updated simulation assumptions for NB-IoT UE demodulation”, Huawei, HiSilicon, Ericsson, Qualcomm, Samsung, ZTE, RAN4 #79AH