3GPP TSG-RAN WG4 Meeting #80	R4-166679

Gothenburg, Sweden, 28 - 30 June 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3821 | rev | 1 | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Power headroom reporting requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  |  | Date: |  | 2016-06-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The power headroom reporting requirements are currently missing for NB-IOT UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: Power headroom reporting under normal coverage |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear how the UE will report the result of the meaurement, what ranges and resolution to use. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 9.1.8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.512-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

### 9.1.23	Power Headroom for UE Category NB1

The requirements in this clause shall apply for power headroom for UE Category NB1 as defined in [31].

The power headroom provides the serving eNB with information about the differences between the UE configured maximum output power (PCMAX,) defined in TS 36.101 [5] and the estimated power for UL-NSCH transmission of the serving cell [3].

#### 9.1.23.1	Period

The reported power headroom shall be estimated over 1 subframe of NPUSCH transmissions.

#### 9.1.23.2	Reporting Delay

The power headroom reporting delay is defined as the time between the beginning of the power headroom reference period and the time when the UE starts transmitting the power headroom over the radio interface. The reporting delay of the power headroom shall be 0 ms, which is applicable for all configured triggering mechanisms for power headroom reporting.

#### 9.1.23.3	Report Mapping for UE Category NB1

##### 9.1.23.3.1	Report Mapping for UE Category NB1 in normal coverage

The power headroom reporting range is from -23 ...+28 dB for UE category NB1 in normal coverage. Table 9.1.23.3.1-1 defines the report mapping.

Table 9.1.23.3-1: Power headroom report mapping for UE category NB1 in normal coverage

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -23 PH 4 |
| POWER_HEADROOM_1 | 4 PH 16 |
| POWER_HEADROOM_2 | 16 PH 28 |
| POWER_HEADROOM_3 | PH ≥ 28 |

##### 9.1.23.3.2	Report Mapping for UE Category NB1 in enhanced coverage

The power headroom reporting range is from -23 ...+13 dB for UE category NB1 in enhanced coverage. Table 9.1.23.3.2-1 defines the report mapping.

Table 9.1.23.3.2-1: NB-IOT power headroom report mapping in enhanced coverage

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -23 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

---END OF CHANGE 1---