3GPP TSG-RAN WG4 Meeting #80 	R4-166708

Gothenburg, Sweden, 22nd - 26th August 2016

Agenda item:	10.3.1

Source: 	NTT DOCOMO, INC.

Title: 	Evening AH minutes on Co-existence study for WP5D

Document for:	Approval

Introduction

The contribution summarizes minutes of evening AH meeting on Co-existence study for WP5D.

- Simulation assumptions

  - Network layout

  - Propagation model

  - Beam forming antenna pattern model

  - UL TPC model

  - ACLR/ACS model (ACIR model)

  - Received power model

  - Other simulation parameters

  - Simulation description

  - Evaluation metric

- Work Plan

  - For calibration procedure

  - For coexistence study

List of contributions

The list of contributions

| R4-165007 | NR coexistence study methodology and assumption | Intel Corporation |
| --- | --- | --- |
| R4-165008 | Preliminary simulation results of NR coexistence study for calibration | Intel Corporation |
| R4-165251 | Discussion on NR co-existence assumptions | China Telecom |
| R4-165495 | On modelling ACS for receiver antenna arrays | Ericsson |
| R4-165500 | Discussion on the uplink power control of NR | ZTE Corporation |
| R4-165501 | Discussion on spatial pattern for NR ACLR | ZTE Corporation |
| R4-165751 | Calibration results for 5G NR | CATT |
| R4-165752 | co-existence results for urban macro for 5G NR | CATT |
| R4-165753 | co-existence results for dense urban for 5G NR | CATT |
| R4-165754 | further considerations on 5G NR co-existence study | CATT |
| R4-165808 | Work plan on co-existence study for WP 5D | NTT DOCOMO INC. |
| R4-165809 | Simulation assumptions for co-existence study for WP 5D | NTT DOCOMO INC. |
| R4-165831 | Methodology and further assumptions for NR coexistence study | Qualcomm Incorporated |
| R4-165832 | Calibration curves for NR coexistence in indoor scenario | Qualcomm Incorporated |
| R4-165833 | Calibration curves for NR coexistence in urban macro scenario | Qualcomm Incorporated |
| R4-165899 | On modelling the spatial shape of ACLR | Ericsson |
| R4-165938 | Evaluation results for NR coexistence study in urban macro scenario | Samsung |
| R4-165939 | Evaluation results for NR coexistence study in dense urban scenario | Samsung |
| R4-165940 | Evaluation results for NR coexistence study in indoor scenario | Samsung |
| R4-166101 | Discussion on 5G simulation assumptions for WP5D evaluation | CMCC |
| R4-166102 | Preliminary simulation results of 5G NR for calibration | CMCC |
| R4-166198 | TP on Co-existence simulation scenario, assumption, and methodology for TR 38.803 | NTT DOCOMO INC. |
| R4-166221 | Simulation assumptions and preliminary results for NR co-existence study | NEC |
| R4-166378 | Further discussion on co-existence simulation assumptions | Huawei, Hisilicon |
| R4-166380 | WF on co-existence simulation assumptions | Huawei, Hisilicon |
| R4-166382 | 5G NR coexistence calibration in indoor scenario | Huawei, Hisilicon |

WP 5D co-existence study simulation assumptions

Network layout model

Urban macro

Session chair Note:

For network layout, most companies propose Hexagonal grid, 19-site, 57 sectors, with wrap-around which is used in TR38.900.

For Multi operator layout:

- Option 1: only uncoordinated operation

- Option 2: both coordinated and uncoordinated operations

| Coordinated Operation: each network with co-location of sites | Uncoordinated Operation: Aggressor network’s sites are located at the first network´s cell edge |
| --- | --- |

Discussion:

Can we agree with the Hexagonal grid, 19-site, 57 sectors, with wrap-around model as urban macro network model?

No comments.

Decision: Agree

Can we agree with both coordinated operation and uncoordinated operations as Multi operator layouts?

Ericsson: coordinated  0% grid shift and uncoordinated operation with 100% Grid shift should be also agreed.

Decision: Agree

Can we agree with following parameters for urban macro network layout?

Qualcomm: we discussed this morning on ISD and outage % impact. Everyone is ok with this, we are ok to keep it.

Nokia: if we keep 80%, then, most of UEs are indoor and this is less throughput situation as many UEs have extra penetration loss. So that it would be better to rethink this assumption.Qualcomm: we don’t have a strong preference. We need to look at outage that is our comment. Only average throughput?

DCM: Metric should be discussed carefully. It is challenging to decide it in this meeting.

Chair: Except for indoor UE ratio, can we agree with other parameters?

Intel: On system throughput, if UEs are in outage, do we need to consider these UE’s throughput? When we consider single system performance, when we calculate throughput, do we need to consider those UEs which are out of service.

Nokia: We considered 0 throughput UEs in the impact from the adjacent channel system.

Chair: now we are discussing UEs in outage area.

Nokia: my comment is a response to Intel.

Qualcomm: we can look at CDF of throughput for this scenario.

Nokia: But we want to fix some parameters for the calibration purpose to minimize the work.

Chair: One of the main purposes is establishing the parameters for calibration.

Nokia: Can I suggest using 50% indoor UE ratio?

Chair: there is no objection.

Huawei: why do we need to revise indoor UE ratio ?

Nokia: 80% of UEs see penetration loss based on the current assumption so that we see less impact of interference due to penetration loss.

Chair: can we use 50% as working assumption?

Huawei: we agree that there are many UEs do not get interferences. We also can peak SNR. We can add 80% to the table and 50% is an optional.

Nokia: we want to reduce the number of parameters otherwise the workload will increase. We can select 80% but we cannot use 5% threshold.

CMCC: Firstly, BF has large pathloss. Is there any approved 50% UE indoor ratio? We are not sure where this 50% comes from.

Nokia: we can keep 80%. Then, if we see some problem later, then we can fix it.

Huawei: we should not change assumptions as we made comments in the morning. We have evaluation metric in section 3.9. if we find out 5% throughput is not valid, then, we can discuss it later. If we have 80% UEs in indoor, so that most of UEs are in outage. So, it is better to modify this section but we should keep the simulation assumptions.

Decision: Following parameters are agreed

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 500 m |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | Specified in TR38.900 |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa | Specified in TR38.900 |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Multi operator layout |  | both coordinated operation and uncoordinated operation |  |

Dense urban

Session chair Note:

For network layout,

- Option 1: Manhattan grid

- Option 2: Random drop: 9 micro BSs per macro BS (Note: Micro layer under two layer deployments, but all UEs communicate with micro BS)

For Multi operator layout:

- Option 1: based on Manhattan grid model

- Option 2: randomly

| Manhattan model |  |
| --- | --- |
| Random drop model (operator 1 is illustrated) (for macro cellular, each network with co-location of sites) |  |

Discussion on the online session

- Can UMi channel model specified in TR38.900 be used for Manhattan model?

Discussion:

Which layout should we assumed for dense urban?

Option1:

Option2: ZTE, Intel, Samsung, Huawei, Qualcomm, China telecom, NEC

Agreement: Option 2

Can we agree with following parameters for dense urban network layout?

Nokia: 38.900 considered shadowing correlation so that we would like to include Between cite of 0.5.

Intel: This is for Macro, right? Not micro. In the past, if UEs are in the house, then they have the same chances regardless of Macro or Micro. 0.5 applies to both Macro and Micro. This was the assumption in the past.

Intel: For this sharing study, we consider only one layer. I’m not quite clear how this shadowing applies.

DCM: did RAN4 consider shadowing between two systems in past co-existence study?

Nokia: Yes, we did. All the cites get the same shadowing. You can see the history in table 6.3.2.1 in TR 37.809.

Decision: Following parameters are agreed

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | random drop |  |
| Inter-site distance |  | randomly |  |
| BS antenna height |  | 10 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS | Specified in TR38.900 |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 [6] |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 10m |  |
| Channel model |  | UMi | Specified in TR38.900 |
| Shadowing correlation |  | Between cite: 0.5 | Table 6.3.2.1 in TR37.809 |
| Multi operator layout |  | randomly |  |

For Manhattan grid model

A modified version of the Manhattan grid pattern from 3GPP TR 25.942 [5] (Figure 4.8) is used as a micro cell layout. The proposed modification produces a symmetrical grid pattern that can be replicated into a larger grid. An additional column with 3 new nodes has been added resulting in a 12 by 12 block grid, as shown in Figure 6.3.1.2-1. In addition, 3 nodes on the top of the grid have been removed since they are redundant with the nodes on the bottom of the original grid when wrap-around is employed. As per the Manhattan grid definition in 3GPP TR 25.942 [5], each block is 75m x 75m and each street is 15m wide. This results in the modified grid having a dimension of 1080m x 1080m.

Ericsson: we should not use TRP.

Intel: Is this minimum distance applicable to all of them?

Huawei: Why is the number of eNB 9? We should consider 3 as well.

Ericsson: we tend to agree with Huawei. Minimum distance between the operators. This should be clarified.

Chair: why does Huawei assume three?

Huawei: we think that the number of three is more suitable for adjacent co-existence study.

Ericsson: whenever we considered minimum distance of Micro TRPs between different operators which is collocated. It was 10 m.

ZTE: in this scenario, we don’t consider cluster concept?

Chair: YES.

Decision:  Following parameters are agreed

| For random drop model |  |  |
| --- | --- | --- |
| Assume 3BS |  |  |
| Number of the micro TRPs per macro TRP | Minimum distance between Micro TRPs in same operator (m) | Radius of UE dropping within a cluster: R (m) |
| 3 | 57.9 | <28.9 |
| 6 | 42.4 | <21.2 |
| 9 | 32 | <16 |

Note that: “TRP” will be changed.

Need to check Minimum distance between Micro TRPs in different operators (m)

Indoor

Session chair Note:

For network layout,

- Option 1: 50m x 120m, 12BSs (TR38.900 modelling)

- Option 2: 50m x 120m, [2-4]BSs

| Option 1 (operator 1 is illustrated) |  |
| --- | --- |
| Option 2 (operator 1 is illustrated) |  |

For Multi operator layout:

- Option 1: centre of other operator’s BSs

- Option 2: randomly with minimum distance

Discussion:

Which layout should we assumed for indoor hotspot?

Option 1: Ericsson, Intel, NEC, ZTE, Qualcomm

Option 2:

Agreement: Option 1

Which option for multi operator layout should we assumed?

ZTE: maybe we should shift 5 or 3m as minimum distance like LAA study. Minimum distance between different operators is 3 m.

Ericsson: we agree with ZTE. We should say that minimum distance not inter-site distance.

Qualcomm: inter-site distance should be replaced with minimum distance.

Intel: this shift is for randomly or fixed.

Qualcomm: we set minimum distance that is it. People can use more UEs as far as the distance is kept.

Agreement: Option 2, minimum distance between difference operators: 3m

Can we agree with following parameters for indoor hotspot?

China telecom: UE antenna height should be 1 m based on TR38.900.

Decision: Following parameters are agreed

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Option 1 |  |
| Inter-site distance |  | 20m |  |
| BS antenna height |  | 3 m | ceiling |
| UE location | Outdoor/indoor | Indoor |  |
|  | LOS/NLOS | LOS and NLOS | Specified in TR38.900 |
|  | UE antenna height | 1 m | Specified in TR38.900 |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 0 m |  |
| Channel model |  | Indoor Office | Specified in TR38.900 |
| Shadowing correlation |  | NA |  |
| Multi operator layout |  | Option 1 |  |

Propagation model

Session chair Note:

Channel models specified in TR38.900 are used.

Discussion:

Can we agree with following propagation models?

Decision:  Following parameters are agreed

Pathloss

The pathloss models are summarized in Table 7.4.1-1 and the distance definitions are indicated in Figure 7.4.1-1 and Figure 7.4.1-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 7.4.1-1.

| Figure7.4.1-1: Definition of d2D and d3D for outdoor UEs | Figure 7.4.1-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UEs. Note that d3D-out+d3D-in=d2D-out+d2D-in2+hBS-hUT2 |
| --- | --- |

Note that d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for RMa, UMa and UMi-Street Canyon. d2D-in shall be UT-specifically generated.

Table 7.4.1-1: Pathloss models

| Scenario | Pathloss [dB], fc is in GHz and d is in meters (6) | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| UMa LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 25 m |
| UMa NLOS |  | σSF =6 | 10 m < d2D < 5 000 m 1.5 m ≦ hUT ≦ 22.5 m hBS = 25 m Explanations: see note 3 |
| UMi - Street Canyon LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
| InH - Office LOS |  | σSF=3.0 | 1<d3D<100m |
| InH - Office NLOS |  | σSF=8.03 | 1<d3D<86m |
|  | Optional | σSF=8.29 | 1< d3D <86m |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified formatand and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |

7.4.2 	LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 7.4.2-1.

Table 7.4.2-1 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMi – Street canyon | Outdoor users: Indoor users: Use d2D-out in the formula above instead of d2D |
| UMa | Outdoor users: where and Indoor users: Use d2D-out in the formula above instead of d2D |
| Indoor – Open office |  |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

7.4.3	O-to-I penetration loss

The pathloss incorporating O-to-I building penetration loss is modelled as in the following:

PL = PLb + PLtw + PLin + N(0, σP2)

where PLb is the basic outdoor path loss given in Section 7.4.1. PLtw is the building penetration loss through the external wall, PLin is the inside loss dependent on the depth into the building, and σP  is the standard deviation for the penetration loss.

PLtw is characterized as:

is an additional loss is added to the external wall loss to account for non-perpendicular incidence;

, is the penetration loss of material i, example values of which can be found in Table 7.4.3-1.

pi is proportion of i-th materials, where ; and

N is the number of materials.

Table 7.4.3-1. Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass | Lglass=2+0.2⋅f |
| IRR glass | LIRRglass=23+0.3⋅f |
| Concrete | Lconcrete=5+4⋅f |
| Wood | Lwood=4.85+0.12⋅f |
| Note: f is in GHz |  |

Table 7.4.3-2 gives PLtw, PLin and σP  for two O-to-I penetration loss models. The O-to-I penetration is UT-specifically generated, and is added to the SF realization in the log domain.

Table 7.4.3-2 O-to-I penetration loss model

|  | Path loss through external wall: PLtw [dB] | Indoor loss: PLin [dB] | Standard deviation: σP [dB] |
| --- | --- | --- | --- |
| Low-loss model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10 | 0.5d2D-in | 4.4 |
| High-loss model | 5-10log100.7⋅10-LIRRglass10+0.3⋅10-Lconcrete10 | 0.5d2D-in | 6.5 |

d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for RMa, UMa and UMi-Street Canyon. d2D-in shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

The composition of low and high loss is a simulation parameter that should be determined by the user of the channel models, and is dependent on the use of metal-coated glass in buildings and the deployment scenarios. Such use is expected to differ in different markets and regions of the world and also may increase over years to new regulations and energy saving initiatives. Furthermore, the use of such high-loss glass currently appears to be more predominant in commercial buildings than in residential buildings in some regions of the world.

The pathloss incorporating O-to-I car penetration loss is modelled as in the following:

PL = PLb + N(μ, σP2)

where PLb is the basic outdoor path loss given in Section 7.4.1. μ = 9, and σP = 5. Optionally, for metallized car windows, μ = 20 can be used. The O-to-I car penetration loss models are applicable for at least 0.6-60 GHz.

Beam forming antenna pattern model

BS Beam forming antenna pattern model for Urban macro

Session chair Note:

Discussion:

Discussed based on WF from Huawei.

Nokia: we would like to know the difference between AAS BF model and model captured in the draft WF by Huawei.

Ericsson: there are some small differences such as antenna pattern.

Huawei: What is the comments from Nokia?

Nokia: we don’t see any justification to change the AAS model. Why do we need to change AAS model.

Huawei: Then, we would like to ask Nokia read our paper. There are a lot of issues not to be resolved in AAS modelling.

Nokia: In your paper, you have already proposed to use UE specific BF. What kinds of BF metric you are considering.

Ericsson: there would be some differences like antenna spacing etc. our proposal is that we see the draft WF Huawei is going to prepare.

Qualcomm: the issue is not BF model but rather many parameters to generate models.

Decision: 	no agreement

BS Beam forming antenna pattern model for Dense urban

Session chair Note:

Discussion:

Decision: 		The document was not treated.

BS Beam forming antenna pattern model for Indoor

Session chair Note:

Discussion:

Decision: 		The document was not treated.

UE Beam forming antenna pattern model

Session chair Note:

Discussion:

Qualcomm: are there any opinions on BF gain?

ZTE: Can we follow the RAN1 assumption?

Qualcomm: we are checking the RAN1 assumption. We are going to propose some by downselection.

Nokia: we need to think about antenna patter direction.

Qualcomm: our plan is mostly following RAN1 assumptions.

Nokia: we may need to set the deadline otherwise our work is completely relying on their work.

Qualcomm: Random orientation is more sense. Nokia wants to propose UEs to have specific orientation always to eNB? I think we would like to see what RAN1 agreed and to discuss what RAN4 should do.

ZTE: we can share the spreadsheet on the reflector.

Decision: 	no agreement

Beam direction

Session chair Note:

Discussion:

Can we assume ideal beam direction as a simulation assumption?

Decision: 		The document was not treated.

ACLR/ACS model (ACIR model)

Session chair Note:

To be added later

Discussion:

Discussed based on Draft WF from Ericsson.

Ericsson: we shared draft WF.

Nokia: Flat ACLR means in spatial domain only?  People may think this applies to Frequency domain as well. Underlying assumption, it is better to say that 100% correlation. Also, this single ACS should have the same story.

Qualcomm: On this disclaimer part, we do believe that this is the case. We should study this based on this.

Ericsson: At this stage, we do not have to make a decision on correlated or not, or worst case or not.  On ACS, the intention is not for blocking.

ZTE: our assumption is 100% correlation. If we check the AAS study, each emission is created by each panel.

Ericsson: different beams use difference subcarriers.

Decision: 		The document was not treated.

Received power model

Session chair Note:

To be added later

Discussion:

Can we agree with following methodology?

RX_PWR = TX_PWR – Max (pathloss – G_TX – G_RX, MCL)

where:

RX_PWR is the received signal power, TX_PWR is the transmitted signal power

G_TX is the transmitter antenna gain, G_RX is the receiver antenna gain

MCL is the minimum coupling loss

Do we need MCL in the above definition?

Chair: if we specify minimum distance, can MCL be removed?

ZTE: we propose to remove MCL.

Nokia: Can we clarify that we are using minimum distance. We also need to state that we use minimum distance. Then, with this clarification, we don’t need MCL.

Ericsson: we need minimum distance. We need to be careful about actual gain.

Decision: 		No agreement

UL TPC model

Session chair Note:

Proposal from Nokia

(a) For 30 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1a | 1 | 108 | 109 | 110 | 111 |
| Set 2a | 0,8 | TBD | TBD | 119 | 121 |

(b) For 70 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 15 MHz bandwidth | 10 MHz bandwidth | 5 MHz bandwidth |
| Set 1a | 1 | 116 | 116 | 117 | 119 |
| Set 2a | 0,8 | TBD | TBD | 126 | 128 |

Proposal from ZTE

Table 1: Power level of Po_pusch of NR

| Po_pusch | NR scheduled BW |  |
| --- | --- | --- |
|  | 60MHz per UE | 180MHz per UE |
| Power Level | -73.2dBm | -68.4dBm |

Table 2: Power control algorithm parameter [30GHz&70GHz]

| Parameter set | Gamma | CLxile |  |
| --- | --- | --- | --- |
|  |  | 60MHz per UE | 180MHz per UE |
| Set 1 | 1 | 96.2 | 91.4 |

Discussion:

Decision: 		No agreement

Other simulation parameters

Session chair Note:

To be added later

Discussion:

Can we agree with following parameters?

| Parameters | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Traffic model | Full buffer | Full buffer | Full buffer |
| DL power control | NO | NO | NO |
| UL power control | YES | YES | YES |
| BS max TX power in dBm | 23dBm | 43dBm | 33dBm |
| UE max TX power in dBm | 23dBm | 23dBm | 23dBm |
| BS Noise figure in dB | 5dB | 5dB | 5dB |
| UE Noise figure in dB | 13dB | 13dB | 13dB |
| Handover margin | 3dB | 3dB | 3dB |

Are there any other parameters to be specified for co-existence simulation?

Decision: 		The document was not treated.

Simulation description

Session chair Note:

Discussion:

Can we agree with following methodology? IF YES, which Option in step 2 should be used?

Qualcomm R4-165831 (Main differences compared to the classical RAN4 methodology are Step 2 and Step 3.)

- Aggressor and victim network are generated.

- UE associations: UE are associated to base station based on coupling loss. In this step there are two possible alternatives:

  - Associations are made assuming a single element pointing to the sector direction.

  - Associations are made assuming the MxN array pointing at the sector direction.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This done for both victim and aggressor networks.

- SINR Throughput are measured in the victim systems without considering ACI, i.e. ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

- SINR and throughput are computed considering ACI: ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

- RF parameters are determined based degradation cause by ACI: LossACI=1-ThputACIThputSINGLE.

Decision: 		The document was not treated.

Evaluation metric

Session chair Note:

RAN4 agreements in the last meeting are as below;

Adopt scaled Shannon's formula with update truncation and attenuation parameters until we get RAN1, we will use 36.942 Scaled Shannon’s formula.

Throughput [bpsHz] = 0for SINR<SINRMINα×S(SINR)for SINRMIN<SINR<SINRMAXTHRMAXfor SINR>SINRMAX

Discussion:

Decision: 		The document was not treated.

Co-existence scenarios

Session chair Note:

We have already agreed that at least following scenarios are assumed;

- UL to UL and DL to DL interference

- eMBB

- Indoor hotspot, Urban macro, Dense urban

- 30GHz and 70GHz

Discussion:

Can we agree following scenarios ?

| No. | Aggressor | Victim | Simulation frequency | Direction | Usage scenario | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NR | NR | 30 GHz | DL to DL | eMBB | Indoor hotspot |
| 2 | NR | NR | 30 GHz | DL to DL | eMBB | Urban macro |
| 3 | NR | NR | 30 GHz | DL to DL | eMBB | Dense urban |
| 4 | NR | NR | 30 GHz | UL to UL | eMBB | Indoor hotspot |
| 5 | NR | NR | 30 GHz | UL to UL | eMBB | Urban macro |
| 6 | NR | NR | 30 GHz | UL to UL | eMBB | Dense urban |
| 7 | NR | NR | 70 GHz | DL to DL | eMBB | Indoor hotspot |
| 8 | NR | NR | 70 GHz | DL to DL | eMBB | Dense urban |
| 9 | NR | NR | 70 GHz | UL to UL | eMBB | Indoor hotspot |
| 10 | NR | NR | 70 GHz | UL to UL | eMBB | Dense urban |

Decision: 		The document was not treated.

Work plan

For calibration procedure

To be added later

For coexistence study

To be added later

Conclusions

To be added later