3GPP TSG-RAN WG4 Meeting #80	R4-166770

Gothenburg SE, 22 - 26 Aug 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3724 | rev | 1 | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Cat-M1 PRACH test cases for FDD in Enhanced Coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Perf |  |  |  |  |  |  | Date: |  | 2016-08-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Cat-M1 PRACH Test Cases for FDD in Enhanced Coverage are not defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define Cat-M1 PRACH Test Cases for FDD in Enhanced Coverage |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No Cat-M1 PRACH Test Cases for FDD in Enhanced Coverage |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section A.6.2.x1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

### A.6.2.x1	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage

#### A.6.2.x1.1	Test Purpose and Environment

The purpose of this test is to verify whether the behavior of the random access procedure of a Cat-M1 UE in Enhanced Coverage is according to the requirements, whether the PRACH power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the RSRP measurement and the configured criterion in RSRP-ThresholdsPrach [2].  This test will verify the requirements in Clause 6.2.2, Clause 6.2.3 and Clause 7.1.2 in an AWGN model.

For this test a single cell is used. The test parameters are given in tables A.6.2.x1.1-1 and A.6.2.x1.1-2.

Table A.6.2.x1.1-1: General test parameters for FDD contention based random access test

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Pattern Note 1 |  | OP.21 FDD | As defined in A.3.2.1.21. |  |
| PDSCH parameters Note 2 |  | R.22 FDD | As defined in A.3.1.4.4 |  |
| MPDCCH parameters Note 2 |  | R.18 FDD | As defined in A.3.1.3.4 |  |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As defined in A.3.1.2.1 |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| MPDCCH_RA | dB |  |  |  |
| MPDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RA Note 1 | dB |  |  |  |
| OCNG_RB Note 1 | dB |  |  |  |
|  | dBm/15 KHz | -98 |  |  |
|  | dB | -12 |  |  |
| Note 3 | dB | -12 |  |  |
| RSRP Note 3 | dBm/15 KHz | -110 |  |  |
| Io Note 3 | dBm/9 MHz | -70 |  |  |
| Propagation Condition | - | AWGN |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The PDSCH and MPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, RSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |

Table A.6.2.x1.1-2: RACH-Configuration parameters for FDD contention based random access test

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| Parameters not per CE Levels |  |  |  |  |  |
| powerRampingStep | dB2 |  |  |  |  |
| preambleInitialReceivedTargetPower | dBm-120 |  |  |  |  |
| preambleTransMax | n6 |  |  |  |  |
| maxHARQ-Msg3Tx | 4 |  |  |  |  |
| rar-HoppingConfig | Off |  |  |  |  |
| Parameters per CE Levels |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 | Level 3 |  |
| ra-ResponseWindowSize (per CE) | sf20 | sf80 | sf180 | sf320 |  |
| mac-ContentionResolutionTimer (per CE) | sf80 | sf120 | sf200 | sf480 |  |
| PreambleMappingInfo {firstPreamble, lastPreamble} | {0, 9} | {10,19} | {20,29} | {30,39} |  |
| Note: For further information see Clause 6.3.2 in TS 36.331. |  |  |  |  |  |

Table A.6.2.x1.1-3: PRACH-Configuration parameters for FDD contention based random access test

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| Parameters not per CE Levels |  |  |  |  |  |
| rsrp-ThresholdsPrach | {16,26,36} |  |  |  | Corresponding to {-124, -114, -104} dBm as defined in Section 9.1.21.5 |
| mpdcch-startSF-CSS-RA | v1 |  |  |  |  |
| referenceSignalPower | -5 dBm/15 KHz |  |  |  | As defined in clause 6.3.2 in TS 36.331. |
| maxHARQ-Msg3Tx | 4 |  |  |  | As defined in table 5.7.1-2 in TS 36.211 |
| Backoff Parameter Index | 2 |  |  |  | As defined in table 7.2-1 in TS 36.321 |
| Configured UE transmitted power () | 23 dBm |  |  |  | As defined in clause 6.2.5 in TS 36.101 |
| Parameters per PRACH CE Levels |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 | Level 3 |  |
| prach-ConfigIndex | 4 | 4 | 4 | 4 | As defined in table 5.7.1-2 in TS 36.211 |
| prach-FreqOffset | 0 | 0 | 0 | 0 |  |
| prach-StartingSubframe | sf2 | sf4 | Sf8 | sf16 |  |
| maxNumPreambleAttempt | n3 | n5 | n7 | n10 |  |
| numRepetitionPerPreambleAttempt | n1 | n4 | n16 | n64 |  |
| mpdcch-NarrowbandsToMonitor | 2 | 2 | 2 | 2 |  |
| mpdcch-NumRepetition-RA | r8 | r8 | r128 | r128 |  |
| prach-HoppingConfig | Off | Off | Off | Off |  |
| Note 1: See Clause 6.3.2 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |

#### A.6.2.x1.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.6.2.x1.2.1	Random Access Response Reception

To test the UE behavior specified in Subclause 6.2.2, the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after [5] preamble transmission attempts (the preamble may be transmitted multiple times in each attempt) have been received by the System Simulator. In response to the first [4] preamble transmission attempts, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2. The power of the first preamble shall be -30 dBm. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.5.1.1 of TS 36.101 [5]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.5.2.1 of TS 36.101 [5].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.x1.2.2	No Random Access Response Reception

To test the UE behavior specified in subclause 6.2.2.1.2, the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after [5] preamble transmission attempts have been received by the System Simulator. The System Simulator shall not respond to the first [4] preamble transmission attempts.

The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window.

In addition, the power applied to all preambles shall be in accordance with what is specified in Subclause 6.2.2. The power of the first preamble shall be -30 dBm with an accuracy specified in clause 6.3.5.1.1 of TS 36.101 [5]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.5.2.1 of TS 36.101 [5].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Subclause 7.1.2.

A.6.2.x1.2.3	Receiving a NACK on msg3

To test the UE behavior specified in subclause 6.2.2.1.3, the System Simulator shall NACK all UE msg3 following a successful Random Access Response.

The UE shall re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of HARQ re-transmissions is reached.

A.6.2.x1.2.4	Reception of an Incorrect Message over Temporary C-RNTI

To test the UE behavior specified in Subclause 6.2.2.1.5, the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

A.6.2.x1.2.5	Reception of a Correct Message over Temporary C-RNTI

To test the UE behavior specified in Subclause 6.2.2.1.5, the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

The UE shall send ACK if the Contention Resolution is successful.

A.6.2.x1.2.6	Contention Resolution Timer expiry

To test the UE behavior specified in Subclause 6.2.2.1.6, the System Simulator shall not send a response to a msg3.

The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

A.6.2.x1.2.7	PRACH Resource Selection

The UE shall select PRACH resources and transmits or re- transmits PRACH preambles using the PRACH resources and PRACH configuration corresponding to the coverage enhancement level 1.

--- END OF CHANGES ---