3GPP TSG-RAN WG4 Meeting #80                                   R4-166105

Gothenburg, China, 22 - 26 Aug, 2016

Agenda Item:		8.1.2

Source: 		CMCC

Title: 	TP on TR 36.714-00-00 for CA_40D and CA_41D

Document for:		Discussion

- Introduction

Intra-band contiguous CA of 3UL in B40 and B41 were approved in Ran#72 meeting [1]. This TP is based on the analysis in [2].

- Reference

- RP-160822, Revised WID: LTE Advanced intra-band CA Rel-14 for xDL/yUL including contiguous and non-contiguous spectrum

- R4-166106, Discussion on RF requirements for 3UL intra-band contiguous CA of CA_40D and CA_41D, CMCC

- Text Proposal

<Start of Text Proposal for TR 36.714-00-00>

- 6.x	CA_40D_3UL_BCS0

- 6.x.1	Channel bandwidths per operating band

Table 6.x.1-1 defines the supported E-UTRA CA configurations and bandwidth combination set for LTE-advanced 3UL intra-band contiguous CA in Band 40.

Table 6.x.1-1: E-UTRA CA configurations and bandwidth combination sets for CA_40D

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] Channel bandwidths for carrier [MHz] |  |  |
| CA_40D | CA_40D | 10, 15, 20 | 20 | 20 | 60 | 0 |
|  |  | 20 | 10, 15 | 20 |  |  |
|  |  | 20 | 20 | 10, 15 |  |  |

- 6.x.2	UE maximum output power for

The UE maximum output power for CA_40D is proposed to reuse requirement for bandwidth class C.

Table 6.x.2-1 CA UE Power Class for intra-band contiguous CA_40D

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_40D |  |  |  |  | 23 | +2/-2 |  |  |

- 6.x.3	UE Maximum Output power for modulation / channel bandwidth

The requirements are FFS.

- 6.x.4 Power control

Requirements for bandwidth class B and C can be reused.

- 6.x.5 Transmit modulation quality

The requirements are FFS.

- 6.x.6 Spectrum emission mask

The requirements are FFS.

- 6.x.7 UTRA and E-UTRA ACLR requirements

Requirements for bandwidth class B and C can be reused.

- 6.x.8 Spurious emission band UE co-existence

The requirements are FFS.

- 6.x.9 Transmit intermodulation

Requirements for bandwidth class B and C can be reused.

- 6.y	CA_41D_3UL_BCS0

- 6.y.1	Channel bandwidths per operating band

Table 6.y.1-1 defines the supported E-UTRA CA configurations and bandwidth combination set for LTE-advanced 3UL intra-band contiguous CA in Band 41.

Table 6.y.1-1: E-UTRA CA configurations and bandwidth combination sets for CA_41D

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] Channel bandwidths for carrier [MHz] |  |  |
| CA_41D | CA_41D | 10 | 20 | 15 | 60 | 0 |
|  |  | 10 | 15, 20 | 20 |  |  |
|  |  | 15 | 20 | 10, 15 |  |  |
|  |  | 15 | 10, 15, 20 | 20 |  |  |
|  |  | 20 | 15, 20 | 10 |  |  |
|  |  | 20 | 10, 15, 20 | 15, 20 |  |  |

- 6.y.2	UE maximum output power

The UE maximum output power for CA_41D is proposed to reuse requirement for bandwidth class C.

Table 6.y.2-1 CA UE Power Class for intra-band contiguous CA_41D

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_41D |  |  |  |  | 23 | +2/-22 |  |  |
| NOTE 2: If all transmitted resource blocks (Figure 5.6A-1) over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

- 6.y.3 Spurious emission band UE co-existence

Table 6.x.8-1 shows the spurious emission band UE co-existence for CA_41D.

Table 6.y.3-1: Spurious emission band UE co-existence for CA_41D

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_41D | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 23, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 65, 66 | FDL_low | - | FDL_high | -50 | 1 |  |

- 6.y.4 UE maximum output power with additional requirements

The requirements are FFS.

- 6.y.5 Additional Spectrum emission mask for CA_41D

The requirements are FFS.

<End of Text Proposal for TR 36.714-00-00>