# 3GPP TSG-RAN WG4 Meeting #80	   	                                           R4- 166910

# Gothenburg, Sweden, 22 – 26 Aug, 2016

Title:                           TP for TR 36.714-02-02 on REFSENS for dual uplink of CA_8A-41A

Source: 	CMCC

Agenda Item:	8.6.2

Document for:	Approval

1.	Introduction

The revised WID of supporting the LTE Advanced 2 Band Carrier Aggregation (2DL/2UL) of Band 8 and Band 41 was approved in RAN#70.

In this contribution, MSD requirements are proposed for LTE Advanced 2 Band Carrier Aggregation (2DL/2UL) of Band 8 and Band 41.

2.	Discussion

The test bandwidths need to follow the way forward on 2UL inter-band CA MSD test configurations [1].

“Choose a specific carrier frequency allocation such that IMD is center-aligned with victim DL carrier to attest the worst-case self-desensitization condition.

Choose 5-MHz bandwidth with full RB for all carriers (if supported in a CA combination) to essentially comply with the Rel-8 UL configurations for REFSENS tests and approach the worst-case self-desensitization condition.

If 5-MHz bandwidth is not supported in certain CA combinations, then choose 10 MHz with full RB or the RB number which complies with Rel-8 UL configurations for REFSENS tests. ”

As shown above. Choose 5-MHz bandwidth with full RB for all carriers (if supported in a CA combination) to essentially comply with the Rel-8 UL configurations for REFSENS tests and approach the worst-case self-desensitization condition. So 5 MHz carrier bandwidth is chosen for B8.

According to the WF on MSD definition for 2UL Inter-band CA combinations that have multiple IMD’s for one band [2], IMD3 and IMD5 will be specified in the specification.

Table 2-1 MSD for IMD3 and IMD5

| CA | band | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |
| B8+B41 | B8 | IMD3 | fB41 – 2*fB8 | 882.5 | 5 | 25 | 927.5 | 12.1 |
|  | B41 |  |  | 2685 | 10 | 50 |  | N/A |
|  | B8 | IMD5 | 4*fB8-fB41- | 900 | 5 | 25 | 945 | 5.7 |
|  | B41 |  |  | 2655 | 10 | 50 |  | N/A |

3.	Reference

[1] R4-144031, Way forward on 2UL inter-band CA MSD test configurations, MediaTek Inc., TeliaSonera

[2] R4-164839, “MSD definition for 2UL Inter-band CA combinations that have multiple IMD’s for one band ”, Huawei, Hisilicon, NTT DOCOMO Inc,

4    Text Proposal

<Start of change>

# 6           Dual Uplink Inter-band Carrier Aggregation: Specific Band Combination Part

### 6.4	CA_8A-41A_

<< Unchanged sections omitted >>

6.4.5 	MSD

The 3rd and 5th IMD may fall into own Rx of band 8. Table 6.4.5-1 is MSD analyses which assume 10 dB antenna isolation and single antenna architecture with respect to each IMD order.

Table 6.4.5-1: MSD analysis for IMD3 and IMD5

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_8A-41A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD3 |
|  | 41 | 2685 | 10 | 50 | 2685 | N/A | TDD |  |
| CA_8A-41A | 8 | 900 | 5 | 25 | 945 | 5.7 | FDD | IMD5 |
|  | 41 | 2655 | 10 | 50 | 2655 | N/A | TDD |  |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

----- End of TP -----