3GPP TSG-RAN WG4 RAN4 #80bis                                       R4-167363

Ljubljana, Slovenia, 10 - 14 Oct, 2016

Agenda item:			9.3.3

Source:			Samsung

Title:	Simulation Results on NB-IoT Coexistence Study with CDMA system for Downlink Scenarios

Document for:			Discussion

# 1	Introduction

The study item on NB-IoT RF requirement for coexistence with CDMA was approved at RAN #72. According to WID [1], the study item has set up objectives as follows:

- Identify operating bands for interference evaluation on NB-IoT coexistence with CDMA, such as bands around 800M (B5, B26 etc.) - For the NB-IoT stand-alone operation mode, evaluate coexistence requirement between NB-IoT and CDMA, and further evaluate whether R13 NB-IoT RF requirements could be reused or not. - Identify necessary additional RF requirements to ensure NB-IoT for co-existence with CDMA.

In last RAN4 meeting, simulation assumptions and evaluation methodology for the coexistence study between CDMA and NB-IoT are agreed in the WF [2] as follows:

- Standalone NB-IoT to coexistence with CDMA 1x should be evaluated in this study. Cases; Aggressor system; Victim system; Link Direction; Deployment. 1; NB-IoT; CDMA1x; DL; Uncoordinated. 2; CDMA1x; NB-IoT; DL; Uncoordinated. 3; NB-IoT; CDMA1x; UL; Uncoordinated. 4; CDMA1x; NB-IoT; UL; Uncoordinated. - The evaluation methodology applied for coexistence study of NB-IoT system in TR36.802 is reused. - Static system simulation - The results should be presented as a function of ACS (when NB-IoT is victim) and as a function of ACLR (when NB-IoT is aggressor) - Metrics and criteria for coexistence study - For NB-IoT, SINR loss s less or equal to 1dB - For CDMA , Capacity loss is less or equal to 5% - Simulation parameters of Standalone NB-IoT for R13 coexistence study are reused. - Simulation parameters of CDMA 1x are mainly refer to TR25.942, while the CDMA1x BS and UE’s ACLR and ACS values are derived from 3GPP2 specs in C.S0010-D and C.S0011-D.

In this contribution, we provide the initial simulation results for the downlink coexistence simulation scenarios between NB-IoT and CDMA system according to the agreed simulation assumptions in the WF. And we also provide the simulation results for the uplink cases in our accompanying contribution [3].

# Coexistence Simulation Results

As described in [2], there are total 4 coexistence simulation cases between NB-IoT and CDMA2000 1x system. In this paper, we provide the simulation results for the two downlink cases, i.e. Case 1 and Case 2 in below table.

Table 2.1 Simulation cases of coexistence study for NB-IoT with CDMA

| Cases | Aggressor system | Victim system | Link Direction | Deployment |
| --- | --- | --- | --- | --- |
| 1 | NB-IoT | CDMA1x | DL | Uncoordinated |
| 2 | CDMA1x | NB-IoT | DL | Uncoordinated |
| 3 | NB-IoT | CDMA1x | UL | Uncoordinated |
| 4 | CDMA1x | NB-IoT | UL | Uncoordinated |

The detailed simulation assumptions and parameters are agreed in the WF [2] as listed in the Appendix.

The simulation results are provided in Table 2.2 and Table 2.3 for Case 1 and Case 2 respectively. For Case 1 in Table 2.2, the results are CDMA capacity loss (%) at different NB-IoT BS ACLR value. For Case 2 in Table 2.3, the results are NB-IoT SINR loss (dB) at different NB-IoT UE ACS value.

Note that the NB-IoT BS ACLR value and NB-IoT UE ACS value are both defined on adjacent measurement channel bandwidth 180 kHz.

Table 2.2 Simulation results for Case 1: Downlink, NB-IoT aggressor, CDMA victim

| NB-IoT BS ACLR | 40 | 45 | 50 | 55 | 60 |
| --- | --- | --- | --- | --- | --- |
| CDMA capacity loss (%) | 1.44 | 0.62 | 0.28 | 0.09 | 0.03 |

Table 2.3 Simulation results for Case 2: Downlink, CDMA aggressor, NB-IoT victim

| NB-IoT UE ACS | 20 | 25 | 30 | 35 | 40 |
| --- | --- | --- | --- | --- | --- |
| SINR loss 5%-tile | 1.55 | 0.76 | 0.40 | 0.24 | 0.16 |
| SINR loss 50%-tile | 0.73 | 0.38 | 0.21 | 0.15 | 0.11 |
| SINR loss 95%-tile | 0.14 | 0.07 | 0.03 | 0.02 | 0.01 |
| SINR loss 99%-tile | 0.04 | 0.01 | 0.00 | 0.00 | 0.00 |

Based on the above results, it can be observed that:

- For Case 1, i.e. NB-IoT interfering CDMA in downlink, even when NB-IoT BS ACLR is 40 dB (defined on 180 KHz bandwidth), it can well satisfy the CDMA performance loss criteria that average downlink capacity loss is less than 5%.

- For Case 2, i.e. CDMA interfering NB-IoT in downlink, when NB-IoT UE ACS is 25 dB (defined on 180 KHz bandwidth), the NB-IoT downlink SINR loss is less than 1 dB. So, to satisfy the NB-IoT performance loss criteria, the requirement for NB-IoT UE ACS should be equal or larger than 25 dB for its own bandwidth.

The above observations can be summarized as below:

Observation 1: The CDMA performance loss criteria can be well satisfied even when NB-IoT BS ACLR is 40 dB for its own bandwidth.

Observation 2: To satisfy NB-IoT performance loss criteria, the requirement for NB-IoT UE ACS should be equal or larger than 25 dB for its own bandwidth.

# Conclusion

In this contribution, we provide our results for the downlink scenarios based on the simulation assumptions and parameters provided in the WF [2]. Based on the simulation results, the following observations are given:

Observation 1: The CDMA performance loss criteria can be well satisfied even when NB-IoT BS ACLR is 40 dB for its own bandwidth.

Observation 2: To satisfy NB-IoT performance loss criteria, the requirement for NB-IoT UE ACS should be equal or larger than 25 dB for its own bandwidth.

# Reference

[1] 3GPP RP-161187, “WID: Study on NB-IoT RF requirement for coexistence with CDMA”, China Telecom.

[2] 3GPP R4-166994, “Way Forward on Simulation Assumption and Methodology for the coexistence study between CDMA and NB-IoT”, China Telecom, Samsung, Huawei, Qualcomm, Ericsson.

[3] 3GPP R4-167364, “Updated Simulation Results on NB-IoT Coexistence Study with CDMA system for Uplink Scenarios”, Samsung.

# Appendix

The detailed simulation parameters are provided as below:

Table 5.1 Simulation assumption for coexistence study between NB-IoT and CDMA2000 1x system

|  | NB-IoT standalone | CDMA2000 1x |
| --- | --- | --- |
| Carrier frequency in GHz | 0.85 | 0.85 |
| Size of each nominal channel BW in MHz | 0.2 | 1.23 |
| Transmission bandwidth in MHz | 0.18 | 1.23 |
| Environment | Urban macro | Urban macro |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around |
| Inter-site distance in meter | 750m (Same for aggressor and victim) | 750m (Same for aggressor and victim) |
| System loading and activity | Full buffer 100% | All users with 9.6kbps speech |
| Network location | Non co-located (at cell edge of legacy) | Non co-located with IoT |
| DL subcarrier spacing | 15kHz | n/a |
| UL | See RP-152284 | n/a |
| DL power control | No | TR25.942 (reuse UMTS PC) |
| UL power control | 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB | TR25.942 (reuse UMTS PC) |
| Frequency reuse | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | According to 95% users achieving target of (Eb/N0-0.5)dB; non orthogonality 0.2; target Eb/N0=5.5dB |
| Number of scheduled UE per cell (UL) | 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone | according to 5.5dB noise rise; target Eb/N0=4.0dB no MUD utilized, beta=0 |
| BS antenna height in meter | 30 | 30 |
| BS max TX power in dBm | 43/200kHz | 43 |
| BS Tx power of common channel in dBm* | n/a | 36.5 (R4-030558 22.4% Total Power) |
| BS antenna gain including feeder loss in dBi | 15 | 15 |
| BS antenna pattern | Horizontal (TR36.942) | Horizontal (65deg for -3dB) |
| BS antenna front-back ratio in dB | 20 | 20 |
| UE antenna height in meter | 1.5 | 1.5 |
| UE TX power in dBm | -40 to 23 | -52 to 23 |
| UE antenna gain in dBi | 0 | 0 |
| Building penetration loss | 45.820 Annex D.1 | n/a |
| Cell selection margin in dB | 3 | 3 |
| BS-MS min couple loss in dB | 70 | 70 |
| BS ACLR in dB | 40 to 60** | 37.2 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |
| BS ACS in dB | 40 to 50** | 71 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |
| UE ACLR in dB | 20 to 50** | 34.2 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |
| UE ACS in dB | 20 to 40** | 49.5 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |
| BS noise figure in dB | 5 | 5 |
| UE noise figure in dB | 9 | 9 |
| BS-UE path-loss model | TR36.942 macro urban | TR36.942 macro urban |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 | 10 |
| Shadowing correlation | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 |
| Evaluation metrics | SINR vs ACS (as victim) | Capacity vs NB-IoT ACLR (as victim). For DL, capacity is the number of satisfied users. For UL, capacity is the number of users when 5.5dB noise rise is reached |
| Carrier separation*** | 1.1MHz to CDMA(central frequency separation) | 1.1MHz to NB-IoT(central frequency separation) |

*DL overhead channel resource consumption, including pilot, paging and sync overhead.

**Similar to coexistence study in TR36.802, NB-IoT ACLR and ACS are defined for its own bandwidth size.

*** Other carrier separation options are not excluded.