3GPP TSG-RAN WG4 Meeting #80bis 	R4-167870

# Ljubljana, SI, 10 - 14 Oct 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 0867 | rev |  | Current version: | 13.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: NPRACH performance requirements (TS36.104) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  |  | Date: |  | 2016-09-29 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Introduction of NPRACH performance for NB-IoT |  |  |  |  |  |  |  |  |
| Summary of change: |  | New sections 8.6 for NPRACH performance for NB-IoT and A.x for new NPRACH test preambles |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No specified requirements for NPRACH demodulation performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections 8.6 and A.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | Y |  | Test specifications |  |  | TS 36.141 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----<< START OF THE CHANGE 1 >>-----------

## 8.6	Performance requirements for NPRACH

### 8.6.1	NPRACH False alarm probability

The false alarm requirement is valid for any number of receive antennas, for all frame structures and for any channel bandwidth.

The false alarm probability is the conditional total probability of erroneous detection of a preamble (i.e. erroneous detection from any detector) when the preamble is not present.

#### 8.6.1.1	Minimum requirement

The false alarm probability shall be less than or equal to 0.1%.

### 8.6.2	NPRACH detection requirements

The probability of detection is the conditional probability of correct detection of a preamble when the preamble is present. The following cases are counted as incorrect detection of a preamble when the preamble is present:

- Miss-Detection

  - not detecting any preamble at all

  - detecting a wrong  preamble other than the one that is sent

- Timing estimation error

  - timing estimation error occurs for a correctly detected preamble when the estimated timing is larger than [3.75]μs away from the strongest path.

#### The test NPRACH preambles are listed in Table A.x-1.

#### 8.6.2.1	Minimum requirements

With the SNR levels listed in table 8.6.2-1, the probability of miss-detection shall be less than or equal to 1% and the timing estimation error probability shall also be less than or equal to 1%.

Table 8.6.2.1-1 NPRACH missed detection requirements

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 |
| 1 | 2 | AWGN | 0 | [TBD] | [TBD] |
|  |  | EPA1 Low | 200 Hz | [TBD] | [TBD] |
|  | 4 | AWGN | 0 | [TBD] | [TBD] |
|  |  | EPA1 Low | 200 Hz | [TBD] | [TBD] |
|  | 8 | AWGN | 0 | [TBD] | [TBD] |
|  |  | EPA1 Low | 200 Hz | [TBD] | [TBD] |

-----<< END OF THE CHANGE 1>>-----------

## -----<< START  OF THE CHANGE 2>>-----------

## A.x	Test preambles for NPRACH

Table A.x -1 Test preambles for NPRACH

| NPRACH Burst format | Format 0 | Format 1 |
| --- | --- | --- |
| Number of repetition | 8 | 8 |
| Number of subcarriers | 12 | 12 |
| Cell-ID | 0 | 0 |
| Initial subcarrier index | 0 | 0 |

-----<< END OF THE CHANGE 2>>-----------