3GPP TSG RAN WG4 Meeting #80bis	R4-167876

Ljubljana, SI, 10 - 14 Oct 2016

Agenda Item:	5.7.4

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Impact of NB-IoT UL Timing Error

Document for:	Discussion

# Introduction

For NB-IoT, UE UL Tx timing errors is currently defined as [80Ts] in TS 36.133 [1]. At the time when the requirement was defined, it seems there was no enough consideration on the potential impact of the large UL Tx timing errors on NPUSCH performance. Since 80Ts equals to CP/2, UE UL Tx timing errors of [80Ts] seemed not necessarily causing NPUSCH timing error to be out of the CP window.

In RAN4#80, the potential impact of the large NPUSCH timing error due to the combination of UE UL Tx timing error, TA estimation error and other issues was discussed in [2], where it was pointed out that UE UL Tx timing errors of [80Ts] together with other errors may cause the NB-IoT UL Timing Error for NPUSCH received at eNB far beyond the CP window. As a result of the discussion, a WF was thus also agreed to further investigate impact of UL Timing Errors on NPUSCH Performance [3] though simulation with  NPUSCH UL Timing Error at BS side to be set within the range of [4.5μs], based on the consideration of UE UL Tx Timing Error and NPRACH TA timing estimation errors.

In this paper, we will further investigate the impact of the NB-IoT UL Timing Error on NPUSCH performance based on NPUSCH simulation results with  NPUSCH UL Timing Error at BS side to be set within the range of [4.5μs], as agreed in the WF [3].

# Simulation Assumptions

For the NPUSCH format 1, previously agreed NPUSCH simulation assumptions in [4] will be used. Since the simulation assumptions in [4] does not include the timing error, NPUSCH timing error in the range [4.5Ts]  was added into the simulation assumptions as agreed in the WF[3].

# Table 2-1: Simulation assumption for NPUSCH format 1

| Simulation number | BW | Singe-/Multi-tone | Sub-carrier spacing | repetition level | Number of tones | Modulation (Note 4) | RFC | Propagation Condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 2 | 180KHz | Single | 15KHz | 1 | 1 | BPSK | R.NB3-2 | AWGN | 1 x 2 | 70% | TBD |
| 3 | 180KHz | Multi-tone | 15KHz | 1 | 12 | BPSK | R.NB3-2 | AWGN | 1 x 2 | 70% | TBD |
| Note 1: Assume the frequency offset value is [0] and channel estimation lengths are 16ms for 3.75kHz and 4ms for 15kHz. For case 3 with repetition level 2, use 2ms channel estimation length. Note 2: Assume HARQ retransmission RV0 or RV2, refer to RAN1 spec TS 36.213 Note 3: Interesting companies are encouraged to provide the simulation results in the next RAN4 meeting for all the scenarios if they can. Note 4: see TS36.211 Clause 10.1 for the detailed modulation |  |  |  |  |  |  |  |  |  |  |  |

Table 2-2 FRC parameters for NPUSCH

| Reference channel | R.NB3-2 |
| --- | --- |
| Sub-carrier spacing (kHz) | 15 |
| Number of tone | 1 |
| Diversity | No |
| Modulation | BPSK |
| IMCS / IRU | 0 / 1 |
| Payload size (bits) | 32 |
| Allocated resource unit | 2 |
| Code rate (target) | 1/3 |
| Code rate (effective) | 0.29 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 96 |
| Total symbols per resource unit | 96 |
| Maximum number of transmissions | [4] |

# Simulation Results

Figure 1 shows the simulation results for the case with single tone, 15KHz, and 1 repetition .As shown in the Figure 1, the impact of the UL timing error within the range of [4.5μs] on the NPUSCH is insignificant. The impact is less than 0.2dB with UL timing error within the range of [15μs].

Figure 2 shows the simulation results for the case with 12- tones, 15KHz, and 1 repetition . As shown in the Figure 2, the impact of the UL timing error within the range of [4.5μs] on the NPUSCH can be very significant. The impact shows up after the UL timing error causes the NPUSCH receiving time out of the CP window, and then the impact increases rapidly after that.

Figure 1. Impact of UL timing error for NPUSCH Performance (single tone, 15KHz, repetition level (?))

Figure 2. Impact of UL timing error for NPUSCH Performance (12-tones, 15KHz, repetition level (?))

# Summary

In this paper, we further investigated the impact of the NB-IoT UL Timing Error on NPUSCH performance though NPUSCH simulation results as agreed in the WF [3]. From the investigation, we found that the impact of NB-IoT UL Timing Error on NPUSCH performance can be very significant for multi-tone NPUSCH transmissions, once the NB-IoT UL Timing Error causes the NPUSCH time of the arrival at eNB side exceeding the CP window. Therefore, we propose to carefully consider reducing the NB-IoT UL Timing Error to minimize the impact on the NPUSCH performance, including reducing the UE UL timing errors to be smaller than [80Ts].

# References

- 3GPP TS 36.133

- R4-165418, “NB-IoT UL Timing Error”, Nokia

- R4-166652, “WF on Simulation Assumptions on the Impact of UL Timing Errors on NPUSCH Performance”, Nokia

- R4-165359, “Updated simulation assumptions for NB-IoT BS demodulation performance requirements”, Huawei, Ericsson, Nokia, ZTE