3GPP TSG-RAN WG4 Meeting #80-bis	R4-167886

Ljubljana, SLOVENIA,   10 - 14 October, 2016

Source:	Hughes Network Systems LTD

Title:	A-MPR for 1.4 and 3 MHz Channel BWs in Band 65

Agenda item:	5.11.2	Other specifications (36.101)

Document for:	Approval

Abstract

This contribution builds on the WF R4-167885, the requirement for Band 65 A-MPR for Band 34 protection and how it can be implemented in the specification 3GPP TS 36.101

## Introduction

## At the 3GPP TSG RAN Meeting #72, Busan, Korea June 13 - 16, 2016 document RP-161204 was postponed and the meeting concluded that a definition of a new E-UTRA operating Band XY is required. [1]. Since the RAN Meeting #72 and as a result of numerous discussions with Chipset vendors and Telecom Operators, no work activity or developments for commercial BAND 65 devices have been requested or started. Also to date, BAND 65 has no installed device base available.

## 2		Discussion

Within the European Union (EU), the frequency band 1980-2010 MHz and 2170-2200 MHz (MSS Band) can be used for terrestrial mobile networks operated as a Complementary Ground Component (CGC) to Mobile Satellite Services (MSS) in the same frequency band. In the EU, the 2 GHz MSS bands are segmented into two MSS operators through Commission Decision 2009/449/EC [2]. One MSS system in 1980-1995 MHz and 2170-2185 MHz (Inmarsat as operator), and the other in 1995-2010 MHz and 2185-2200 MHz (EchoStar Mobile Limited as operator).

Each operator was given the EU-wide authority to offer a CGC as an integral part of their MSS network. MSS and terrestrial CGC (E-UTRA) services are meant to coexist in this band and the MSS operator actively manages the spectrum allocated to each service.

The spectrum band for each operator (15+15 MHz) is:

•	Inmarsat: 1980 – 1995 MHz Uplink, 2170 - 2185 MHz Downlink

•	EchoStar Mobile Limited (EML): 1995 – 2010 MHz Uplink, 2185 - 2220 MHz Downlink

This arrangement, and each MSS operator’s spectrum relative to Band 1, is shown in Figure 2.1

Figure 2.1:  Band 65

## Observations for Discussion

Observation 1.

B65 takes into account changes in the regulatory environment to the usage of the unpaired bands as per CEPT Report 52 and draft ECC/DEC/(15)AA. CEPT Report 52 concluded the usage of VLCC (Video link and Cordless Camera) for PMSE and ad-hoc PPDR in the frequency band 2010-2025 MHz.

- Therefore, since 3GPP E-UTRA Operating Band 34 (2010-2025 MHz) is not used within the EU, and it is doubtful 1.4 and 3 MHz will coexist with Band 34, then protection of Band 34 is not necessary to specify.

Observation 2.

Outside the EU and ASIA, where Band 34 is or may be used A-MPR and protection of Band 34 is required.

- 3GPP TS 36.101 Tables A-MPR for “NS_24” and “NS_25”, will need to be defined.

- A-MPR values for 1.4 and 3 MHz could be selected from the worst case for 5 MHz and modified at a later date using A-MPR versioning, once scenarios are more apparent.

- Proposal

We propose the following options, for Band 34 protection and how it can be implemented in the specification 3GPP TS 36.101

Option 1.

Adding a note and text to “Table 6.2.4-1: Additional Maximum Power Reduction (A-MPR)” that NS_24 and NS_25 and additional requirements are only applicable for 5 MHz or wider Channel Bandwidths.

Option 2.

Update Table 6.2.4-19 and Table 6.2.4-20 with 10 dB and 5 dB A-MPR respectively for 1.4 MHz and 3 MHz. No study needed.

Option 3.

Do an A-MPR study and update Table 6.2.4-19 and Table 6.2.4-20 with proper A-MPR values for 1.4 MHz and 3 MHz.

## References

- RP-161204, “New WID: Adding 1.4 and 3 MHz Channel Bandwidth to E-UTRA operating Band 65 for CGC (Complementary Ground Component) operations in Region 1”

- Commission Decision 2009/449/EC, on the selection of operators of pan-European systems providing mobile satellite services (MSS), 13 May 2009