3GPP TSG-WG4 Meeting #80bis	R4-168039

# Ljubljana, Slovenia, 10th – 14th October 2016

Agenda item: 	6.3.3.1

Source: 	Qualcomm Incorporated

Title: 	RSRP/RSRQ Measurement Accuracy Simulation Results

Document for:	Approval

# 1	Introduction

RAN4 is agreed [1] to further analyze NRSRP and NRSRQ measurement accuracy and define accuracy requirements. This section provides an overview of the agreed approach taken to simulate and specify requirements.

Minimum number of sub frames containing NRS per L1 measurement period are to be used by the UE to define NRSRP/NRSRQ measurement accuracy requirements.

Subset of the samples in the measurement period will be used for coherent averaging. Measurement accuracy requirements will be derived for maximum value obtained at the 5th and 95th percentiles of CDF.

RF margin of around 2-3 dB will be included when setting the requirement.

The following is the set of simulation assumptions for measurement accuracy study:

Table 1: Simulation parameters for NB-IOT measurement accuracy studies

| Parameters | Value | Comments |
| --- | --- | --- |
| Measurement bandwidth | 1 resource blocks | Both RSRP and RSRQ measured over 1 RB |
| System bandwidth | 1 resource blocks |  |
| L1 measurement period | 400 ms, 800 ms |  |
| Number of subframes containing NRS per L1 measurement period | 30 for 400ms L1 measurement period 60 for 800ms L1 measurement period |  |
| L3 filtering | Disabled |  |
| DRX | OFF |  |
| Antenna configuration | 1TX, 1RX |  |
| Mobility | Stationary UEs, |  |
| Channel model | AWGN, ETU 5Hz, EPA 5Hz | Models stationary device |
| Measurement type | NB-RS only based |  |
| CP length | Normal |  |
| Carrier frequency | 2 GHz |  |
| Ec/Iot | -15dB, …, 10 dB | AWGN noise |
| Frequency error modelling | +/-50 Hz | With respect to reference cell. Companies are encouraged to provide results also for +/- 100Hz |

The simulation assumed that only SF#0 and SF#4 in each frame contain NRS. Three subframe coherent averaging is used which implies that the NRS tones of the 6 slots in SF#0, SF#4 and SF#10 are coherently averaged once every 40ms. The resulting NRSRP/NRSRQ result every 40ms is non-coherently averaged over the measurement period. Measurement accuracy results shown below are reported at the 95th-%ile point on the CDF.

# 2	AWGN Simulation Results

## 2.1	NRSRP Measurement Accuracy

The following table shows NRSRP Measurement Accuracy at 95-%tile point for AWGN:

|  | Meas Period = 400ms | Meas Period = 800ms |
| --- | --- | --- |
| SNR=10dB | 3.3 dB | 4 dB |
| SNR=-6dB | 6.2 dB | 5.9 dB |
| SNR=-15dB | 12.3 dB | 12.3dB |

# 3	ETU5Hz Simulation Results

## 3.1	NRSRP Measurement Accuracy

The following table shows NRSRP Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms |
| --- | --- | --- |
| SNR=10dB | 5.5 dB | 4 dB |
| SNR=-6dB | 7.3 dB | 5.6 dB |
| SNR=-15dB | 12.6 dB | 12 dB |

# 4	EPA5Hz Simulation Results

## 4.1	NRSRP Measurement Accuracy

The following table shows NRSRP Measurement Accuracy at 95-%tile point:

|  | Meas Period = 400ms | Meas Period = 800ms |
| --- | --- | --- |
| SNR=10dB | 5.8 dB | 4 dB |
| SNR=-6dB | 7.2 dB | 6 dB |
| SNR=-15dB | 12.7 dB | 12 dB |

Since NRSSI accuracy is expected to be very good, NRSRP and NRSRQ accuracy requirements can be the same.

# 5	Conclusions

This paper contains simulation results for NRSRP and NRSRQ measurement accuracy for the purpose of alignment and subsequent requirement specification.

Proposal 1: NRSRP/NRSRQ Measurement accuracy for normal coverage can be set to 6dB.

Proposal 2: NRSRP/NRSRQ Measurement accuracy for enhanced coverage can be set to 12dB.

Proposal 3: Measurement period should be set as 800ms for both normal and enhanced coverage modes.

# 6	References

- R4-79AH-0274, “Wayforward on NB-IoT RRM measurement accuracy simulation assumption”, Huawei, HiSilicon