3GPP TSG-RAN WG4 Meeting #80bis 	R4-168308

# Ljubljana, Slovenia, 10-14 Oct 2016

Agenda item:	8.27.3

Source: 	Intel Corporation

Title: 	On RRM of 1Rx UE based on Cat.1

Document for:	Discussion

# Introduction

RAN Plenary #73 has approved a new Rel-14 WI [1] of ‘a new UE category with single receiver based on Category 1 for LTE’, the first discussion gets started from RAN4#80bis. In this contribution the possible impact on current RRM requirements are provided to trigger the discussion.

# Discussion Scope of 1-RX UE RRM

In [1] the scope of RRM part was proposed as below, including both core requirements and performance requirements.

- Core requirement

- Specify RRM core requirements for Category 1 UE with a single receiver for intra-frequency and inter-frequency mobility [RAN4]

  - RLM and intra-frequency mobility requirements (measurements) for Category 0 UE could be reused

- Performance requirement

- Measurement accuracy requirements for intra and inter-frequency measurements

  - Intra-frequency requirements defined for Cat.0 can be reused

- Define test cases to verify the requirements

It was realized that some of the RRM requirement of Cat0 UE could be reused for Cat1 single receiver UE, since Cat0 UE has single antenna receiver as well. In this contribution, the further analysis of impact on RRM from Cat1 single receiver is provided. To be in line with the scope section in [1], we also divide them into core requirement part and performance requirement part.

- Core requirement

The impact on RRM core requirements are summarized in the following table.

| Core requirement for Cat1 single receiver UE | Shall be revisited or not | Priority | Comments |
| --- | --- | --- | --- |
| Cell selection | No | High | It can be same as the current requirement |
| Intra-freq Cell reselection | Yes | High |  |
| Inter-freq Cell reselection | Yes | High |  |
| Inter-RAT Cell reselection | Yes | Low |  |
| FDD-FDD HO | Yes | High |  |
| TDD-TDD HO | Yes | High |  |
| TDD-FDD/FDD-TDD HO | Yes | Medium | It can reuse the requirement of FDD-FDD/TDD-TDD HO requirement |
| Inter-RAT HO | Yes | Low |  |
| RRC Re-establishment | Yes | High | The time of searching target PCell and the SI reading time might be changed. |
| RRC Connection Release with Redirection to other RAT | Yes | Low |  |
| RLM | Yes | High | It can reuse the requirement for Cat0 UE |
| Intra-freq cell identification | Yes | High | It can reuse the requirement for Cat0 UE |
| Inter-freq cell identification | Yes | High |  |
| Intra-freq RSRP/RSRQ measurement | Yes | High | It can reuse the requirement for Cat0 UE |
| Inter-freq RSRP/RSRQ measurement | Yes | High |  |
| Inter-RAT cell identification | Yes | Low |  |
| Inter-RAT measurement | Yes | Low |  |
| Rx-Tx time difference measurement | Yes | Medium |  |
| Intra/inter-freq RSTD measurement | Yes | Medium |  |
| Intra/inter-freq/inter-RAT measurements with autonomous gaps | Yes | Low | This requirement is designed for CSG cell |

- Performance requirement

The impact on RRM performance requirements are summarized in the following table.

| Performance requirement for Cat1 single receiver UE | Shall be revisited or not | Priority | Comments |
| --- | --- | --- | --- |
| Intra-freq RSRP/RSRQ measurement accuracy | Yes | High | It can reuse the requirement for Cat0 UE |
| Inter-freq RSRP/RSRQ measurement accuacy | Yes | High |  |
| Inter-RAT measurement accuracy | Yes | Low |  |
| Rx-Tx time difference accuracy | Yes | Medium |  |
| Intra/inter-freq RSTD accuracy | Yes | Medium |  |
| Intra/Inter-freq RS-SINR | Yes | Low | The side condition is still open now |
| SSTD Accuracy | Yes | Low | Applied for DC feature |

# Conclusions

In this contribution, we analyzed the possible impact on RRM after introducing 1Rx Cat1 UE to trigger the discussion in RAN4.

# Reference

[1] RP-161879, “Requirements for a new UE category with single receiver based on Category 1 for LTE”, QC, 3GPP TSG-RAN Meeting #73, New Orleans, USA, September 19th -22nd, 2016