3GPP TSG-RAN WG4 Meeting #80bis	      		R4-168509

Ljubljana, Slovenia, 10 – 14 October, 2016

Source: 	Sony

Title:	UE cat M1 out of band blocking

Agenda Item:	5.5.1

Document for:	Approval

# Introduction

The out of band blocking for UE category M1 is specified in Table 7.6.2.1-2 in TS 36.101 [1]. During the work with. UE category NB1 the relevance of out of band blocking Range 4 (Table 7.6.2.1-2 in TS 36.101) was put in question and was finally removed [2] (Table 7.6.2.1F-1 in TS36.101).

# Discussion

UE category M1 is similar to UE category NB1 in that both categories specify half duplex (HD) operation and both categories target low cost UEs. HD operation makes it possible to design the receiver with less stringent filters. However the current specification of Range 4 in Table 7.6.2.1-2 in TS 36.101 still implies stringent filtering for bands 2, 5, 12 and 17, similar as needed for full duplex operation. Moreover, during the specification work for UE category NB1 the stringent requirements for Range 4 was regarded obsolete [2].

Due to the large similarities between categories M1 and NB1 it is recommended to remove Range 4 also for UE category M1.

Proposal: Range 4 requirement for out of band blocking for cat M1 to be omitted.

# Conclusion

In this contribution we discussed the relevance of Out of band blocking requirements for Range 4 for UE category M1 and come to the following recommendation:

Proposal: Range 4 requirement for out of band blocking for cat M1 to be omitted.

# References

TS 36.101

R4-78AH-0231, 	“WF for NB-IoT REFSENS, repetitions and OOB”, Nokia, Vodafone, Sony, Telecom Italia, Intel, NTT Docomo, Mediatek, Skyworks, Huawei, Qualcomm, CMCC,