3GPP TSG-RAN WG4 #80bis	R4-168546

Ljubljana, Slovenia, 10– 14 October 2016

Source: 	Ericsson

Title:  	Details on multi-node outage tests for Rel-13 LAA

Agenda Item:	7.1.2

Document for:	Discussion

# Introduction

During RAN plenary in June 2016, a new study item has been approved in [1]. Previously, three way forwards [2],[3],[4]have been agreed on coexistence tests for Rel-13 LAA in RAN4#78bis, RAN4#79 and RAN4#80, respectively.

The coexistence tests are divided in two parts: LBT functionalities tests and multi-node tests. LBT functionalities are captured in TS36.141 in RAN4#79 meeting and have been updated later. The multi-node tests will be captured in TR 36.789.

In this contribution, we provide more detailed understanding on multi-node outage tests.

# Multi-node outage tests

In R4-162989, RAN4 has agreed to have a maximum of two priority classes. It is worth mentioning here that, from spec point of view, different priority classes are optional. So, it’s not mandatory to implement all priority classes, thus, an LAA node may only implement one priority class (as an example). Thus, it is understood that:

Observation: Tests should only be performed for channel priority class capabilities that are supported (declared) by the LAA node.

As a result, if an LAA node is capable of performing only one channel priority class, then the LAA BS will be tested for only one priority class. Similarly, maximum two priorities apply for a node that can do anything more than one class.

The latest agreements are described in [4]. As we describe in our companion contribution, we use the following for outage tests:

| Receive signal levels | Above -72dBm/20MHz and below -82dBm/20MHz |
| --- | --- |
| Choices for IEEE 802.11 devices | N commercial IEEE 802.11 AP and M commercial IEEE 802.11 STA devices from different vendors should be used. For IEEE 802.11 APs and STAs, the devices should come from multi-vendor and be multi-standard (i.e. multi-generation IEEE 802.11 systems). N can be 3 and M can be 2. |
| Test setup | Same as throughput test setup |

### Test cases

In addition to previous test cases, another set of tests can be done for LAA BS that also implements priority class 1. The test case can be described as follows:

|  | Transmitter | Receiver | Traffic | Criterion |
| --- | --- | --- | --- | --- |
| Case -1 | IEEE 802.11 | IEEE 802.11 | VoIP traffic in all nodes | The criterion should be investigated in more detail. One possibility could be: outage should not be worse in second system receiver when the transmitter is replaced |
| Case- 2 | LAA | IEEE 802.11 | VoIP traffic in all nodes |  |

### Metric

For channel priority class 1, i.e. for VoIP traffic, the metric will be the outage that the victim devices experience. The outage can be defined as VoIP client behavior, such as when does the VoIP client drop the packets and how many percentage of the packets are dropped within a certain period.

However, more understanding on how to test this behavior should be discussed in RAN4.

### Load

In our opinion, best effort traffic model is generally more suitable for the tests. One underlying principle can be that, same level of traffic need to be considered for both cases. This means that, the traffic that is assumed for WiFi-WiFi in case 3 should also be used in LAA-WiFi case in case 3. In this way, the performance comparison will be more reasonable.

It remains to be understood whether or not we should consider any background traffic in addition to the VoIP traffic while performing the tests.

# Conclusion

In this paper, we have summarized our current understanding and questions on outage tests related to multi-node tests for Rel-13 LAA. We propose RAN4 companies to consider the above the discussions while finalizing the multi-node outage tests.

# References

RP-161197, New Study Item proposal: Multi-node tests for LAA, Huawei, HiSilicon, Ericsson, Qualcomm, Nokia

R4-162989, “Way Forward on LAA coexistence tests”, Ericsson, Huawei, Qualcomm, Nokia, RAN4 #78bis

R4-164971, WF on LAA co-existence testing, Source: Qualcomm, Ericsson, Huawei, Nokia, RAN4#79.

R4-166971, WF on LAA Multi-node test, Huawei, Ericsson, Qualcomm, Nokia

R4-166542, Details on multi-node tests for Rel-13 LAA, Source: Ericsson

R4-168549, TP on Throughput tests for TR36.789: Multi-node tests for LAA, Ericsson

R4-168545, “Further details on multi-node throughput tests for Rel-13 LAA”, Ericsson