TSG-RAN Working Group 4 (Radio) meeting #80bis	R4-168559

Ljubljana, Slovenia, 10th – 14th October 2016

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	OTA EVM of AAS base station transmitters

Agenda item:	8.12.2.1 [AASenh_BS_LTE_UTRA]

Document for:	Approval

## Introduction

Presently, the conducted EVM requirements of AAS base station transmitters are reused from the current requirements for EVM as specified in TS 36.104 and TS 25.104 for UTRA and E-UTRA, respectively. Reference [3] summarizes the decisions that have been reached for OTA AAS base station transmitters. Non-ideal analogue devices used in transmitters introduce phase and amplitude errors that subsequently lead to EVM. One major source of phase errors is phase noise of local oscillators. At the 3GPP TSG-RAN WG4 meeting#80 in Gothenburg, Sweden , reference [1] provided an insight into the effect of correlated phase-noise sources on OTA EVM through mathematical analyses. This document further continues the discussion and present simulation results. Finally, we present our observations based on simulation results and mathematical analyses.

## Discussion

In conducted testing of AAS transmitters, the EVM requirement is specified per TAB connector. As shown in Figure 1, the signal from one TAB connector is fed to a signal processing chain. The output signal of the per-subcarrier amplitude/phase correction block is used to calculate the EVM. Table 1 presents the minimum conducted EVM requirements as specified in TS 36.104. Unlike the conducted EVM, the OTA EVM exhibit spatial variations because beams are steered towards desired user devices. Reference [3] states that OTA EVM requirements for user-specific beams should be specified spatially according to declared beams within the EIRP accuracy. A question arises as to whether the declared EIRP beams correspond to good EVM in all test scenarios. There could exist test scenarios in which beams with the best EIRP but degraded EVM. Thus, the declared EIRP beams should be supplemented with other performance metrics, e.g., antenna-array radiation patterns.

While it is generally sufficient to consider the beamsteered signal in the main lobe for the EVM measurement, it is not known how the signal impairments (e.g., due to phase noise of local oscillators) affect the OTA EVM tested at such a center position. Let us consider an AAS base station with n transceiver units. The received signal at a given time instance is a superposition of the signals radiated by all antenna-array elements is [4]

s= k=1nakej(θk+φk)																								(1)

where θk = the phase shifts due to a carrier frequency

ak= the amplitude

φk = the phase noise

The error vector ∆s is obtained by subtracting the ideal signal from equation (1)

∆s= k=1nak ej(θk+φk)- k=1nak ejθk

= k=1nak ejθk(ejφk- 1)																				(2)

We can calculate its root mean square value as

EVMRMS2= E[|∆s|2]|sideal|2= var∆s+ |E[∆s]|2|sideal|2 																				(3)

where  var  =  variance

E[.] = the mean value

AAS Base Station under TX test

1

TRXU 1

2

OTA EVM measurement

TRXU 2

.

.

.

Antenna Array

Radio Distribution Network

.

.

.

TRXU NTRXU

NTABC

TRXU Array

CP Removal

FFT

Per-subcarrier Amplitude/Phase correction

Conducted EVM measurement

Symbol detection

Figure 1: Conducted and OTA AAS BS EVM measurement

| Modulation scheme for PDSCH | Required EVM [%] |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |

Table 1: Minimum Conducted EVM requirements for E-UTRA carriers (Table 6.5.2.-1, TS 36.104)

In order to analyze the impact of phase noise on EVM, we consider three cases, namely fully correlated, partially correlated and uncorrelated phase noise.

Case 1: Fully correlated phase noise

In case the phase noise is fully correlated in the different transmitters, φk= φ for all k. Now, considering only the signal in the center of the main lobe, that is when the phases of all antenna-array elements align in a point of constructive interference, i.e., when θk=0 for all k, equation (2) simplifies to

∆scor= (ejφ- 1) k=1nak 																					(4)

Let x= ejφ- 1 and using equation (3), the RMS EVM value is given by (see [2] for a complete derivation)

EVMcor,RMS2= σx2+ |μx|2 																						(5)

where σx = standard deviation of x

μx = mean of x

Case 2: Fully uncorrelated phase noise

In case of uncorrelated phase noise, equation (4) becomes

∆suncor= k=1nak (ejφk-1) 																				(6)

Let xk= ejφk- 1 and using equation (3), the RMS EVM is

EVMuncor,RMS2= σx2σa2na2+ σx2n+ |μx|2 																		(7)

where σx = standard deviation of xk

μx = mean of xk

a  = the average of the amplitude coefficients ak

σa = standard deviation of ak

In the special case where all the amplitudes are the same, i.e., when ak=a for all k and thus σa=0, equation (7) reduces to

EVMuncor,RMS2= σx2n+ |μx|2 																					(8)

Case 3: Partially correlated phase noise

This case occurs when one local oscillator may serve more than one transceiver but not all. The effect is that there will be groups of transceivers which have correlated phase noise. In addition, there will be multiple ways to connect the group of local oscillators to the transceivers. Following the same derivation principles as in Case 2, the RMS EVM can expressed as (see [2] for more details)

EVMptcor,RMS2= σx2m+ |μx|2 																					(9)

where  m = the number of phase noise source <n.

As the phase noise is relatively small, i.e., μx ≈ 0, and n ≫1, we can relate equations (5), (8) and (9) as follows

EVMuncor,RMS2 ≤ EVMptcor,RMS2 ≤ EVMcor,RMS2 															(10)

From equation (10), we can observe that a high correlation of phase noise sources will result in bad EVM. Therefore, the OTA requirement of AAS base station transmitters should capture the worst-case scenario; however, the present of phase noise in an AAS base station is implementation dependent.

Figure 2: EVM of a beam directed at 40° for fully, partially correlated and uncorrelated phase noise

Figure 2 shows simulation results of RMS EVM as a function of a directed beam at 40° for a number of phase noise configurations as shown in Table 2. As shown in Figure 2, the RMS EVM of fully correlated phase noise is the worst and then followed by the partial correlated ones. As mentioned, the present phase noise in an AAS base station is implementation dependent, we can make the following observation:

Observation 1: for user-specific beams, the OTA EVM requirement should not only include the EVM in the main lobe only;

Observation 2:  non-user-specific beams (cell-wide beams) should be taken into account as well. Intuitively, the EVM for cell-wide beams is worse than the EVM of user-specific beams.

| Configuration |  | m | n |
| --- | --- | --- | --- |
| 1111 | Fully correlated | 1 | 4 |
| 1234 | Uncorrelated | 4 |  |
| 1221 | Partially correlated | 2 |  |
| 1122 | Partially correlated | 2 |  |
| 1212 | Partially correlated | 2 |  |

Table 2: Configurations of phase noise sources

## Conclusions

This document has examined the impact of phase noise on OTA EVM of the user-specific beamformed signal. The OTA EVM is worse if phase-noise sources are correlated; however, the degree of phase-noise correlation depends on AAS base station implementation. Thus, we make the following observations:

Observation 1: for user-specific beams, the OTA EVM requirement should not include only the EVM in the main lobe only as the fully correlated phase noise is an exceptional case;

Observation 2:  non-user-specific beams (cell-wide beams) should serve as a base line for the OTA EVM requirement of the user-specific beams. Intuitively, the EVM for cell-wide beams is worse than the EVM of user-specific beams.

## References

[1]   Nokia, OTA EVM of AAS base station transmitters, R4-166194, RAN4#80

[2]  T. Höhne, and V. Ranki, Phase Noise in Beamforming, IEEE Transactions on Wireless Communications, vol. 9, no. 12, December 2010

[3]   R4-165783, Measuring unwanted emission total radiated power, Ericsson