3GPP TSG RAN4 Meeting #80bis	R4-168860

Ljubljana, Slovenia, 22 – 26 August, 2016

Title: 	TP for TR36.714-00-02 on MSD for 3+7+8 3DL 2UL CA

Source: 	Huawei, HiSilicon, LG Electronics, CHTTL

Agenda item:	8.7.2

Document for:	Other (Approval)

# 1	Introduction

This contribution discusses MSD requirements for 3+7+8 3DL 2 UL CA.

# 2	Discussion

This contribution is much based on our earlier one [2] with some corrections. This CA combination has several intermodulation products falling on top of own RX, see table below. Please note that this combination is the very first example when two intermodulations, and in addition, third harmonic fall on top of each other.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B8 | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |
|  | B3 | IMD2 | fB3 + fB8 | 1735 | 5 | 25 | B7 | 2635.0 | 10 |
|  | B8 |  |  | 900 | 5 | 25 |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB8 | 1772.5 | 5 | 25 | B7 | 2635.0 | 10 |
|  | B8 |  |  | 910 | 5 | 25 |  |  |  |
|  | B3 | IMD2+IMD3+H3 | fB3 + fB8, 2*fB3 - fB8 | 1780 | 5 | 25 | B7 | 2670 | 10 |
|  | B8 |  |  | 890 | 5 | 25 |  |  |  |

Table 1 Example test points

In addition this combination has also a scenario where IMD2 and IMD3 from B3+B8 both hit on top of B7 but they do not overlap. The example below depicted one possible scenario. Note that the UL RB allocations in the example scenario are wider than in respective single band RFESENS.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3+7+8 | B3 | IMD2+IMD3 | fB3 + fB8, 2*fB3 - fB8 | 1762.5 | 15 | 75 | B7 | 2650 | 20 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |

We use the following RF front-end architecture in our analysis. Another alternative would be common triplexer based architecture. There is harmonic trap assumed in B8 path for this CA case because harmonic trap for was assumed for B8+B7 1UL/2DL CA and for B3+B7+B8 1UL/3DL CA.

Figure 1 Common diplexer -architecture

We analyze the amount of IMD using the following component linearity assumptions:

| Component | IP2 (dBm) | IP3 (dBm) |
| --- | --- | --- |
| Antenna switch | 112 | 68 |
| Diplexer | 115 | 86 |
| Quadplexer | 113 | 74 |
| PA forward mixing | 31 | 31.5 |
| PA reverse mixing | 38 | 28 |
| LNA | 5 | -5 |

Table 2 General linearity parameters

B8 duplexer TX rejection at B7		30

Quadplexer B8 TX rejection    		45

Quadplexer B3 TX rejection			45

Antenna isolation 					10

PCB isolation PA-PA 				60

PCB isolation PAout-RX				65

RFIC Pout B3/B8	/B7					0

Quadplexer X-band Iso B3->B7		50

Diplexer isolation						15

H trap attenuation						15

PA gain								28

Based on these parameters we get the following MSD. Please note that the CF is the same as in [1]. For the case where B3+B8 IMD2, IMD3, and B8 third harmonic fall on top of each other at B7 DL the impact of the third harmonic is negligible because it is more than 20dB below the MSD due to IMD2 and IMD3. Thus “H3” is not included in the MSD table.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B8 | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 | 0.62 | 21.6 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |  |  |
|  | B3 | IMD2 | fB3 + fB8 | 1735 | 5 | 25 | B7 | 2635.0 | 10 | 0 | 32.0 |
|  | B8 |  |  | 900 | 5 | 25 |  |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB8 | 1772.5 | 5 | 25 | B7 | 2635.0 | 10 | 0 | 22.1 |
|  | B8 |  |  | 910 | 5 | 25 |  |  |  |  |  |
|  | B3 | IMD2+IMD3 | fB3 + fB8, 2*fB3 - fB8 | 1780 | 5 | 25 | B7 | 2670 | 10 | 0 | 32.5 |
|  | B8 |  |  | 890 | 5 | 25 |  |  |  |  |  |

We did also calculate IMD values for the case when IMD2 and IMD3 hit on top of B7 DL but do not overlap. The results are as follows. Please note these IMD’s don’t include CF’s that will in this case be fairly significant. This is an example calculation with UL RB allocations wider than in respective single band RFESENS.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | IMD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3+7+8 | B3 | IMD2 | fB3 + fB8 | 1762.5 | 15 | 75 | B7 | 2650.0 | 20 | 29.0 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB8 | 1762.5 | 15 | 75 | B7 | 2650.0 | 20 | 19.2 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |  |
|  | B3 | IMD2+IMD3 | fB3 + fB8, 2*fB3 - fB8 | 1762.5 | 15 | 75 | B7 | 2650 | 20 | 29.5 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |  |

Then finally we consider which of the possible test points shall be in standard. MSD for B8 is automatically standardized as it is the only one for B8. For B7 there are three possible testpoints; IMD2, IMD3 and case where these two overlap together with H3. According to WF the treatment of the case where IMD’s overlap is FFS. For this band combination, we proposed to verify the MSD for B7 only in case where IMD2, IMD3.

| CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B8 | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 | 0.62 | 21.6 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |  |  |
|  | B3 | IMD2+IMD3 | fB3 + fB8, 2*fB3 - fB8 | 1780 | 5 | 25 | B7 | 2670 | 10 | 0 | 32.5 |
|  | B8 |  |  | 890 | 5 | 25 |  |  |  |  |  |

# 3	Collection of MSD results for 3DL/2UL CA UE

Table 3 shows the merged MSD results from interested companies.

Table 3 MSD results for 3DL/2UL CA UE with self-interference issues

| DL CA | UL CA | IMD |  | Qualcomm | LGE | Huawei | Avg. MSD [dB] | Proposed MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B8 | B3 | IMD3 | 2*fB3 - fB7 | TBD | 14.1 | 21.6 | TBD | TBD |
|  | B7 |  |  |  |  |  |  |  |
|  | B3 | IMD2+IMD3 | fB3 + fB8, 2*fB3 - fB8 | TBD | 25.5 | 32.5 | TBD | TBD |
|  | B8 |  |  |  |  |  |  |  |

# Text proposal for TR36.714-00-02

### 5.1.5	Summary of interference studies

Table 5.1.5-1 summarizes the CA band combinations with self-interference problems for CA_3DL/2UL CA.

Table 5.1.5-1: Summary of Self-interference analysis for 2UL/3DL CA

| downlink CA configuration | uplink CA Configuration | Harmonic relation to 3rd band without uplink | intermodulation to 3rd band without uplink | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| CA_1A-3A-40A | CA_1A-3A | - | 5th order | Yes | 8.0 Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
| CA_1A-5A-40A | CA_1A-5A | - | 4th order | - | 9.0 |
| CA_1A-3A-7A | CA_1A-3A | - | - | Yes | N/A |
|  | CA_1A-7A | - | - |  | Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
|  | CA_3A-7A | - | - |  | Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
| CA_1A-7A-8A | CA_1A-7A | - | 5th order | - | No Need The IMD5 is fairly small and the overlapped region is quite small portion |
|  | CA_1A-8A | B8 3rd harmonic |  | - | Table 7.3.1A-0a for CA_1A-7A-8A [TS36.101] |
| CA_1A-8A-40A | CA_1A-8A | - | - | - | N/A |
| CA_1A-42C | CA_1A-42A | - | 4th order | - | No impact when consider a fixed Tx-Rx separation of 190MHz in Band 1 own Rx band. |
| CA_2A-4A-5A | CA_2A-4A | - | - | - | N/A |
| CA_2A-4A-29A | CA_2A-4A |  |  |  | N/A |
| CA_2A-12B | CA_2A-12A | - | - | - | N/A |
| CA_2A-12A-30A | CA_2A-12A | - | 4th order | - | 12.0 |
| CA_3A-5A-40A | CA_3A-5A | - | - | - | N/A |
| CA_3A-7A-8A | CA_3A-7A |  | 3rd order | - | TBD |
|  | CA_3A-8A | B8 3rd harmonic | 2nd & 3rd order | - | Table 7.3.1A-0a for CA_3A-7A-8A [TS36.101] TBD |
| CA_3A-8A-40A | CA_3A-8A | - | - | - | N/A |
| CA_3A-42C | CA_3A-42A | - | - | - | Studied in 2UL/2DL CA in Rel-14. N/A |
| CA_4A-12B | CA_4A-12A | - | - | - | N/A |
| CA_4A-12A-30A | CA_4A-12A | - | - | - | N/A |
| CA_7C-28A | CA_7C | - | - | - | N/A |
|  | CA_7A-28A | - | - | - | N/A |
| CA_19A-42C | CA_19A-42A | - | 4th order | - | Studied in 2UL/2DL CA in Rel-14. Consider specific spectrum of the operator N/A |
| CA_21A-42C | CA_21A-42A | - | - | - | N/A |

Table 5.1.5-2 summarizes the CA_ band combinations with self-interference problems for CA_4DL/2UL CA.

Table 5.1.5-2: Summary of Self-interference analysis for 2UL/4DL CA

| downlink CA configuration | uplink CA Configuration | Harmonic relation to 3rd and 4th band without uplink | intermodulation to 3rd and 4th band without uplink | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| CA_1A-3A-8A-40A | CA_1A-3A | - | 5th order into B40 | Yes | No need (already covered 3DL CA_1A-3A-40A with 2UL CA_1A-3A) |
|  | CA_1A-8A | 2nd harmonic into B3 | - |  | Table 7.3.1A-0a for CA_1A-3A-8A-40A [TS36.101] Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
|  | CA_3A-8A | - | - |  | N/A |
| CA_1A-3A-5A-40A | CA_1A-3A | - | 5th order into B40 | Yes | No need (already covered 3DL CA_1A-3A-40A with 2UL CA_1A-3A) |
|  | CA_1A-5A | - | 4th order into B40 |  | No need (already covered 3DL CA_1A-5A-40A with 2UL CA_1A-5A) |
|  | CA_3A-5A | - | - |  | Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
| CA_1A-3A-7A-8A | CA_1A-3A | - | - | Yes | N/A |
|  | CA_1A-7A | - | 5th order into B8 |  | No need (already covered 3DL CA_1A-7A-8A with 2UL CA_1A-7A) |
|  | CA_1A-8A | 2nd harmonic into B3 & | - |  | Table 7.3.1A-0a for CA_1A-3A-7A-8A [TS36.101] Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
|  |  | 3rd harmonic into B7 | - |  | Table 7.3.1A-0a for CA_1A-3A-7A-8A [TS36.101] |
|  | CA_3A-7A |  | 3rd order into B8 |  | No need (already covered 3DL CA_3A-7A-8A with 2UL CA_3A-7A) |
|  | CA_3A-8A | 3rd harmonic into B7 | 2nd & 3rd order into B7 |  | Table 7.3.1A-0a for CA_1A-3A-7A-8A [TS36.101] No need (already covered 3DL CA_3A-7A-8A with 2UL CA_3A-8A) |
| CA_2A-4A-5A-29A | CA_2A-4A | - | - | - | N/A |
| CA_3A-7C-28A | CA_3A-7A | - | 2nd order into B28 | - | No need (already covered 3DL CA_3A-7A-28A with 2UL CA_3A-7A) |
|  | CA_7A-28A | - | 2nd order into B3 | - | No need (already covered 3DL CA_3A-7A-28A with 2UL CA_7A-28A) |
|  | CA_7C | - | - | - | N/A |

Based on the MSD test configuration as summarized in Table 5.1.5.1 & 5.1.5.2, it is proposed to define the following REFSENS exceptions and test configurations in the core specifications for 3DL/2UL inter-band CA with IMD problem as shown in Table 5.1.5.3.

Table 5.1.5.3: Proposed 3DL/2UL inter-band CA REFSENS exceptions and test configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | 3rd DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode |
| CA_1A-3A-40A | CA_1A-3A | 1 | 1950 | 5 | 25 | 2140 | 5 | N/A | TDD-FDD |
|  |  | 3 | 1735 | 5 | 25 | 1830 | 5 |  |  |
|  |  | 40 |  |  |  | 2380 | 5 | [8.0] |  |
| CA_1A-5A-40A | CA_1A-5A | 1 | 1977.5 | 5 | 25 | 2167.5 | 5 | N/A | TDD-FDD |
|  |  | 5 | 826.5 | 5 | 25 | 871.5 | 5 |  |  |
|  |  | 40 |  |  |  | 2305 | 10 | 9.0 |  |
| CA_2A-12A-30A | CA_2A-12A | 2 | 1885 | 5 | 25 | 1965 | 5 | N/A | FDD |
|  |  | 12 | 708.5 | 5 | 25 | 738.5 | 5 |  |  |
|  |  | 30 | 2308 | 5 | 25 | 2353 | 5 | 12.0 |  |
| CA_3A-7A-8A | CA_3A-7A | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD |
|  |  | 7 | 2530 | 10 | 50 | 2650 | 10 |  |  |
|  |  | 8 | 895 | 5 | 25 | 940 | 5 | TBD |  |
|  | CA_3A-8A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 |  |  |
|  |  | 7 | 2550 | 10 | 50 | 2670 | 10 | TBD |  |

The required MSD levels are derived by taking average values of proposed MSD levels from interested companies. If there are outliers, the sources for differences will be discussed and applicability for averaging considered case by case.