3GPP TSG-RAN WG4 Meeting #81 	R4-1609045

Reno, NV, US, 14 – 18 November, 2016

Source:	SoftBank Corp.

Title:	TP for TR36.714-02-01: MSD study of LTE-A Inter-band Carrier Aggregation (2DL/1UL) of Band 11 and Band 28

Agenda item:	8.2.2

Document for:	Approval

1.  Introduction

2DL/1UL of 11A-28A was firstly approved in [1] and is subject to the 2nd harmonic (H2) fall-down from B28 Tx to B11 Rx. Other than MSD, TP [2] was already approved. This paper proposes MSD for the completion of this combination. TP is also proposed to reflect the discussion.

This paper also includes some modifications to the approved TP [1].

2.  Architectural assumptions

For B11+B28, a quadplexer is not needed to due to its large frequency separation and cascaded diplexer and duplexer is adequate. For this diplexer, we can assume either extended High-Low common diplexer (covering B11 as high) or dedicated one, as long as the proposed values are taken for ∆TIB and ∆RIB in [2] which can accommodate both architectures, i.e. ∆TIB of B11 is set to 0.4dB.

3.  Estimation of MSD

In general, so far, the level of H2 in PA is assumed around 35dBc and the rejection of common diplexer is set around 15dB. For rejection performance of H2 in Band 28 duplexers, current B28B products can offer 32-35dB in the minimum values at H2[3-5]. In addition, in [2], it was assumed to insert HTF in B28 path to suppress H2.

With those, the following is the estimation of harmonic fall down. While this table does not include IP2 effects of RF components other than PA since, judging from studies so far, it does not seem to give significant impacts to the final MSD values.

Table 1:  2nd harmonic effect estimation

|  | Attenuation @ H2 | Power[dBm] | Note |
| --- | --- | --- | --- |
| PA out (H2) |  | -8 (= 27dBm – 35dB) | H2: 35dBc |
| Conductive path H2 power level in Tx |  |  |  |
| Duplexer out | 32dB | -40 | Worst value of [3-5] |
| HTF out | 30dB | -70 |  |
| (Common) diplexer out | 15dB | -85 |  |
| Rx H2 power level (Main Path) |  |  |  |
| PCB Coupling Converted to @antenna Port | 70dB 4dB IL | -78 -74 | PA out(H2) – (70dB) |
| Tx Conductive Path contribution @ antenna port |  | -85 |  |
| Composite @ antenna port |  | -73.7 |  |
| Rx H2 power level (Diversity Path) |  |  |  |
| PCB Coupling Converted to Antenna Port | 70dB 4dB IL | -78 -74 |  |
| Tx Conductive Path contribution @ antenna port | 10dB (Coupling loss) | -95 |  |
| Composite @ antenna port |  | -74 |  |

From the table, it can be seen that (1) PCB coupling is the dominant factor since HTF suppresses H2 through conductive path and (2) at antenna ports, around -74dBm interference will be injected to Band 11 Rx due to H2. This could give following desense, assuming -73.7dBm H2 power falling down into both main and diversity paths for simplicity:

Table 2: MSD estimation

| B11 receive BW | 5MHz | 10MHz | Note |
| --- | --- | --- | --- |
| B28 Tx RB number | 12 | 25 | Assuming that H2 falls down to (almost) full band of B11 Rx of interest |
| Noise floor | -98.5 | -95.5 | NF: 9dB |
| Interference (H2) @ antenna port | -73.7 | -73.7 |  |
| Composite (Noise+H2) @ antenna port | -73.7 | -73.7 |  |
| Required wanted signal power @ antenna port (under H2 fall-down) | -75.2 | -75.2 | Required SNR: -4dB (Diversity gain included), IM:2.5dB |
| B11 REFSENS | -100 | -97 |  |
| MSD | 24.8 | 21.8 | Required wanted signal power – REFSENS |

4.  Other modifications

Firstly, in [2], two values of TIB,c and RIB were proposed according to WF [6] but thus far in REL-14, it seems that only the single value with HTF was proposed such as in 12A-66A. So it is proposed to delete the values when HTF is not inserted from section 6.22.4 of TP.

In addition, in the first table (Table 6.22.1-1) of the original TP, 3MHz support in Band 28 was missed. This error is also corrected.

5.  Conclusion

This paper is to propose MSD values for 11A-28A. TP is attached to capture the discussion above.

6.  Reference

[1]     RP-160616	Revision of WI LTE Advanced inter-band CA Rel-14 for 2DL/1UL, Qualcomm (Rapporteur)

[2]     R4-161903    TP for TR36.714-02-01: IMD/Harmonics Issues on LTE-A Inter-band Carrier Aggregation (2DL/1UL) of Band 11 and Band 28, Softbank Corp.

[3]     SAYEY733MBD0F0A Data sheet, Murata

[4]     D5PF788M0M3Y9 Data sheet, Taiyo Yuden

[5]     B8549 Data sheet, TDK Epcos

[6]     R4-155396	Way Forward on Pcell support exclusion request for A2, Vodafone, MICROSOFT EUROPE SARL, NTT DOCOMO

Annex: TP for TR36.714-02-01 (v0.3.1)

[Unchanged portions skipped]

6.22	CA_11A-28A_BCS0

6.22.1	Operating bands for CA

Table 6.22.1-1: Inter-band CA

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_11-28 | 11 | 1427.9 MHz | – | 1447.9 MHz | 5, 10 | 1475.9 MHz | – | 1495.9 MHz | 5, 10 | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 3, 5, 10, 15, 20 | 758 MHz | – | 803 MHz | 3, 5, 10, 15, 20 | TDD |

6.22.2		Channel bandwidths per operating band for CA

Table 6.22.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| CA_11A-28A | 11 |  |  | Yes | Yes |  |  | 0 |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of  TS 36.101 [6] when operating in single carrier mode.

6.22.3	Co-existence studies

The harmonic relations of Band 11 and Band 28 are calculated as Table 6.22.3-1. It is observed that the 2nd harmonic of Band 28 Tx will cover the whole receiving band of Band 11.

Table 6.22.3-1: Impact of UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 11 | 1427.9 | 1447.9 | 1475.9 | 1495.9 | 2855.8 | 2895.8 | 4283.7 | 4343.7 | 2951.8 | 2991.8 | 4427.7 | 4487.7 |
| 28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 1516 | 1606 | 2274 | 2409 |

Table 6.22.3-2: Band 11 and Band 28 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 1475.9 | 1495.9 |
| 2nd order harmonics frequency range (MHz) | 1516 | 1606 | 2951.8 | 2991.8 |
| 3rd order harmonics frequency range (MHz) | 2274 | 2409 | 4427.7 | 4487.7 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 672.9 | 737.9 | 2233.9 | 2298.9 |
| 3rd order IMD products | ¦f2_high – 2*f1_low¦ | ¦f2_low – 2*f1_high¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 20.1 | 130.1 | 2148.8 | 2233.8 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 2991.9 | 3101.9 | 3709.8 | 3794.8 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 738 | 823 | 1430.9 | 1540.9 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1_low – f2_BWmax) | (f1_high + f2_BWmax) | (f2_low – f1_BWmax) | (f2_high + f1_BWmax) |
| IMD frequency limits (MHz) | 748 | 813 | 1455.9 | 1515.9 |

It can be seen from Table 6.22.3-2 that:

- the 2nd harmonics of BS transmitting in Bands 11 and 28 may fall into none of the BS receive band of 3GPP

- the 3rd harmonics may fall into BS receive band of Bands 30 and 40

- the 2nd IMD products may fall into BS receive band of Bands 12, 17, 28, 44 and 68

- the 3rd IMD products may fall into the BS receive band of Bands 11, 13, 14, 18, 21, 26, 27, 28, 43, 44 and 45,

assuming (except the last row) the whole 20MHz DL frequency of Band 11 and the whole 45 MHz DL frequency of Band 28.

If the BS is only transmitting up to 10 MHz DL in Band 11 and 20MHz DL in Band 28 as stated in the WID, then the 3rd IMD products will not fall into the BS receive band of Band 11, 18, 26 and 28 as shown in the last row in table 4. (Note that even so, Band 28 is still subject to 2nd IMD fall down.)

It should be noted that Bands 12, 13, 14, 17, 27, 30, 40, 43 and 45 are not intended for use in the same geographical area as Bands 11 and 28. In addition, co-existence between Band 28 and Band 44 is unlikely as Band 44 is TDD variant of APAC-700 spectrum then it could be expected that the deployment of Band 28 and 44 is mutually exclusive. Therefore, the focus here will be on the harmonics and IMD falling into Bands 21 and 28.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 21 or 28 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 11 and 28 BS transmitters do not share the same antenna with Band 21 or 28 BS receiver.

.

6.22.4	ΔTIB and ΔRIB values

Concerning CA of a mid-band and a low-band, default values of ∆TIB=0.3dB and ∆RIB=0dB were agreed as a framework but conventionally for CA including B11 and B21, ∆TIB=0.4dB has been proposed/accepted for mid-band to accommodate “extended Low-High” diplexer. In addition, a harmonic trap filter (HTF) is assumed in Band 28 path that introduces additional relaxation of 0.3dB for Tx and 0.2dB for Rx.

For two simultaneous DL and only one UL, the TIB,c and RIB values are given in the tables below.

Table 6.22.4-2: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_11A-28A | 11 | 0.4 |
|  | 28 | 0.6 |

Table 6.22.4-3: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_11A-28A | 11 | 0 |
|  | 28 | 0.2 |

As pointed out in section 3, there is a harmonic relation for the 2nd harmonic of Band 28 Tx falling down to Band 11 Rx. Thus, we need to evaluate MSD in Band 11 when Band 28 is used as Pcell.

6.22.5	REFSENS requirements

As noted above, the 2nd harmonic of B28 Tx will fall into B11 Rx. The following table is REFSENS exception when harmonic fall down occurs.

Table 6.22.5-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1,4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_11A-28A10 | 11 |  |  | -75.2 | -75.2 |  |  | FDD |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 6.22.5-2: Uplink configuration for the low band

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_11A-28A | 28 |  |  | 12 | 25 |  |  | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in TS 36.101 Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to TS 36.101 Table 7.3.1-2 applies. |  |  |  |  |  |  |  |  |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"