3GPP TSG-RAN WG4 Meeting #81 	R4-1609048

Reno, NV, US, 14 – 18 November, 2016

Source:	SoftBank Corp.

Title:	TP for TR36.714-03-01: the support of CA_8A-28A-41A BCS0

Agenda item:	8.3.2

Document for:	Approval

1.  Introduction

B8+B28+B41 was approved in 3DL/1UL basket WID in the second last RAN-P [1] but left untouched for one RAN-P cycle. This paper is to evaluate IM/harmonics and relaxation required. This includes the relevant TP to complete this combination.

Note that this TP needs agreements on the additions of:

- BCS1 in 8A-41A, which is to modify BW supports both in Band 8 and Band 41 [2] and

- BCS1 in 28A-41A, which is to add 15MHz and 20MHz BW support in B28 [3].

2. Basic Configurations

As written in the WID, the following configurations are assumed for B8+B28+B41.

Table 1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8-28-41 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 2: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_8A-28A-41A | 8 |  |  | Yes | Yes |  |  | 50 | 0 |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |

3.  Co-existence

Firstly, the 3rd order IMD products caused in the BS by transmitting of Band 8, Band 28 and Band 41 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 3 below:

Table 3: Band 8, Band 28 and Band 41 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 925 | 960 | 2496 | 2690 |
| 3rd order IMD products | ¦f1-high + f2-high – f3-low¦ |  |  | ¦f1-low + f2-low – f3-high¦ |  |  |
| IMD frequency limits (MHz) | 733 |  |  | 1007 |  |  |
| 3rd order IMD products | ¦f1-low + f3-low – f2-high¦ |  |  | ¦f1-high + f3-high – f2-low¦ |  |  |
| IMD frequency limits (MHz) | 2294 |  |  | 2568 |  |  |
| 3rd order IMD products | ¦f2-low + f3-low – f1-high¦ |  |  | ¦f2-high + f3-high – f1-low¦ |  |  |
| IMD frequency limits (MHz) | 2618 |  |  | 2892 |  |  |
| 3rd order IMD products | ¦f1-low + f2-low + f3-low¦ |  |  | ¦f1-high + f2-high + f3-high¦ |  |  |
| IMD frequency limits (MHz) | 4179 |  |  | 4453 |  |  |

It can be seen from Table 3 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (8 + 28 + 41) may fall into BS receive band of Bands 5, 6, 7, 8, 13, 14, 18, 19, 20, 26, 27, 28, 30, 38, 40, 41 and 44. However, Bands 13, 14, 20, 27 and 30 are not intended for use in the same geographical area as Bands 8, 28 and 41. Band 44 can be excluded as it is TDD variant of Band 28 and Band 41 can also be excluded as its Tx and Rx are mutual exclusive . Therefore, the focus here is on the 3rd order IMD products falling into Bands 5, 6, 7, 8, 18, 19, 26, 28, 38 and 40.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Bands 5, 6, 7, 8, 18, 19, 26, 28, 38 and 40 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (8 + 28 + 41) BS transmitter does not share the same antenna with Bands 5, 6, 7, 8, 18, 19, 26, 28, 38 and 40 BS receiver.

Therefore, it is recommended that Band (8 + 28 + 41) BS transmitter should not share the same antenna with Bands 5, 6, 7, 8, 18, 19, 26, 28, 38 and 40 BS receiver to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Bands 5, 6, 7, 8, 18, 19, 26, 28, 38 and 40 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

For UE, Table 4 gives the 2nd and 3rd harmonic for CA with B8+ B28 + B41. The 3rd harmonic of Band 8 UL will fall into upper part of Band 41 DL but it was concluded in 2DL of 8A-41A BCS0 that a harmonic trap filter is not needed due to spectrum occupancies of interested operators. In this 3CA, the same can be applied. Other than that, it is found that none of the products falls into the B8, B28 or B41 UE receiving bands.

Table 4: UE harmonic table for 3DL/1UL

| UE DL Carriers with B8, B28, B41 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 1760 to 1830 |
| 2*fy_low to 2*fy_high | 1406 to 1496 |
| 2*fz_low to 2*fz_high | 4992 to 5380 |
| 3*fx_low to 3*fx_high | 2640 to 2715 |
| 3*fy_low to 3*fy_high | 2109 to 2244 |
| 3*fz_low to 3*fz_high | 7488 to 8070 |

4.  ΔTIB and ΔRIB

The followingΔTIB and ΔRIB are proposed based on its constituent 2DL/1ULs:

Table 5: ΔTIB of constituent 2DL/1UL

| ΔTIB | 8+28 | 8+41 | 28+41 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0.6 | 0.3 | - | 0.6 |
| B28 | 0.5 | - | 0.3 | 0.5 |
| B41 | - | 0.3 | 0.3 | 0.3 |

Table 6: ΔRIB of constituent 2DL/1UL

| ΔRIB | 8+28 | 8+41 | 28+41 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0.2 | 0 | - | 0.2 |
| B28 | 0.1 | - | 0 | 0.1 |
| B41 | - | 0 | 0 | 0 |

Note that in B8+B28, Pcell is limited to Band 8 due to antenna performance issues. Then notes are put to indicate this limitation.

Table 7: Proposed ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_8A-28A-41A15 | 8 | 0.6 |
|  | 28 | 0.5 |
|  | 41 | 0.3 |
| NOTE 15: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 41 |  |  |

Table 8: Proposed ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_8A-28A-41A14 | 8 | 0.2 |
|  | 28 | 0.1 |
|  | 41 | 0 |
| NOTE 14: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 41 |  |  |

5.  REFSENS

B8+B41 CA is subject to REFSENS exception as per Table 7.3.1A-0a of [2], i.e. REFSENS is not specified when the 3rd harmonic of Band 8 Tx falls into Band 41 Rx. Then for this combination, it is necessary to add exception information below:

Table 9 (7.3.1A-0a): Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_8A-28A-41A22 | 8 |  |  | N/A | N/A |  |  | FDD |
|  | 28 |  |  | N/A | N/A | N/A | N/A |  |
|  | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| NOTE 22: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of Band 8 for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of Band 41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |

6.  Summary

This paper investigates all the necessary elements to complete CA_8A-28A-41A. The relevant TP is shown in Annex.

7.  Reference

[1]      RP-160945     Revised WID: LTE Advanced inter-band CA Rel-14 for 3DL/1UL, Huawei (Rapporteur).

[2]      R4-1609046   TP for TR36.714-02-01: the support of CA_8A-41A BCS1,

[3]      R4-1609047   TP for TR36.714-02-01: the support of CA_28A-41A BCS1

Annex: TP for TR36.714-03-01 (v0.5.0)

[Unchanged portions skipped]

6.Y 	CA_8A-28A-41A_BCS0

6.Y.1	Operating bands for CA

Table 6.Y.1-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8-28-41 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.Y.2	Channel bandwidths per operating band for CA

Table 6.Y.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_8A-28A-41A | 8 |  |  | Yes | Yes |  |  | 50 | 0 |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: For the UE that signals support of a 3DL CA bandwidth combination set, the UE shall support all single carrier channel bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 and all the 2DL CA bandwidth combination sets in Table 5.6A.2-1 of TS 36.101 whose channel bandwidths are entirely covered by the signaled 3DL CA bandwidth combination set. |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

6.Y.3 	Harmonics and IMD study

6.Y.3.1	For the BS

The 3rd order IMD products caused in the BS by transmitting of Band 8, Band 28 and Band 41 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.Y.3.1-1 below:

Table 6.Y.3.1-1: Band 8, Band 28 and Band 41 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 925 | 960 | 2496 | 2690 |
| 3rd order IMD products | ¦f1-high + f2-high – f3-low¦ |  |  | ¦f1-low + f2-low – f3-high¦ |  |  |
| IMD frequency limits (MHz) | 733 |  |  | 1007 |  |  |
| 3rd order IMD products | ¦f1-low + f3-low – f2-high¦ |  |  | ¦f1-high + f3-high – f2-low¦ |  |  |
| IMD frequency limits (MHz) | 2294 |  |  | 2568 |  |  |
| 3rd order IMD products | ¦f2-low + f3-low – f1-high¦ |  |  | ¦f2-high + f3-high – f1-low¦ |  |  |
| IMD frequency limits (MHz) | 2618 |  |  | 2892 |  |  |
| 3rd order IMD products | ¦f1-low + f2-low + f3-low¦ |  |  | ¦f1-high + f2-high + f3-high¦ |  |  |
| IMD frequency limits (MHz) | 4179 |  |  | 4453 |  |  |

It can be seen from Table 3 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (8 + 28 + 41) may fall into BS receive band of Bands 5, 6, 7, 8, 13, 14, 18, 19, 20, 26, 27, 28, 30, 38, 40, 41 and 44. However, Bands 13, 14, 20, 27 and 30 are not intended for use in the same geographical area as Bands 8, 28 and 41. Band 44 can be excluded as it is TDD variant of Band 28 and Band 41 can also be excluded as its Tx and Rx are mutual exclusive . Therefore, the focus here is on the 3rd order IMD products falling into Bands 5, 6, 7, 8, 18, 19, 26, 28, 38 and 40.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Bands 5, 6, 7, 8, 18, 19, 26, 28, 38 and 40 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (8 + 28 + 41) BS transmitter does not share the same antenna with Bands 5, 6, 7, 8, 18, 19, 26, 28, 38 and 40 BS receiver.

Therefore, it is recommended that Band (8 + 28 + 41) BS transmitter should not share the same antenna with Bands 5, 6, 7, 8, 18, 19, 26, 28, 38 and 40 BS receiver to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Bands 5, 6, 7, 8, 18, 19, 26, 28, 38 and 40 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

6.Y.3.2	For the UE

Table 6.Y.3.2-1 gives the 2nd and 3rd harmonic for CA with B8+ B28 + B41. The 3rd harmonic of Band 8 UL will fall into upper part of Band 41 DL but it was concluded in 2DL of 8A-41A BCS0 that a harmonic trap filter is not needed due to spectrum occupancies of interested operators. In this 3CA, the same can be applied. Other than that, it is found that none of the products falls into the B8, B28 or B41 UE receiving bands.

Table 6.Y.3.2-1: UE harmonic table for 3DL/1UL

| UE DL Carriers with B8, B28, B41 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 1760 to 1830 |
| 2*fy_low to 2*fy_high | 1406 to 1496 |
| 2*fz_low to 2*fz_high | 4992 to 5380 |
| 3*fx_low to 3*fx_high | 2640 to 2715 |
| 3*fy_low to 3*fy_high | 2109 to 2244 |
| 3*fz_low to 3*fz_high | 7488 to 8070 |

6.Y.4	ΔTIB,c and ΔRIB,c values

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c is defined for applicable bands in Table 6.Y.4-1.

Table 6.Y.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_8A-28A-41A15 | 8 | 0.6 |
|  | 28 | 0.5 |
|  | 41 | 0.3 |
| NOTE 15: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 41 |  |  |

For the UE which supports inter-band carrier aggregation configuration in Table 6.Y.4-2 with uplink in one E-UTRA band, the minimum requirement for reference sensitivity in Table 7.3.1-1 (of TS 36.101 [2]) shall be increased by the amount given in ΔRIB,c in Table 6.Y.4-2 for the applicable E-UTRA bands.

Table 6.Y.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_8A-28A-41A14 | 8 | 0.2 |
|  | 28 | 0.1 |
|  | 41 | 0 |
| NOTE 14: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 41 |  |  |

Note that in B8+B28, Pcell is limited to Band 8 due to antenna performance issues. Then notes are put to indicate this limitation.

6.Y.5	REFSENS requirements

The REFSENS exception is specified as below to reflect exclusion of harmonics in B8+B41:

Table 6.Y.5-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_8A-28A-41A22 | 8 |  |  | N/A | N/A |  |  | FDD |
|  | 28 |  |  | N/A | N/A | N/A | N/A |  |
|  | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| NOTE 22: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of Band 8 for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of Band 41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"