3GPP TSG RAN WG4 Meeting #81	R4-1609251

Reno, USA, 14 – 18 NOV 2016

Agenda Item:	6.3.4.2.2

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Updated Simulation Results for NB-IoT NPUSCH Format 1

Document for:	Discussion

# Introduction

In [1], we provided the ideal NPUSCH Format 1 simulation results based on the agreed simulation assumptions. In this paper, we further provide the updated ideal NPUSCH Format 1 simulation results as well as the practical simulation results with implementation margin for NPUSCH Format 1.

# Simulation Assumptions

For the NPUSCH format 1, RAN4 agreed in RAN4#80 the updated NPUSCH simulation assumptions as shown in the following tables [2].

# Table 2-1: Simulation assumption for NPUSCH format 1

| Simulation number | BW | Singe-/Multi-tone | Sub-carrier spacing | repetition level | Number of tones | Modulation (Note 4) | RFC | Propagation Condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 180KHz | Single | 3.75KHz | 1,16,64 | 1 | BPSK | R.NB3-1 | ETU1 | 1x 2 | [70%] | TBD |
| 2 | 180KHz | Single | 15KHz | 1,16,64 | 1 | [BPSK] | R.NB3-2 | ETU1 | 1 x 2 | [70%] | TBD |
| 3 | 180KHz | Multi-tone | 15KHz | 2,16,64 | 12 | QPSK | R.NB3-7 | ETU1 | 1 x 2 | [70%] | TBD |
| 4 | 180KHz | Multi-tone | 15KHz | 2,16,64 | 6 | QPSK | R.NB3.6 | ETU1 | 1 x 2 | [70%] | TBD |
| 5 | 180KHz | Multi-tone | 15KHz | 2,16,64 | 3 | QPSK | R.NB3.5 | ETU1 | 1 x 2 | [70%] | TBD |
| Note 1: Assume the frequency offset value is [0] and channel estimation lengths are 16ms for 3.75kHz and 4ms for 15kHz. For case 3 with repetition level 2, use 2ms channel estimation length. Note 2: Assume HARQ retransmission RV0 or RV2, refer to RAN1 spec TS 36.213 Note 3: Interesting companies are encouraged to provide the simulation results in the next RAN4 meeting for all the scenarios if they can. Note 4: see TS36.211 Clause 10.1 for the detailed modulation |  |  |  |  |  |  |  |  |  |  |  |

Table 2-2 FRC parameters for NPUSCH for single tone

| Reference channel | R.NB3-1 | R.NB3-2 |
| --- | --- | --- |
| Sub-carrier spacing (kHz) | 3.75 | 15 |
| Number of tone | 1 | 1 |
| Diversity | No | No |
| Modulation | BPSK | BPSK |
| IMCS / IRU | 0 / 1 | 0 / 1 |
| Payload size (bits) | 32 | 32 |
| Allocated resource unit | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 |
| Total symbols per resource unit | 96 | 96 |
| Maximum number of transmissions | [4] | [4] |

Table 2-3 FRC parameters for NPUSCH for multi-tone

| Reference channel | R.NB3-5 | R.NB3-6 | R.NB3-7 |
| --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 15 | 15 |
| Number of tone | 3 | 6 | 12 |
| Diversity | No | No | No |
| Modulation | QPSK | QPSK | QPSK |
| ITBS / IRU | 3 / 0 | 7 / 0 | 9 / 0 |
| Payload size (bits) | 40 | 104 | 136 |
| Allocated resource unit | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 2/3 |
| Code rate (effective) | 0.22 | 0.44 | 0.56 |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 |
| Total number of bits per resource unit | 288 | 288 | 288 |
| Total symbols per resource unit | 144 | 144 | 144 |
| Maximum number of transmissions | [4] | [4] | [4] |

# Simulation Results

As defined in [2], NPUSCH Format 1 performance requirements will be defined with the SNRs for the throughput equal to or larger than 70% fraction of maximum throughput of the fixed reference channels.

Table 3-1 shows the ideal simulation results with 15KHz subcarrier spacing for single and multi-subcarrier NPUSCH transmission.

# Table 3-1 Ideal NPUSCH format 1 Simulation Results (15KHz Subcarrier spacing)

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Repetition number |  | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 15KHz | 1 | ETU 1Hz Low | A16-2 | 1 |  | 70% | -4.1 |
|  |  |  |  |  |  | 16 |  | 70% | -10.9 |
|  |  |  |  |  |  | 64 |  | 70% | -15.0 |
|  |  |  | 3 | ETU 1Hz Low | A16-3 |  | 2 | 70% | -4.8 |
|  |  |  |  |  |  |  | 16 | 70% | -10.3 |
|  |  |  |  |  |  |  | 64 | 70% | -14.2 |
|  |  |  | 6 | ETU 1Hz Low | A16-4 |  | 2 | 70% | -1.9 |
|  |  |  |  |  |  |  | 16 | 70% | -8.2 |
|  |  |  |  |  |  |  | 64 | 70% | -11.7 |
|  |  |  | 12 | ETU 1Hz Low | A16-5 |  | 2 | 70% | -0.9 |
|  |  |  |  |  |  |  | 16 | 70% | -9.4 |
|  |  |  |  |  |  |  | 64 | 70% | -14.0 |

# Table 3-2 Practical NPUSCH format 1 Simulation Results (15KHz Subcarrier spacing)

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Repetition number |  | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 15KHz | 1 | ETU 1Hz Low | A16-2 | 1 |  | 70% | -2.1 |
|  |  |  |  |  |  | 16 |  | 70% | -8.4 |
|  |  |  |  |  |  | 64 |  | 70% | -12.0 |
|  |  |  | 3 | ETU 1Hz Low | A16-3 |  | 2 | 70% |  |
|  |  |  |  |  |  |  | 16 | 70% |  |
|  |  |  |  |  |  |  | 64 | 70% |  |
|  |  |  | 6 | ETU 1Hz Low | A16-4 |  | 2 | 70% |  |
|  |  |  |  |  |  |  | 16 | 70% |  |
|  |  |  |  |  |  |  | 64 | 70% |  |
|  |  |  | 12 | ETU 1Hz Low | A16-5 |  | 2 | 70% |  |
|  |  |  |  |  |  |  | 16 | 70% |  |
|  |  |  |  |  |  |  | 64 | 70% |  |

# Conclusion

In this paper, we provided the updated ideal simulation results and practical simulation results with implementation margin for NPUSCH Format 1 with 15KHz subcarrier spacing for single and multi-subcarrier NPUSCH transmission based on the agreed simulation assumptions.

# References

- R4-168733, “Ideal Simulation Results for NPUSCH Format 1”, Nokia, Alcatel-Lucent Shanghai Bell

- R4-165359, “Updated simulation assumptions for NB-IoT BS demodulation performance requirements”, Huawei, Ericsson, Nokia, ZTE