3GPP TSG RAN WG4 Meeting #81	R4-1609252

Reno, USA, 14 – 18 NOV 2016

Agenda Item:	6.3.4.2.2

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Updated Simulation Results for NPUSCH Format 2

Document for:	Discussion

# Introduction

In [1], we provided the ideal simulation results for NPUSCH Format 2 based on the updated NPUSCH simulation assumptions [2].  In this paper, we provide the updated ideal simulation results for NPUSCH Format 2 as well as the practical simulation results with implementation margin for NPUSCH Format 2.

# Simulation Assumptions

For the NPUSCH format 2, RAN4 agrees to define the following test cases, and related main configurations are listed as in Table 2-1. The test metric is:

- The DTX to ACK probability, i.e. the probability that ACK is detected when nothing was sent, shall not exceed 1%;

- The ACK missed detection probability shall not exceed 1% at the given SNR.

# Table 2-1: Simulation assumption for NPUSCH format 2

| Test number | BW | Singe-/Multi-tone | Sub-carrier spacing | repetition level | Modulation | Propagation Condition | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 180KHz | Single | 3.75KHz | 1,16,64 | BPSK | EPA5 | 1x 2 | TBD |
| 2 | 180KHz | Single | 15KHz | 1,16,64 | BPSK | EPA5 | 1 x 2 | TBD |
| Note 1: Assume the frequency offset value is [0] and channel estimation lengths are 16ms for 3.75kHz and 4ms for 15kHz; For repetition level 1, the transmission length is 2 subframes, i.e. 8ms and 2ms for 3.75KHz and 15KHz sub-carrier spacing respectively. Then assume using 1 subframe for channel estimation length. |  |  |  |  |  |  |  |  |

# Simulation Results

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent per NPUSCH format 2 transmissions. The updated ideal simulation results with 15KHz subcarrier spacing for ACK missed detection is presented in Table 3-1. The practical results with implementation margin is presented in Table 3-2.

Table 3-1 Ideal Simulation Results for NPUSCH format 2

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Number of allocated subcarriers | Subcarrier spacing | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | EPA 5 Low | 1 | 15KHz | 1 | 3.34 |
|  |  |  |  |  | 16 | -9.10 |
|  |  |  |  |  | 64 | -14.19 |

Table 3-2 Practical Simulation Results with Implementation Margin for NPUSCH format 2

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Number of allocated subcarriers | Subcarrier spacing | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | EPA 5 Low | 1 | 15KHz | 1 | 5.34 |
|  |  |  |  |  | 16 | -6.50 |
|  |  |  |  |  | 64 | -11.5 |

# Conclusion

In this paper, we provided the practical simulation results for NPUSCH Format 2 with 15KHz subcarrier spacing for ACK missed detection according to the updated simulation assumptions.

# References

- R4-167875, Ideal Simulation Results for NPUSCH Format 2, Nokia, Alcatel-Lucent Shanghai Bell

- R4-165359, “Updated simulation assumptions for NB-IoT BS demodulation performance requirements”, Huawei, Ericsson, Nokia, ZTE

- R4-167065, “CR: Demodulation requirements for NPUSCH format 2 (Rel-13)”, Huawei