3GPP TSG-RAN WG4 Meeting #81                                                                                         R4-1609341

Reno, NV, USA, 14 – 18 Nov, 2016

Agenda Item:	6.3.4.2.1

Source: 	Samsung

Title: 	Simulation results for NB-IoT PRACH

Document for:	Discussion

Introduction

In this contribution, we give simulation results for NB-IoT PRACH.

Discussion

Evaluation assumptions were summarized in table below.

| Parameter | Values |
| --- | --- |
| NPRACH preamble format | 0 |
| Number of Repetitions | 8, 32 |
| Number of Subcarriers | 12 |
| Antenna configuration | 1 x 2 |
| Narrowband physical layer cell identity | 0 |
| NPRACH signature | 0 |
| frequency offset | 0Hz (AWGN); 200Hz EPA1 |
| Propagation channels | AWGN, EPA1 |
| Detection performance | Missed detection rate (1%) with false alarm rate (0.1%) |
| Timing estimation | Timing error probability with the limit of 3.646us |
| Timing Offset | [30]us |

Alignment and impairment results for Required SNR for Missed detection rate 1% are given in table 1 and table 2 below.

Table 1. NPRACH alignment results

| Evaluation cases | Required SNR [dB] |
| --- | --- |
| NPRACH AWGN Case 1.1 0Hz repetition = 8 1x2 (Cell-ID=0 and signature=0) with 3.646us | -4.5 |
| NPRACH AWGN Case 2.1 0Hz repetition = 32 1x2 (Cell-ID=0 and signature=0) with 3.646us | -9.0 |
| NPRACH EPA1 Case 1.1 200Hz repetition = 8 1x2 (Cell-ID=0 and signature=0) with 3.646us | 3.0 |
| NPRACH EPA1 Case 2.1 200Hz repetition = 32 1x2 (Cell-ID=0 and signature=0) with 3.646us | -2.1 |

Table 2. NPRACH impairment results

| Evaluation cases | Required SNR [dB] |
| --- | --- |
| NPRACH AWGN Case 1.1 0Hz repetition = 8 1x2 (Cell-ID=0 and signature=0) with 3.646us | -2.5 |
| NPRACH AWGN Case 2.1 0Hz repetition = 32 1x2 (Cell-ID=0 and signature=0) with 3.646us | -7.0 |
| NPRACH EPA1 Case 1.1 200Hz repetition = 8 1x2 (Cell-ID=0 and signature=0) with 3.646us | 5.0 |
| NPRACH EPA1 Case 2.1 200Hz repetition = 32 1x2 (Cell-ID=0 and signature=0) with 3.646us | -0.1 |

Conclusion

In this contribution, we give simulation results for NB-IoT PRACH.