3GPP TSG-RAN WG4 Meeting #81	R4-1609355

Reno, Nevada, USA, 14 – 18 November 2016

Agenda Item:	11.3.1

Source: 	Samsung

Title: 	Simulation results for NR coexistence study in indoor scenario

Document for:	Discussion

# 1. Introduction

NR coexistence study is essential for evaluating proper RF parameters like ACLR, ACS, etc. In RAN4#80, urban macro, dense urban, and indoor were considered as baseline simulation scenarios. In RAN4#80bis meeting, the remaining parameters of indoor scenario, i.e., the topology, antenna model, throughput formula etc. were fixed. Based on the agreements, this contribution presents the ACIR evaluation results for NR coexistence study in indoor scenario at 30 GHz carrier frequency.

# 2. Discussion

## 2.1 Coexistence simulation case

The NR eMBB is assumed under synchronized network, where aggressor and victim have the same configuration.

Table 2.1-1 Simulation case of coexistence study for NR

| Cases | Operation mode | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- |
| 1 | TDD | NodeB of NR eMBB | UE of NR eMBB | Downlink |
| 2 | TDD | UE of NR eMBB | NodeB of NR eMBB | Uplink |

## 2.2 Simulation parameters

Network layout, propagation model etc. are the same with [1]. BS beamforming, UE beamforming are in agreement with [2], [3], respectively. SINR to Throughput mapping follows [4]. Beamforming are employed at both the BS side and UE side. In addition, only the antenna element gain of BSs and UEs are considered in the cell selection process.

# 3. Simulation results

## 3.1 Downlink

This section presents the downlink simulation results. Since with NF 9 dB and 11 dB, the curves coincide, and  the difference in absolute values is very small, less than 0.1, we only provide the results of NF 9 dB.

Table 3.1-1: Throughput loss at given ACIRs for downlink

| ACIR | 5 dB | 10 dB | 15 dB | 20 dB | 25 dB | 30 dB | 35 dB | 40 dB | 45 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mean throughput | 14.99 | 8.73 | 4.61 | 2.17 | 0.89 | 0.29 | 0.08 | 0.02 | 0 |
| 5%-tile UE throughput | 37.45 | 19.29 | 8.00 | 1.57 | 0.37 | 0.04 | 0.01 | 0 | 0 |

Figure 3.1-1: Throughput loss versus ACIR for downlink.

From Figure 3.1-1, the required ACIR to guarantee a 5% throughput loss are 14.5 dB and 17.4 dB for mean throughput and 5%-tile UE throughput, respectively. The impact of NF on the simulation results is negligible, NF 9 dB and 11 dB generates almost the same results.

## 3.2 Uplink

This section presents the uplink simulation results. In this case, NF 9 dB and NF 11 dB generates exactly the same results, it is only necessary to provide the results of one case, NF 9 dB.

Table 3.2-1: Throughput loss at given ACIRs for uplink

| ACIR | 5 dB | 10 dB | 15 dB | 20 dB | 25 dB | 30 dB | 35 dB | 40 dB | 45 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean throughput (%) | 6.75 | 3.33 | 1.46 | 0.57 | 0.20 | 0.07 | 0.02 | 0.01 | 0 |
| 5%-tile UE throughput | 32.53 | 16.06 | 5.69 | 1.38 | 0.53 | 0.13 | 0.01 | 0 | 0 |

Figure 3.2-1: Throughput loss versus ACIR for uplink.

From Figure 3.2-1, the required ACIR are 7.6 dB and 15.8 dB for mean throughput and 5%-tile UE throughput, respectively. In this case, NF almost has no impact on the throughput since it can be cancelled out in the SINR formula.

From the simulation results of both downlink and uplink, we have the following observation:

Observation 1: NF 9 dB and NF 11 dB generate almost the same ACIR evaluation results for both downlink and uplink in indoor scenario.

# 4. Conclusions

This proposal presents ACIR evaluations for coexistence study in indoor scenario at 30 GHz carrier frequency. Based on the simulation results, we obtain the following observation:

Observation 1: NF 9 dB and NF 11 dB generate almost the same ACIR evaluation results for both downlink and uplink in indoor scenario.

# 5. References

- R4-168953_r1, “WF on Simulation assumptions of Co-existence study for WP5D”, NTT Docomo.

- R4-168794 [5G NR], “Further Way forward on Dense urban Layout and BS Beamforming Model_v4,” Huawei.

- R4-167123, “WF on UE beamforming model for the ITU WP5D coexistence simulations”, Qualcomm.

- R4-168767 [5G NR] “WF on SINR vs throughput mapping_v1”, Huawei