3GPP TSG-RAN WG4 RAN4 #81                                        R4-1609359

Reno, Nevada, USA, 14 - 18 November, 2016

Agenda item:			9.3.1

Source:			Samsung, China Telecom

Title:	TP for TR 36.752: Conclusion

Document for:			Approval

# 1	Introduction

The study item on NB-IoT RF requirement for coexistence with CDMA was approved at RAN #72 [1]. In RAN4 #80, the details of coexistence simulation methodology has been well discussed with the way forward agreed [2], and the coexistence study simulation results from companies are provided and aligned in RAN4 #80Bis [3]. Based on the results and group discussion, the conclusion on two Scenarios impacting BS requirements has been made, captured in the RAN4 #80Bis chairman notes:

- Rel-13 NB-IoT BS ACLR requirement is stringent enough to guarantee the protection for CDMA system, and no additional requirement is required. - Rel-13 NB-IoT BS ACS requirement is stringent enough to guarantee the coexistence with neighboring CDMA system, and no additional requirement is required.

And the following way forward is also agreed [4]:

- RAN4 will study whether the achievable ACLR value obtained from Rel-13 NB-IoT UE RF requirement can guarantee the coexistence with CDMA, with the following method to obtain the achievable ACLR values from Rel-13 requirement: - Using NB-IoT UE spectrum emission mask specified in Table 6.6.2F.1-1, with 385kHz edge to edge separation for CDMA, i.e., around 49dB ACLR (integrated over 180kHz adjacent channel frequency based on the emission mask with 23dBm assumed for Tx power). - The way to determine the required ACLR value for coexistence with CDMA from submitted simulation results: - FFS. - Companies are encouraged to check the simulation and bring results in RAN4 #81 for further alignment and analysis, by focusing on the following scenarios: - Scenario-2: DL CDMA BS interfere to NB-IoT UE. - Scenario-3: UL NB-IoT UE interfere to CDMA BS. - For better alignment and more time for company to perform analysis, companies should provide their updated simulation results for Scenario 2 and 3, over RAN4 email reflector before 31th Oct. (one week before RAN4#81 contribution submission deadline). - In RAN4 #81, RAN4 should make the conclusion on whether or not Rel-13 NB-IoT UE RF requirement can satisfy the coexistence with legacy CDMA system.

In this contribution, we provide the text proposal for conclusion part in the final technical report TR36.752.

# Text Proposal

<<< Start of text proposal for TR >>>

# 8	Conclusion

As one of the most promising and competitive IoT technology, Narrowband IoT RF requirements have been completed in Rel-13 standardization while limited consideration has been taken in NB-IoT RF evaluation and specifications, which however challenges worldwide CDMA operators to handle the inter-operator coexistence issue as well as carrier planning for NB-IoT deployment. Thus this study item with results documented in TR36.752 was approved in order to evaluate coexistence requirement between NB-IoT and CDMA, and then further assess whether Rel-13 NB-IoT RF requirements could be reused or not.

After potential NB-IoT operating bands being identified for interference evaluation on NB-IoT coexistence with CDMA, extensive studies from various industry participants have been conducted with the following conclusion achieved: With more than 1.1MHz central carrier separation between NB-IoT and CDMA, or equivalently 385kHz edge to edge frequency separation, Rel-13 NB-IoT BS RF requirement is stringent enough to guarantee the coexistence with neighboring CDMA system, and no additional requirement is needed. For UE side, Rel-13 NB-IoT UE ACS requirement is stringent enough to guarantee the coexistence with neighboring CDMA system, and Rel-13 NB-IoT UE RF requirement can provide around 49dB ACLR under the condition of 385kHz edge to edge separation, which can guarantee the coexistence with neighboring CDMA system considering practical NB-IoT UE implementation margin, and no additional requirement is needed.

<<< Unchanged sections omitted >>>