3GPP TSG-RAN WG4 Meeting #81	R4-1609517

Reno, USA, 14 - 18 November, 2016

Title: 	Impairment results of NPBCH Demodulation for NB-IoT

Source: 	CATT

Agenda Item:	6.3.4.1.1

Document for:	Discussion

- Introduction

In RAN4#80bis Meeting, the Narrow Band IoT (NB-IoT) work item (WI) has reached a way forward on the NPBCH demodulation performance [1]. Interested companies are encouraged to provide NPBCH demodulation performance with impairment errors evaluated with the “keep-trying” decoding method. And also the simulation results are provided to meet the 1% BLER requirement for simulation cases as described in [2]. In this contribution, we provide simulation results to show the BLER performance for NPBCH with some power configuration error correction from the results we submitted last meeting.

- NPBCH Simulation Assumptions and Results

For NPBCH, two simulation cases have been agreed in RAN4#80bis Meeting [2]. In this contribution, the main simulation parameters are accordingly configured as in Table 1-4.  For Table2, Simulation Case 1 is for the single–antenna port scenario performance with only one NPBCH TTI, and for Table 3, Simulation Case 2 is for the 2 Tx antenna port with multiple NPBCH TTIs.

Table 1: Test Parameters for NPBCH

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Downlink power allocation | NPBCH_RA | dB | 0 | -3 |
|  | NPBCH_RB | dB | 0 | -3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 0 |

Table 2: Minimum performance NPBCH for Single-antenna port performance with one NPBCH TTI

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1 | 200 KHz | R.NB1.1 | EPA1 | 1 x 1 | 1 | TBD |

Table 3: Minimum performance NPBCH for 2 Tx Antenna Port with several NPBCH TTI-s

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 2 | 200 KHz | R.NB1.2 | EPA1 | 2 x 1 | 1 | TBD |

Table 4: Reference Channel for Category NB1 UE

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.1 | R.NB.2 |
| Number of transmitter antennas |  | 1 | 2 |
| Channel bandwidth | KHz | 200 | 200 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 50/1600 | 50/1600 |
| Payload (without CRC) | Bits | 34 | 34 |

To meet the NPBCH coverage requirements, the “keep-trying” decoding method for NPBCH may need to be performed.  It is defined as follows [2]:

The receiver characteristics of the NPBCH are determined by the probability of miss-detection of the NPBCH (Pm-bch), which is defined as

For the performance with a NPBCH TTI decoding, Where A is the number of correctly decoded N-MIB TB within one NPBCH TTI and B is the number of transmitted N-MIB TB within a NPBCH TTI .

For the performance with several NPBCH TTI-s decoding without soft combing between each NPBCH TTI interval, where A is the number of  interval of TBD ms continuous NPBCH TTI-s where there is at least one correctly decoded N-MIB TB, and B is the number of  interval of transmitted TBD ms continuous NPBCH  TTI-s

For Simulation Case 2, the NPBCH performances are evaluated without soft combining among different NPBCH TTI intervals.

In this contribution, we try to evaluate the accurate acquisition time and the corresponding SNR values.

The alignment simulation results of BLER vs. SNR for Simulation Case 1 are shown in Fig.1.

Fig.1. Single-antenna port performance with one NPBCH TTI

Talbe5 Summary of NPBCH performance of Simulation Case 1

| NPBCH Acquisition Time (ms) | Target SNR (dB) of Alignment results | Target SNR (dB) of Impairment results |
| --- | --- | --- |
| 1*640ms | -3.9 | -1.6 |
| 2*640ms | -6.8 | -4.5 |
| 3*640ms | -8.6 | -6.3 |
| 6*640ms | -12.0 | -9.7 |

Observation 1:  When the NPBCH acquisition time is 640ms, the BLER of 1% is achieved at SNR around -3.9dB of alignment result and -1.6dB of impairment result.

Observation 2:  When the NPBCH acquisition time is 3*640ms, the BLER of 1% is achieved at SNR around -8.6dB of alignment result and -6.3dB of impairment result.

Proposal 1:   For Simulation Case 1, the SNR for NPBCH is around -1.6dB with impairment.

Proposal 2:   For Simulation Case 1 to ensure certain coverage, the NPBCH acquisition time is at least 3*640ms.

The alignment simulation results of BLER vs. SNR for Simulation Case 2 is shown in Fig.2.

Fig.2. 2 Tx Antenna Port with several NPBCH TTI-s

Talbe6 Summary of NPBCH performance of Simulation Case 2

| NPBCH Acquisition Time (ms) | Target SNR (dB) of Alignment results | Target SNR (dB) of Impairment results |
| --- | --- | --- |
| 1*640ms | -7.6 | -5.3 |
| 2*640ms | -10.4 | -8.1 |
| 3*640ms | -11.5 | -9.2 |
| 4*640ms | -12.3 | -10 |
| 5*640ms | -12.9 | -10.6 |
| 6*640ms | -13.3 | -11 |
| 7*640ms | -13.6 | -11.3 |
| 8*640ms | -13.9 | -11.6 |
| 9*640ms | -14.2 | -11.9 |
| 10*640ms | -14.5 | -12.2 |

Observation 3:  When the NPBCH acquisition time is 640ms, the BLER of 1% is achieved at SNR around -7.6dB of alignment result and -5.3dB of impairment result.

Observation 4:  When the NPBCH acquisition time is 8*640ms, the BLER of 1% is achieved at SNR around -13.9dB of alignment result and -11.6dB of impairment result.

Proposal 3:   For Simulation Case 2 to ensure certain coverage, the NPBCH acquisition time is at least 8*640ms, the SNR for NPBCH is around -11.6dB with impairment.

- Conclusion

In this contribution, we provided simulation results to show the BLER performance for NPBCH.

Proposal 1:   For Simulation Case 1, the SNR for NPBCH is around -1.6dB with impairment.

Proposal 2:   For Simulation Case 1 to ensure certain coverage, the NPBCH acquisition time is at least 3*640ms.

Proposal 3:   For Simulation Case 2 to ensure certain coverage, the NPBCH acquisition time is at least 8*640ms, the SNR for NPBCH is around -11.6dB with impairment.

- Reference

[1] R4-167117, “Way forward on NPBCH Performance Requirements”, Intel, Qualcomm, Samsung, Ericsson, LGE,   OCT. 2016

[2] R4-168950, “CR on NPBCH Fixed Reference Channel for NB-IoT”, Qualcomm Incorporated, OCT. 2016