3GPP TSG-RAN WG4 Meeting #81	R4-1609518

Reno, USA, 14 - 18 November, 2016

Title: 	Impairment results of NPDCCH Demodulation for NB-IoT

Source: 	CATT

Agenda Item:	6.3.4.1.2

Document for:	Discussion

- Introduction

In RAN4#80bis Meeting, the Narrow Band IoT (NB-IoT) work item (WI) has reached a way forward [1]. Interested companies are encouraged to provide simulation results including impairment error using the repetition numbers specified [2]. In this contribution, we provide our impairment simulation results including the DC leakage margin to show the BLER performance for NPDCCH.

- NPDCCH Simulation Assumptions and Results

For NPDCCH, four simulation cases have been agreed in RAN4 [2]. In this contribution, the main simulation parameters are accordingly configured as in Table 1 and Table 2. And the reference channel configuration is given is Table 3. The errors of the repetition numbers for each simulation case in [2] have been corrected according to [1] as shown below.

Table 1: Minimum performance NPDCCH for Single-antenna cases

| Test number | Deployment mode | Repetition number (Rmax) | Operated carrier | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | Stand-alone/Guard-band | 128 | Anchor | R.NB.3 FDD | EPA5 | 1 | 1 | TBD |
| 2 | Stand-alone/Guard-band | 1024 | Non-anchor | R.NB.3 FDD | ETU1 | 1 | 1 | TBD |

Table 2 Minimum performance NPDCCH for Transmit diversity cases

| Test number | Deployment mode | Repetition number (Rmax) | Operated carrier | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 3 | In-band | 64 | Anchor | R.NB.4 FDD | EPA5 | 2 | 1 | TBD |
| 4 | In-band | 512 | Non-anchor | R.NB.4 FDD | ETU1 | 2 | 1 | TBD |

Table 3	   RMC table for NPDCCH

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.3 FDD | R.NB.4 FDD |
| Number of NRS ports |  | 1 | 2 |
| Channel bandwidth | MHz | 0.2 | 0.2 |
| Aggregation level | NCCE | 2 | 2 |
| DCI Format |  | N1 | N1 |
| Payload (without CRC) | Bits | 23 | 23 |

The alignment simulation result of BLER vs. SNR for Simulation Case 1 is shown in Fig.1.

Fig.1. BLER vs. SNR for Simulation Case 1

The alignment simulation result of BLER vs. SNR for Simulation Case 2 is shown in Fig.2.

Fig.2. BLER vs. SNR for Simulation Case 2

The alignment simulation result of BLER vs. SNR for Simulation Case 3 is shown in Fig.3.

Fig.3. BLER vs. SNR for Simulation Case 3

The alignment simulation result of BLER vs. SNR for Simulation Case 4 is shown in Fig.4.

Fig.4. BLER vs. SNR for Simulation Case 4

Result summary table:

| Simulation Number | Propagation Condition | Antenna Configuration | Repetition Number | Target SNR (dB) | Target SNR (dB) with impairment |
| --- | --- | --- | --- | --- | --- |
| 1 | EPA5 | 1x1 | 128 | -7.5 | -5.2 |
| 2 | ETU1 | 1x1 | 1024 | -13.9 | -11.6 |
| 3 | EPA5 | 2x1 | 64 | -6.5 | -4.2 |
| 4 | ETU1 | 2x1 | 512 | -12.3 | -10.0 |

- Conclusion

In this contribution, we provide simulation results to show the BLER performance for NPDCCH for different simulation cases.

Proposal: For Simulation Case 1 to 4, the SNR values with impairment are -5.2dB,-11.6dB,-4.2dB and -10.0dB respectively.

- Reference

[1] R4-168936, “Way forward on NB-IoT UE demodulation requirements”, Ericsson, OCT 2016

[2] R4-168739, “Correction of NPDCCH demodulation requirements”, Ericsson, OCT 2016