3GPP TSG-RAN WG4 Meeting 81										     R4-1609742

Reno, Nevada, USA, 14 – 18 Nov, 2016

Source: 	China Telecom

Title: 	TP for TR36.752 coexistence simulation results

Agenda Item:	9.3.3

Document for:	Approval

Background

In RAN plenary #72 the new study on NB-IoT RF requirement for coexistence with CDMA was approved [1]. A WF was agreed in [2] in RAN4#80bis meeting, and companies are encouraged to check the simulation and bring results in RAN4 #81 for further alignment and analysis, by focusing on the Scenarios 2&3.

Based on the WF, the latest simulation results were received and summarized in [3]. This contribution provides a text proposal to captured simulation results into TR 36.752.

- References

- RP-161187, “Study on NB-IoT RF requirement for coexistence with CDMA”, China Telecom, Sprint, SK Telecom, Qualcomm, Huawei, HiSilicon, Samsung

- R4-168962, WF on co-existence study between NB-IoT and CDMA, Samsung, China Telecom, Qualcomm, ,Ericsson, Huawei

- R4-1609741, Summary of coexistence results, China Telecom, Samsung, Huawei, Ericsson, Qualcomm

- Text Proposal

<Start of text proposal for TR>

- 6.4	Coexistence simulation results

- 6.4.1 Case1 – Downlink, NB-IoT aggressor, CDMA1x victim

Table 6.4.1 Simulation results for Case1

| Sourcing | Case 1 NB-IoT -> CDMA1x | CDMA 850MHz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS ACLR | 30 | 35 | 40 | 45 | 50 | 55 | 60 |
| Samsung | Capacity loss |  |  | 1.44 | 0.62 | 0.28 | 0.09 | 0.03 |
| Huawei | Capacity loss | 1.6 | 0.36 | 0.14 | 0.08 | 0.07 |  |  |
| Ericsson | Capacity loss |  |  | 0.10 | 0.03 | 0.01 | 0.00 | 0.00 |

- 6.4.2 Case2 – Downlink, CDMA1x aggressor, NB-IoT victim

Table 6.4.2 Simulation results for Case2

| Sourcing | Case 2 CDMA1x -> NB-IoT | CDMA 850MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | UE ACS | 20 | 25 | 30 | 35 | 40 |
| Samsung | SNR loss 5% | 1.55 | 0.76 | 0.40 | 0.24 | 0.16 |
|  | SNR loss 50% | 0.73 | 0.38 | 0.21 | 0.15 | 0.11 |
|  | SNR loss 95% | 0.14 | 0.07 | 0.03 | 0.02 | 0.01 |
|  | SNR loss 99% | 0.04 | 0.01 | 0.00 | 0.00 | 0.00 |
| Huawei | SNR loss 5% | 7.9 | 4.33 | 2.7 | 1.17 | 0.81 |
|  | SNR loss 50% | 4.59 | 2.9 | 1.68 | 0.97 | 0.42 |
|  | SNR loss 95% | 2.01 | 1.36 | 0.98 | 0.58 | 0.38 |
|  | SNR loss 99% | 1.99 | 0.34 | 0 | 0 | 0 |
| Ericsson | SNR loss 5% | 6 | 2.8 | 1.6 | 0.9 | 0.7 |
|  | SNR loss 50% | 1.3 | 0.8 | 0.5 | 0.3 | 0.2 |
|  | SNR loss 95% | 0.3 | 0.2 | 0 | 0 | 0 |
|  | SNR loss 99% | 0.1 | 0 | 0 | 0 | 0 |
| Qualcomm | SNR loss 5% | 3,77 | 1.76 | 0.89 | 0.47 | 0.25 |
|  | SNR loss 50% | 1.09 | 0.63 | 0.36 | 0.2 | 0.11 |
|  | SNR loss 95% | 0.27 | 0.14 | 0.07 | 0.03 | 0.02 |
|  | SNR loss 99% | 0.07 | 0.04 | 0.01 | 0.007 | 0.002 |

- 6.4.3 Case3 – Uplink, NB-IoT aggressor, CDMA1x victim

Table 6.4.3a Simulation results for Case3

| Sourcing | Case 3 NB-IoT -> CDMA1x 3 UEs - 60 kHz | CDMA 850MHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE ACLR | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 55 |
| Samsung | Capacity loss |  | 92.00 | 57.09 | 27.66 | 8.25 | 2.82 | 0.75 |  |
| Huawei | Capacity loss | 85.60 | 52.90 | 22.20 | 8.50 | 3.20 | 1.90 | 0.90 |  |
| Ericsson | Capacity loss | 73.18 | 65.23 | 52.78 | 35.49 | 18.50 | 7.96 | 2.66 |  |
| Qualcomm | Capacity loss |  |  | 52.24 | 27.75 | 12.33 | 5.02 | 2.02 | 0.88 |

Table 6.4.3b Simulation results for Case3

| Sourcing | Case 3 NB-IoT -> CDMA1x 12 UEs - 15 kHz | CDMA 850MHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE ACLR | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 55 |
| Samsung | Capacity loss |  |  | 80.9 | 41.55 | 12.34 | 3.91 | 1.06 |  |
| Qualcomm | Capacity loss |  |  | 64.76 | 35.68 | 17.18 | 7.05 | 2.64 | 1.06 |

Table 6.4.3c Simulation results for Case3

| Sourcing | Case 3 NB-IoT -> CDMA1x 48 UEs - 3.75 kHz | CDMA 850MHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE ACLR | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 55 |
| Samsung | Capacity loss |  |  | 86.14 | 41.48 | 13.15 | 4.15 | 1.39 |  |
| Huawei | Capacity loss | 93.2 | 61.1 | 27.3 | 9.2 | 5 | 2.4 | 0.6 |  |
| Qualcomm | Capacity loss |  |  | 71.8 | 44.31 | 21.41 | 8.81 | 3.26 | 1.23 |

- 6.4.4 Case4 – Uplink, CDMA1x aggressor, NB-IoT victim

Table 6.4.4a Simulation results for Case4

| Sourcing | Case 4 CDMA1x -> NB-IoT 3 UEs - 60 kHz | CDMA 850MHz |  |  |
| --- | --- | --- | --- | --- |
|  | BS ACS | 40 | 45 | 50 |
| Samsung | SNR loss 5% | 0.01 | 0.01 | 0.01 |
|  | SNR loss 50% | 0.03 | 0.02 | 0.02 |
|  | SNR loss 95% | 0.10 | 0.09 | 0.08 |
|  | SNR loss 99% | 0.10 | 0.09 | 0.08 |
| Huawei | SNR loss 5% | 0.03 | 0.01 | 0.01 |
|  | SNR loss 50% | 0.18 | 0.06 | 0.03 |
|  | SNR loss 95% | 0.13 | 0.04 | 0.01 |
|  | SNR loss 99% | 0.22 | 0.07 | 0.02 |
| Ericsson | SNR loss 5% | 0.40 | 0.40 | 0.40 |
|  | SNR loss 50% | 0.10 | 0.10 | 0.10 |
|  | SNR loss 95% | 0.00 | 0.00 | 0.00 |
|  | SNR loss 99% | 0.20 | 0.20 | 0.20 |
| Qualcomm | SNR loss 5% | 0.001 | 0 | 0 |
|  | SNR loss 50% | 0.001 | 0.001 | 0.001 |
|  | SNR loss 95% | 0.001 | 0 | 0 |
|  | SNR loss 99% | 0.02 | 0.01 | 0 |

Table 6.4.4b Simulation results for Case4

| Sourcing | Case 4 CDMA1x -> NB-IoT 12 UEs - 15 kHz | CDMA 850MHz |  |  |
| --- | --- | --- | --- | --- |
|  | SNR loss 5% | 40 | 45 | 50 |
| Samsung | SNR loss 50% | 0.01 | 0.01 | 0.01 |
|  | SNR loss 95% | 0.03 | 0.03 | 0.03 |
|  | SNR loss 99% | 0.09 | 0.08 | 0.08 |
|  | SNR loss 95% | 0.12 | 0.11 | 0.10 |
| Ericsson | SNR loss 5% | 0 | 0 | 0 |
|  | SNR loss 50% | 0.1 | 0.1 | 0.1 |
|  | SNR loss 95% | 0.3 | 0.3 | 0.3 |
|  | SNR loss 99% | 0.5 | 0.4 | 0.4 |
| Qualcomm | SNR loss 5% | 0 | 0 | 0 |
|  | SNR loss 50% | 0.002 | 0.001 | 0 |
|  | SNR loss 95% | 0.01 | 0.003 | 0.003 |
|  | SNR loss 99% | 0.01 | 0.003 | 0.002 |

Table 6.4.4c Simulation results for Case4

| Sourcing | Case 4 CDMA1x -> NB-IoT 48 UEs - 3.75 kHz | CDMA 850MHz |  |  |
| --- | --- | --- | --- | --- |
|  | BS ACS | 40 | 45 | 50 |
| Samsung | SNR loss 5% | 0.01 | 0.01 | 0.01 |
|  | SNR loss 50% | 0.03 | 0.03 | 0.03 |
|  | SNR loss 95% | 0.09 | 0.08 | 0.07 |
|  | SNR loss 99% | 0.10 | 0.08 | 0.08 |
| Huawei | SNR loss 5% | 0.50 | 0.15 | 0.05 |
|  | SNR loss 50% | 2.17 | 0.79 | 0.27 |
|  | SNR loss 95% | 2.30 | 0.97 | 0.36 |
|  | SNR loss 99% | 2.10 | 0.97 | 0.67 |
| Qualcomm | SNR loss 5% | 0.001 | 0 | 0 |
|  | SNR loss 50% | 0.002 | 0.001 | 0.001 |
|  | SNR loss 95% | 0.01 | 0.003 | 0.002 |
|  | SNR loss 99% | 0.003 | 0 | 0 |

< End of text proposal for TR >