3GPP TSG-RAN WG4 Meeting #81	R4-1609756

Reno, Nevada, USA, November 14th -18th, 2016

Agenda item:	11.5.2.2 - Receiver characteristics

Source: 	Qualcomm Incorporated

Title: 	Out-of-band blockers in the mm-wave spectrum

Document for:	Discussion

# Introduction

Millimeter wave blockers (or jammers) will have significant impact on performance at mm-wave bands. This paper presents preliminary findings on the major sources of blocking in the United States. This is a first step in defining an out-of-band blocker specification for NR. To illustrate the need for such a specification, we present the frequency response of a typical patch antenna.

A key goal of this paper is to invite other companies to investigate spectrum usage in other parts of the world. Additional important data to be gathered are:

- Transmit powers and emission masks of these blockers

- Directivity of the antennas used to transmit the signals

- Physical proximity of these transmitters from 5G networks to determine path loss

# Discussion

The following table lists the major blockers in the mm-wave bands in the United States.

| Start (GHz) | Stop (GHz) | Usage | Comments |
| --- | --- | --- | --- |
| 19.7 | 24 | Satellite | Satellite (smaller bands in the range) |
| 19.7 | 24 | Aviation | Aviation/Radiolocation |
| 19.7 | 24 | Fixed | Fixed (smaller bands in the range) |
| 19.7 | 24 | Land Mobile | Land Mobile (smaller bands in the range) |
| 20.2 | 21.2 | Satellite | Ka-band Downlink |
| 21.2 | 22.55 | Fixed wireless | Fixed Microwave Point-to-Point |
| 22.55 | 23.55 | Fixed wireless | Fixed Microwave Point-to-Point |
| 22.55 | 23.55 | Satellite | Satellite |
| 23.55 | 23.6 | Fixed wireless | Fixed Microwave Point-to-Point |
| 23.6 | 24 | Satellite | Earth exploration |
| 24 | 24.25 | Amateur | Amateur Radio |
| Start (GHz) | Stop (GHz) | Usage | Comments |
| 24 | 24.25 | Unlicensed | Unlicensed Point-to-Point |
| 24 | 24.25 | Unlicensed | ISM |
| 24 | 24.25 | Amateur | Amateur Radio |
| 24.05 | 24.25 | Land Mobile | Private Land Mobile |
| 24.25 | 24.45 | 5G | 5G Proposed FNPRM |
| 24.25 | 27.5 | Satellite | Satellite (smaller bands in the range) |
| 24.25 | 27.5 | Aviation | Aviation/Radiolocation |
| 24.25 | 27.5 | Fixed wireless | Fixed wireless (smaller bands in the range) |
| 24.25 | 27.5 | Land Mobile | Land Mobile (smaller bands in the range) |
| 24.25 | 30 | Radar | Airport Body Scanners |
| 24.25 | 24.45 | Fixed wireless | Fixed point-to-point wireless |
| 24.25 | 24.45 | Fixed wireless | Fixed wireless -- Microwave |
| 24.45 | 25.25 | Satellite | Satellite |
| 24.75 | 25.25 | 5G | 5G Proposed FNPRM |
| 24.75 | 25.05 | Satellite | Satellite: uplink earth to space for Aviation (FCC 47CFR Part 87) |
| 24.75 | 25.05 | Aviation | Aviation |
| 25.05 | 25.25 | Satellite | Satellite : uplink earth to space |
| 25.05 | 25.25 | Fixed wireless | Fixed wireless for point-to-point |
| 25.05 | 25.25 | Fixed wireless | Fixed Microwave P-P |
| 25.25 | 27.5 | Satellite | Satellite-Government |
| 25.25 | 27.5 | Satellite | Ka-band NASA Space to earth |
| 25.25 | 27.5 | Satellite | Non-federal satellite |
| 27.5 | 30 | Satellite | Satellite Fixed (LMDS) |
| 27.5 | 28.35 | Fixed wireless | LMDS |
| 27.5 | 29.5 | Satellite | Earth to space |
| 27.5 | 29.5 | Fixed | Fixed |
| 27.5 | 31 | Satellite | Satellite (earth to space) |
| Start (GHz) | Stop (GHz) | Usage | Comments |
| 27.85 | 28.35 | 5G | R&O |
| 29.1 | 29.25 | Fixed wireless | LMDS |
| 30 | 31 | Satellite | Ka band UL |
| 31 | 31.3 | Fixed wireless | Microwave point-to-point |
| 31 | 31.08 | Fixed wireless | LMDS |
| 31 | 31.3 | Fixed wireless | Microwave point-to-point |
| 31.075 | 31.23 | Fixed wireless | LMDS |
| 31.225 | 31.3 | Fixed wireless | LMDS |
| 31.3 | 32.3 | Satellite | Radio Astronomy/Deep space |
| 31.8 | 33.4 | 5G | 5G Proposed FNPRM |
| 32.3 | 33.4 | Aviation | Aviation |
| 32.3 | 33 | Satellite | Satellite-Satellite/radio navigation |
| 33.4 | 36 | Land Mobile | Private Land Mobile |
| 33.4 | 36 | Satellite | Radiolocation and deep space research |
| 33.4 | 36 | Radar | Radiolocation and deep space research |
| 34.2 | 34.7 | Satellite | Deep space research |
| 35.5 | 36 | Satellite | Earth exploration (active?) Space research (active) |
| 36 | 37 | Satellite | Earth exploration (passive) Radio Astronomy Some are specific locations with antenna arrays |
| 37 | 38.6 | 5G | R&O |
| 37 | 37.5 | Satellite | Government (NASA RX BAND, Military sites need protections…Limited regions) |
| 37 | 37.5 | Government | Government (NASA RX BAND, Military sites need protections…Limited regions) |
| 37 | 38 | Fixed | Space research / Fixed/mobile federal |
| 37 | 38 | Space | Space research / Fixed/mobile federal |
| 37.5 | 42.5 | Satellite | Boeing downlink - proposed |
| 37.5 | 38.6 | Satellite | Satellite |
| 37.5 | 38.6 | Satellite | Satellite downlink : space to earth |
| Start (GHz) | Stop (GHz) | Usage | Comments |
| 38.6 | 40 | 5G | R&O |
| 38.6 | 39.5 | Satellite | Satellite (space to earth) |
| 38.6 | 39.5 | Fixed wireless | Fixed wireless |
| 38.6 | 39.5 | 5G | 5G Proposed |
| 39.5 | 40 | Satellite | Satellite (space to earth-military downlink) Fixed (Part 101) |
| 40 | 42 | Satellite | Satellite downlink (space to earth) |
| 42 | 42.5 | 5G | 5G Proposed FNPRM |
| 47.2 | 50.2 | 5G | 5G Proposed FNPRM |
| 47.2 | 52.4 | Satellite | Boeing Uplink - proposed |
| 50.4 | 52.6 | 5G | 5G Proposed FNPRM |
| 59 | 59.3 | Satellite | Passive earth exploration satellite |
| 59.3 | 64 | Radiolocation | Radio location Unlicensed (e.g. 802.11ad) |
| 64 | 71 | 5G | Unlicensed R&O |
| 64 | 65 | Satellite | Fixed/mobile (federal) Intra-satellite |
| 65 | 71 | Satellite | Satellite |
| 71 | 76 | 5G | 5G Proposed FNPRM |
| 71 | 74 | Fixed | Fixed Federal Space to earth |
| 81 | 86 | 5G | 5G Proposed FNPRM |

# Typical Antenna Response in mm-wave spectrum

Fig. 1 shows a typical dual-polarized patch antenna, while Fig.2 shows the peak gain of the antenna (measured over all angles) versus frequency.  The data, which was obtained by simulations, can be considered to be typical for patch antennas. It is immediately apparent that the antenna provides little to no rejection beyond 28 GHz. The antenna feed-lines could provide another 1-2 dB of rejection, which is insignificant.

Figure 1: Stacked dual-polarized patch antenna

Figure 2: Simulated peak gain vs frequency

# Conclusion

This paper presents preliminary data that broadly outlines different classes of interfering signals in mm-wave bands. A typical antenna response shown in this paper shows the weak rejection of these blockers.

A key objective of this paper is to motivate members of RAN4 to conduct a region-by-region survey of systems deployed near the 3GPP bands of interest – this includes transmit powers, directivity of antennas, and physical proximity of these transmitters from 5G networks so that path loss can be determined. This effort is essential to deriving an out-of-band blocker specification for 5G systems.

# References

- Federal Communications Commission, Report and Further Notice of Proposed Rule Making, Doc. 16-89, July 14, 2016. Washington, D.C. 20554

- Federal Communications Commission,  Ka-band Service Rules Third Report and Order, 12 FCC Rcd at 22319-20 (paras. 21-23).  Washington, D.C. 20554

- Federal Communications Commission , International Bureau, Satellite Division Information, Clarification of 47 C.F.R. § 25.140(b)(2), Space Station Application Interference Analysis, Public Notice, 18 FCC Rcd 25099 (Int'l Bur. 2003) (First Interference Analysis Public Notice), Washington, D.C. 20554.