3GPP TSG RAN4 Meeting #81	R4-1609956

Reno, USA, 14 – 18 November, 2016

Title: 	MSD for 5+66 2UL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.6.2

Document for:	Approval

# 1	Introduction

This contribution discusses MSD requirements for 5+66 UL CA.

# 2	Discussion

This CA combination has IMD2 and IMD5 falling on top of own B5 RX. The testpoint was suggested in [1]:

| CA | UL CA | IMD |  | UL Fc [MHz] | DL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | 3rd DL Fc [MHz] | IMD center [MHz] | DL BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B5+B66 | B5 | IMD2 | fB66-fB5 | 838 | 883 | 5 | 25 | B5 | 883.0 | 883.0 | 5 |
|  | B66 |  |  | 1721 | 2121 | 5 | 25 |  |  |  | 5 |
| B5+B66 | B5 | IMD5 | 2*fB66-3*fB5 | 846.5 | 891.5 | 5 | 25 | B5 | 891.5 | 891.5 | 5 |
|  | B66 |  |  | 1715.5 | 2115.5 | 5 | 25 |  |  |  | 5 |

We use the conventional common diplexer based architecture in our analysis with the following component assumptions. Diplexer was not assumed in diversity path.

| Component | IP2 (dBm) | IP5 (dBm) |
| --- | --- | --- |
| Antenna switch | 112 | 53 |  |
| Diplexer | 115 | 53 |  |
| Duplexer | 100 | 53 |  |
| LNA | 5 | -10 |  |

B5 PA forward mixing IMD2 at PA output with 60dB isolation		-10

B5 PA forward mixing RFIC Pin2Pin IMD2 at PA output				 -14.5

B5 PA forward mixing IMD5 at PA output with 60dB isolation		-39.5

B5 PA forward mixing RFIC Pin2Pin IMD5 at PA output				 -48.5

B5 duplexer TX rejection at B5 RX (forward)						        44

B66 duplexer TX rejection at B5 RB (forward)						  30

B5 TX isolation															  53

B5 RX isolation															  50

Antenna isolation 														  10

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  65

Diplexer isolation															  15

B5 FE loss (diplexer+switch+duplexer)						    	             4.5

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

Based on these parameters we get the following MSD’s. PA forward mixing results are based on measurements with commercial PA’s. CF=1dB in this analysis.

IMD2 dominant factors:

IMD5 dominant factors:

The MSD is as follows.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B5+B66 | B5 | IMD2 | fB66 -fB5 | 838 | 5 | 25 | 883 | 5 | 1 | 35.6 |
|  | B66 |  |  | 1721 | 5 | 25 |  |  |  |  |
|  | B5 | IMD5 | 2*fB66 -3*fB5 | 846.5 | 5 | 25 | 891.5 | 5 | 1 | 5.1 |
|  | B66 |  |  | 1715.5 | 5 | 25 |  |  |  |  |

# 3	Conclusion

MSD analysis for CA_5A-66A was provided with the following results.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B5+B66 | B5 | IMD2 | fB66 -fB5 | 838 | 5 | 25 | 883 | 5 | 1 | 35.6 |
|  | B66 |  |  | 1721 | 5 | 25 |  |  |  |  |
|  | B5 | IMD5 | 2*fB66 -3*fB5 | 846.5 | 5 | 25 | 891.5 | 5 | 1 | 5.1 |
|  | B66 |  |  | 1715.5 | 5 | 25 |  |  |  |  |

# 4	References

[1] R4-168104, “2UL 2DL Possible testpoints”, Qualcomm Inc, RAN4#80bis