3GPP TSG RAN4 Meeting #81	R4-1609963

Reno, USA, 14 – 18 November, 2016

Title: 	MSD for 2+66 2UL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.6.2

Document for:	Approval

# 1	Introduction

This contribution discusses MSD requirements for 2+66 UL CA.

# 2	Discussion

This CA combination has IMD2 and IMD5 falling on top of own B5 RX. The testpoints were suggested in [1]:

| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1855 | 1935 | 1.4 | 6 | B2 | 1935.0 | 1935.0 | 1.4 | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B66 |  |  | 1775 | 2175 | 5 | 25 |  |  |  | 5 | N/A |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1860 | 1940 | 20 | 50 | B2 | 1940.0 | 2040.0 | 20 | TBD |
|  | B66 |  |  | 1770 | 2170 | 20 | 100 |  |  |  | 20 | N/A |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1909.3 | 1989.3 | 1.4 | 6 | B66 | 2112.5 | 2106.1 | 1.4 | N/A |
|  | B66 |  |  | 1712.5 | 2112.5 | 5 | 25 |  |  |  | 25 | TBD |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1883.3 | 1963.3 | 1.4 | 6 | B66 | 2150.0 | 2149.9 | 1.4 | N/A |
|  | B66 |  |  | 1750 | 2150 | 5 | 25 |  |  |  | 5 | TBD |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1855 | 1935 | 5 | 25 | B2 | 1935.0 | 1935.0 | 5 | TBD |
|  | B66 |  |  | 1775 | 2175 | 5 | 25 |  |  |  | 5 | N/A |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1907.5 | 1987.5 | 5 | 25 | B66 | 2112.5 | 2102.5 | 5 | N/A |
|  | B66 |  |  | 1712.5 | 2112.5 | 5 | 25 |  |  |  | 5 | TBD |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1883.3 | 1963.3 | 5 | 25 | B66 | 2150.0 | 2149.9 | 5 | N/A |
|  | B66 |  |  | 1750 | 2150 | 5 | 25 |  |  |  | 5 | TBD |

We note that the second testpoint (IMD5 for B2) is very problematic as the IMD5 center is 2040MHz which is outside the B2 DL. In addition, this these UL frequencies the IMD3 center frequency is 1950 which heavily dominates so called PA forward mixing IMD5 results.

We use the conventional common diplexer based architecture in our analysis with the following component assumptions. B2+B66 is implemented using a quadplexer in diplexer H-branch. Diplexer was not assumed in diversity path.

| Component | IP3 (dBm) | IP5 (dBm) |
| --- | --- | --- |
| Antenna switch | 68 | 53 |  |
| Diplexer | 86 | 53 |  |
| Duplexer | 74 | 53 |  |
| LNA | -6 | -10 |  |

B2 IMD:

B2 PA forward mixing IMD3 at PA output with 60dB isolation    	-20.5

B2 PA forward mixing RFIC Pin2Pin IMD3 at PA output				 -25

B2 PA forward mixing IMD5 at PA output with 60dB isolation    	-19.5

B2 PA forward mixing RFIC Pin2Pin IMD5 at PA output				 -28.5

B66 IMD:

B2 PA forward mixing IMD3 at PA output with 60dB isolation    	-15.5

B2 PA forward mixing RFIC Pin2Pin IMD3 at PA output				 -20

B2 PA forward mixing IMD5 at PA output with 60dB isolation    	-45.5

B2 PA forward mixing RFIC Pin2Pin IMD5 at PA output				 -54.5

B2+B66 quadplexer TX/RX/x-band isolation							  52

B2 quadplexer TX attenuation at B2 RX (forward)			              44

B2 quadplexer TX attenuation at B66 RX (forward)			              44

B66 quadplexer TX attenuation at B2 RX (forward)			              44

B66 quadplexer TX attenuation at B66 RX (forward)			              45

Antenna isolation 														  10

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  65

Diplexer isolation															  15

B5 FE loss (diplexer+switch+duplexer)						    	             4.5

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

Based on these parameters we get the following MSD’s. PA forward mixing results are based on measurements with commercial PA’s. CF=1dB in this analysis, which need to be modified. These note that IMD5 for B2 is heavily dominated by the IMD3 that overlaps IMD5 region.

B2 IMD3 dominant factors:

B2 IMD5 dominant factors:

B66 IMD3 dominant factors:

B66 IMD5 dominant factors:

The MSD is as follows.

| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1855 | 1935 | 1.4 | 6 | B2 | 1935.0 | 1935.0 | 1.4 | 26.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B66 |  |  | 1775 | 2175 | 5 | 25 |  |  |  | 5 | N/A |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1860 | 1940 | 20 | 50 | B2 | 1940.0 | 2040.0 | 20 | 24.0 |
|  | B66 |  |  | 1770 | 2170 | 20 | 100 |  |  |  | 20 | N/A |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1909.3 | 1989.3 | 1.4 | 6 | B66 | 2112.5 | 2106.1 | 1.4 | N/A |
|  | B66 |  |  | 1712.5 | 2112.5 | 5 | 25 |  |  |  | 25 | 29.2 |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1883.3 | 1963.3 | 1.4 | 6 | B66 | 2150.0 | 2149.9 | 1.4 | N/A |
|  | B66 |  |  | 1750 | 2150 | 5 | 25 |  |  |  | 5 | 3.5 |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1855 | 1935 | 5 | 25 | B2 | 1935.0 | 1935.0 | 5 | 26.9 |
|  | B66 |  |  | 1775 | 2175 | 5 | 25 |  |  |  | 5 | N/A |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1907.5 | 1987.5 | 5 | 25 | B66 | 2112.5 | 2102.5 | 5 | N/A |
|  | B66 |  |  | 1712.5 | 2112.5 | 5 | 25 |  |  |  | 5 | 29.2 |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1883.3 | 1963.3 | 5 | 25 | B66 | 2150.0 | 2149.9 | 5 | N/A |
|  | B66 |  |  | 1750 | 2150 | 5 | 25 |  |  |  | 5 | 3.5 |

# 3	Conclusion

MSD analysis for CA_2A-66A was provided with the following results. Discussion on the test points is necessary.

| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1855 | 1935 | 1.4 | 6 | B2 | 1935.0 | 1935.0 | 1.4 | 26.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B66 |  |  | 1775 | 2175 | 5 | 25 |  |  |  | 5 | N/A |
| B2+B66 | B2 | IMD5 (IMD3) | 3*fB2-2*fB66 | 1860 | 1940 | 20 | 50 | B2 | 1940.0 | 2040.0 | 20 | 24.0 |
|  | B66 |  |  | 1770 | 2170 | 20 | 100 |  |  |  | 20 | N/A |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1909.3 | 1989.3 | 1.4 | 6 | B66 | 2112.5 | 2106.1 | 1.4 | N/A |
|  | B66 |  |  | 1712.5 | 2112.5 | 5 | 25 |  |  |  | 25 | 29.2 |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1883.3 | 1963.3 | 1.4 | 6 | B66 | 2150.0 | 2149.9 | 1.4 | N/A |
|  | B66 |  |  | 1750 | 2150 | 5 | 25 |  |  |  | 5 | 3.5 |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1855 | 1935 | 5 | 25 | B2 | 1935.0 | 1935.0 | 5 | 26.9 |
|  | B66 |  |  | 1775 | 2175 | 5 | 25 |  |  |  | 5 | N/A |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1907.5 | 1987.5 | 5 | 25 | B66 | 2112.5 | 2102.5 | 5 | N/A |
|  | B66 |  |  | 1712.5 | 2112.5 | 5 | 25 |  |  |  | 5 | 29.2 |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1883.3 | 1963.3 | 5 | 25 | B66 | 2150.0 | 2149.9 | 5 | N/A |
|  | B66 |  |  | 1750 | 2150 | 5 | 25 |  |  |  | 5 | 3.5 |

# 4	References

[1] R4-168104, “2UL 2DL Possible testpoints”, Qualcomm Inc, RAN4#80bis