3GPP TSG RAN4 Meeting #81	R4-1609971

Reno, USA, 14 – 18 November, 2016

Title: 	MSD for 1+7+46 2UL/3DL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.7.2

Document for:	Approval

# 1	Introduction

This contribution discusses MSD requirements for 1+7+46 2UL/3DL CA. Please note that B46 is assumed DL only in this analysis.

# 2	Discussion

This CA combination has IMD4 and IMD5 falling on top of own B46 RX. The test point for IMD4 was approved in [1] and the test point for IMD5 was suggested in [2]:

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B7+B46 | B1 | IMD4 | 3*fB7 -fB1 | 1950 | 5 | 25 | 5580 | 20 | TBD | TBD |
|  | B7 |  |  | 2510 | 10 | 50 |  |  |  |  |

| CA | UL CA | IMD |  | UL Fc [MHz] | DL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | 3rd DL Fc [MHz] | IMD center [MHz] | DL BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B7+B46 | B1 | IMD5 | 4*fB1 - fB5 | 1960 | 2150 | 5 | 25 | B46 | 5320.0 | 5320.0 | 20 |
|  | B7 |  |  | 2520 | 2640 | 10 | 50 |  |  |  |  |

We use the conventional common triplexer based architecture in our analysis with the following component assumptions. B46 uses separate antenna in main and diversity.

| Component | IP4 (dBm) | IP5 (dBm) |
| --- | --- | --- |
| Antenna switch | 56 | 53 |  |
| Diplexer | 55 | 53 |  |
| Duplexer | 55 | 53 |  |
| LNA | -6 | -10 |  |

B1 PA forward mixing IMD5 at PA output with 60dB isolation	       -60.5

B1 PA forward mixing RFIC Pin2Pin IMD5 at PA output				 -66

B7 PA forward mixing IMD4 at PA output with 60dB isolation	 	 -48

B7 PA forward mixing RFIC Pin2Pin IMD4 at PA output				 -52.5

B7 duplexer TX rejection at B46 RX (forward)						  25

B1 duplexer TX rejection at B46 RX (forward)						  18

B46 filter rejection at B1 TX											  30

B46 filter rejection at B7 TX										        30

Antenna isolation 														  15

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  60

Diplexer isolation															  15

B46 FE loss (switch+trace+filter)								    	          6

LNA NF+IM																    7

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

Based on these parameters we get the following MSD’s. PA forward mixing results are based on measurements with commercial PA’s. CF=0dB in this analysis.

IMD4 dominant factors:

IMD5 dominant factors:

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B7+B46 | B1 | IMD4 | 3*fB7 -2*fB1 | 1950 | 5 | 25 | 5580 | 20 | 0 | 5.3 |
|  | B7 |  |  | 2510 | 10 | 50 |  |  |  |  |
|  | B1 | IMD5 | 4*fB1 -1*fB7 | 1960 | 5 | 25 | 5320 | 20 | 0 | 0 |
|  | B7 |  |  | 2520 | 10 | 50 |  |  |  |  |

# 3	Conclusion

MSD analysis for CA_1A-7A-46A was provided with the following results. We recognize other PA measurements may show different forward mixing characteristics that may lead into non-zero MSD for IMD5.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B7+B46 | B1 | IMD4 | 3*fB7 -2*fB1 | 1950 | 5 | 25 | 5580 | 20 | 0 | 5.3 |
|  | B7 |  |  | 2510 | 10 | 50 |  |  |  |  |
|  | B1 | IMD5 | 4*fB1 -1*fB7 | 1960 | 5 | 25 | 5320 | 20 | 0 | 0 |
|  | B7 |  |  | 2520 | 10 | 50 |  |  |  |  |

# 4	References

[1] R4-168857, “MSD Test configurations for xDL/2UL CA band combinations with self interference issues”, LG, RAN4#80bis

[2] R4-168105, “2UL 3DL Possible testpoints”, Qualcomm Inc, RAN4#80bis