3GPP TSG RAN4 Meeting #81	R4-1609976

Reno, USA, 14 – 18 November, 2016

Title: 	MSD for 3+19+21 2UL/3DL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.7.2

Document for:	Approval

# 1	Introduction

This contribution discusses MSD requirements for 3+19+21 2UL/3DL CA.

# 2	Discussion

This CA combination has IMD5 falling on top of own B3 RX . The test point was approved in [1]:

| B3+B19+B21 | B19 | IMD5 | 4B19 -fB21 | 832.5 | 877.5 | 5 | 25 | B3 | 1869.6 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B21 |  |  | 1460.4 | 1505.4 | 5 | 25 |  |  |  |

We use conventional common triplexer based architecture in our analysis with the following component assumptions. Diplexer/Triplexer is not assumed in diversity.

| Component | IP5 (dBm) |
| --- | --- |
| Antenna switch | 53 |
| Diplexer/Triplexer | 53 |
| Duplexer | 53 |
| LNA | -10 |

B19 duplexer TX attenuation at B3 RX							              28

B21 duplexer TX attenuation at B3 RX									  40

B19 PA forward mixing IMD5 at PA output with 60dB isolation	       -41

B3 FE loss (switch+trace+filter)										          6

Antenna isolation 														  15

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  65

Diplexer isolation															  15

LNA NF																   	    5

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

Based on these parameters we get the following MSD’s. PA forward mixing results are based on measurements with commercial PA’s. CF is 1dB in this analysis.

IMD5 dominant factors:

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B19+B21 | B19 | IMD5 | 4*fB19 -*fB21 | 832.5 | 5 | 25 | 1869.6 | 5 | 1 | 4.4 |
|  | B21 |  |  | 1460.4 | 5 | 25 |  |  |  |  |

# 3	Conclusion

MSD analysis for 2UL/3Dl B3+B19+B21 CA was provided with the following results.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B19+B21 | B19 | IMD5 | 4*fB19 -*fB21 | 832.5 | 5 | 25 | 1869.6 | 5 | 1 | 4.4 |
|  | B21 |  |  | 1460.4 | 5 | 25 |  |  |  |  |

# 4	References

[1] R4-168857, “MSD Test configurations for xDL/2UL CA band combinations with self interference issues”, LG, RAN4#80bis

[2] R4-168105, “2UL 3DL Possible testpoints”, Qualcomm Inc, RAN4#80bis