3GPP TSG-RAN WG4 #81	R4- 1610166

November 14th ‒ 18th, 2016

Reno, Nevada, USA

Agenda item:	11.3.1

Source: 	Qualcomm Incorporated

Title: 	Simulation of results for NR coexistence study: summary and ACIR proposals

Document for:	Approval

# Introduction

In this contribution we present a summary of the adjacent channel coexistence simulation results for NR. The simulation assumptions adopted were the ones agreed in RAN4 #80bis [1][2][3]. Based on the obtained results we make a proposal for the ACIR to be adopted in the ACS/ACLR derivation.

# Discussion

The latest simulation assumptions for NR adjacent channel coexistence study were agreed in [1]. A total of 11 test cases were identified:

- DL to DL, Indoor hotspot, 30GHz

- DL to DL, Urban macro, 30GHz

- DL to DL, Dense urban, 30GHz

- UL to UL, Indoor hotspot, 30GHz

- UL to UL, Urban macro, 30GHz

- UL to UL, Dense urban, 30GHz

- DL to DL, Indoor hotspot, 70GHz

- DL to DL, Dense urban, 70GHz

- UL to UL, Indoor hotspot ,70GHz

- UL to UL, Dense urban, 70GHz

- UL to UL, Urban macro, 30GHz

We ran a full study and the following contributions contain the detail set of results:

- [4] contains analysis for Case 2, Case 5 and Case 11

- [5] contains analysis for Case 3 and Case 6

- [6] contains analysis for Case 1 and Case 4

- [7] contains analysis for Case 8 and Case 10

- [8] contains analysis for Case 7 and Case 9

A summary of the simulation results is presented in Table 1. The results are expressed in terms of ACIR needed to keep the mean and 5%-tile degradation due to ACI within 5%. Based on the simulation results available so far:

- The DL ACIR needed for the mean degradation criteria is always less than 17dB

- The DL ACIR needed for the 5%-tile degradation criteria is always less than 15dB

- The UL ACIR needed for the mean degradation criteria is always less than 10dB

- The UL ACIR needed for the 5%-tile degradation criteria is always less than 15dB

Based on the above observations, we make the following proposals to restrict the ACIR range:

Proposal 1: The DL ACIR adopted to derive NR UE ACS and BS ACLR at 30GHz should be less than or equal to 17dB.

Proposal 2: The UL ACIR adopted to derive NR UE ACLR and BS ACS at 30GHz should be less than or equal to15dB.

Table 1. Summary of simulation results for the NR adjacent channel coexistence study.

| Case | Aggressor BW allocation | Victim BW allocation | Simulation frequency | Direction | Deployment Scenario | Mean | 5%-tile |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 200MHz | 200MHz | 30 GHz | DL to DL | Indoor hotspot | 16.18 | 12.62 |
| 2 | 200MHz | 200MHz | 30 GHz | DL to DL | Urban macro | 12.64 | NA |
| 3 | 200MHz | 200MHz | 30 GHz | DL to DL | Dense urban | 8.4 | 10.54 |
| 4 | 200MHz | 200MHz | 30 GHz | UL to UL | Indoor hotspot | 8.89 | 14.5 |
| 5 | 200MHz | 200MHz | 30 GHz | UL to UL | Urban macro | 2.25 | NA |
| 6 | 200MHz | 200MHz | 30 GHz | UL to UL | Dense urban | 1.12 | 11.48 |
| 7 | 200MHz | 200MHz | 70 GHz | DL to DL | Indoor hotspot | 16.42 | 12.83 |
| 8 | 200MHz | 200MHz | 70 GHz | DL to DL | Dense urban | 5.98 | NA |
| 9 | 200MHz | 200MHz | 70 GHz | UL to UL | Indoor hotspot | 6.92 | 14.18 |
| 10 | 200MHz | 200MHz | 70 GHz | UL to UL | Dense urban | 0 | NA |
| 11 | 20MHz | 20MHz | 30 GHz | UL to UL | Urban macro | In progress | In progress |

Note that simulation results for case 2 are not provided yet. This case represents a particular scenario, since it is an attempt to increase the coverage in the urban macro deployment. Since we observed that this case could create an artifact in terms of the requirement for the cell edge throughput, we are currently running further evaluation to clarify the system level implications of this use case. On the other hand, Cases 1 to 10, i.e. the default scenarios, showed consistent results which are summarized by Proposal 1 and Proposal 2.

# Relationship between ACIR and Tx masks (UEM and SEM)

Another interesting observation is related to the relationship between ACLR and Spectrum Emission Mask (SEM) or Unwanted Emission Mask (UEM). While ACLR is a relative value enabling adjacent channel coexistence, SEM/UEM will mainly address the need to fulfill specific regulatory requirements, i.e. absolute emission level. These two figures are usually correlated, therefore it is also worth considering regulatory requirements in millimeter wave (mmW) bands when defining ACLR requirements.

FCC recently released a Report and Order for 27.5-28.35 GHz and 37.5-40 GHz Bands [9]. The emission requirements are depicted in Figure 1. By fixing the transmitter output power and the system channel bandwidth (BW), it possible to measure a corresponding ACLR by integrating the emission levels within the adjacent channel. If we focus on UL side, i.e. UE requirement, by assuming 23dBm total output power the corresponding ACLR would be ~14dB and ~11dB, for 100MHz and 200MHz channel BW, respectively. This shows that, although a bit tighter, the ACLR derived based on the ACIR results from the coexistence study would be in the same ballpark.

Figure 1. FCC emission requirements from [9].

# Conclusions

In this contribution we summarized the simulation results for the NR adjacent channel coexistence study. Based on the analysis of a very wide range of test cases, we made the following proposals:

Proposal 1: The DL ACIR adopted to derive NR UE ACS and BS ACLR at 30GHz should be less than or equal to 17dB.

Proposal 2: The UL ACIR adopted to derive NR UE ACLR and BS ACS at 30GHz should be less than or equal to 15dB.

# Reference

- R4-168953, “Simulation assumptions for WP5D co-existence simulation”, NTT DOCOMO, INC.

- R4-168794, “Further Way forward on urban micro layout and BS Beamforming Model”, Huawei Technologies Co., Ltd.

- R4-168795, “WF on simulation parameters for Urban macro and TPC model”, NTT DOCOMO, INC., Qualcomm Incorporated.

- R4-1610161, “Simulation results for NR coexistence study: urban macro deployment at 30GHz”, Qualcomm Incorporated.

- R4-1610162, “Simulation results for NR coexistence study: dense urban deployment at 30GHz”, Qualcomm Incorporated.

- R4-1610163, “Simulation results for NR coexistence study: indoor deployment at 30GHz”, Qualcomm Incorporated.

R4-1610164, “Simulation results for NR coexistence study: dense urban deployment at 70GHz”, Qualcomm Incorporated.

- R4-1610165, “Simulation results for NR coexistence study: indoor deployment at 70GHz”, Qualcomm Incorporated.

- 47CFR Part 30 per the FCC Report and Order FCC 16-89.