3GPP TSG-RAN WG4 meeting #81  	R4-1610463

# Reno, Nevada, 14 – 18 November, 2016

Source: 	Ericsson

Title:  	TP for TR 36.789 v0.0.2: Updates in throughput tests procedures

Agenda Item:	7.1.1

Document for:	Approval

# Background

During RAN plenary in June 2016, a new study item has been approved in [1]. A new TR number is assigned for this SI, which is TR 36.789.

A skeleton TR is approved in RAN4#80 in [2]. In this update, two approved TPs from RAN4#80 was incorporated, as listed in [3] and [4]. TR36.789v0.0.2 was approved in [5], which includes [3] and [4].

In this TP, we propose to several changes in Section 6.1, related to throughput tests.

# TP for TR38.803

We propose to remove Section 6.3, as shown below:

<<<<<<< Start of TP >>>>>>>>>

# 6	Multi-node Tests for LAA

## 6.1	Throughput test

<Editor’s note: Include contents related to throughput tests >

### 6.1.1	Test purpose

The purpose of the multi-node throughput test is to verify whether a 5GHz device can achieve a certain level of throughput when other 5GHz systems are present in the spectrum. For scenario 1, the purpose of this test is to evaluate the throughput performance of IEEE 802.11 AP when a LAA eNB transmit in DL. For scenario 3, the purpose of the test is to help 3GPP to validate LAA and enhancement of system performance; there will be no pass/fail criterial for IEEE 802.11 device.

### 6.1.2	Test procedure

The procedure for the throughput test consists of two main steps:

- creating a baseline in which all nodes belong to the same technology

- verifying the impact compared to the baseline when half of the nodes are replaced with another technology

The two steps are adapted depending on the specific scenario under analysis.

For Scenario 1, the procedure is made of the following steps:

- Baseline: IEEE 802.11 AP to IEEE 802.11 AP. In this step Node A and Node B are IEEE 802.11 APs, Node C and Node D are IEEE 802.11 STAs.

- LAA and IEEE 802.11 coexistence. In this step Node B is replaced with LAA eNodeB and Node D is replaced with LAA UEs

The specific DUT configuration for the two steps in case of scenario 1 are listed in Table 6.1.2-1:

Table 6.1.2-1 DUT configuration for the throughput test in case of Scenario 1.

|  | DUT A | DUT B | Criteria |
| --- | --- | --- | --- |
| Step 1: | IEEE 802.11 AP | IEEE 802.11 AP | Impact of IEEE 802.11 transmitter on IEEE 802.11 throughput: DUT B IEEE 802.11 system achieves certain level of throughput in presence of DUT A IEEE 802.11 system. |
| Step 2: | LAA eNB | IEEE 802.11 AP | Impact of LAA transmitter on IEEE 802.11 throughput: DUT B IEEE 802.11 system achieves certain level of throughput in presence of DUT A LAA system. |

The different signal levels to be adopted in Step 1 and 2 are specified in Section 5.1.1.

Step-1: Determination of baseline

The step-1 is detailed below:

- Define a set of IEEE 802.11 APs and associated IEEE 802.11 STAs (terminal devices). APs and STAs need to be considered, composing a total number of [TBD1] AP-STA sets. The APs and STAs to be used are described in section 5.2.4

- Fix Link A-C with one specific AP and one specific STA model taken from the sets defined in a). The wanted signal level for the Link A-C is as specified in section 5.2.5. This is the Reference Link.

- For Link B-D, pick one AP from the set of vendors identified in a), and associate a STA from the set identified in a). The signal level for the Link B-C is specified section 5.2.5. This is the Aggressor Link. The specific STA-AP sets to be considered in this step are described in section 5.2.4. A total number of [TBD2] different configurations is obtained at this stage.

- The Interferer signal links are set as specified in section 5.1.1.

- Push full buffer DL [UDP] traffic for the reference link.

- For Link B-D, push DL [UDP] traffic with different traffic profiles (full buffer or additional profiles if agreed).

- Record the throughput achieved by the Link A-C averaged over 1 minute.

- Repeat [TBD3] times to achieve enough statistical confidence. There will be total of [TBD2] x [TBD2] points per traffic profile.

- Collect the CDF obtained from the data points in g) and record the mean.

Step-2: Coexistence case

- Step 1 is repeated by replacing the nodes in Link B-D with LAA eNodeB (Node B) and LAA UE (Node D). The wanted signal level for the Link B-D is specified in in section 5.2.5.

- The Interferer signal links are set as specified above.

- Performance of the Link A-C is recorded.

- The test is passed the Pass/Fail criteria specified in section 6.1.3 is met.

### 6.1.3	Test metric

We consider within ±10% of the median of the mean throughput as the pass/fail criteria for best effort traffic case (i.e. throughput test). For outage tests, MOS criterion are considered, details are FFS.

The test metric and pass fail criteria are listed below.

| Scenario | Case | Traffic in victim link | Traffic in aggressor link | Pass/fail criteria |
| --- | --- | --- | --- | --- |
| 1 | Baseline (1a) | Best effort | Best effort | ±10% of the median of the mean throughput |
|  | Test (1a) | Best effort | Best effort | ±10% of the median of the mean throughput |
|  | Baseline (1b) | Voice | Best effort | [MOS criterion] |
|  | Test (1b) | Voice | Best effort | [MOS criterion] |
|  |  |  |  | Evaluation criteria |
| 3 | Baseline (3a) | Best effort | Best effort | ±10% of the median of the mean throughput |
|  | Test (3a) | Best effort | Best effort | ±10% of the median of the mean throughput |
|  | Baseline (3b) | Voice | Best effort | [MOS criterion] |
|  | Test (3b) | Voice | Best effort | [MOS criterion] |

### 6.1.4	Test results

## 6.2	Outage tests

<Editor’s note: Include contents related to channel access priority tests >

### 6.2.1	Test purpose

### 6.2.2	Test procedure

### 6.2.3	Test metric

### 6.2.4	Test results

<<<<<<< Start of TP >>>>>>>>>

# Proposal

We propose to accept the above TP for TR38.803.

# References

RP-161197, New Study Item proposal: Multi-node tests for LAA, Huawei, HiSilicon, Ericsson, Qualcomm, Nokia

R4-166543, "Skeleton TR 36.789 for multi-node tests", Ericsson.

R4-166969, TP on Scope of TR 36.789: multi-node coexistence test, Source: Ericsson

R4-166970, TP for multi-node testing TR: Section 4 and 5, Source: Huawei, Ericsson

R4-168544, Updated TR TR 36.783 v0.0.2: Multi-node tests for LAA, Source: Ericsson